VII. AREA OF OPERATION: NAVIGATION SYSTEMS

NOTE: The examiner shall select TASK A and B. If aircraft is not DME equipped, performance of DME arcs shall be tested orally.

# A. TASK: INTERCEPTING AND TRACKING NAVIGATIONAL SYSTEMS AND DME ARCS

REFERENCES: 14 CFR part 91; FAA-H-8083-9, FAA-H-8083-15; FAA-S-8081-4; AIM.

Objective. To determine that the applicant:

- Exhibits instructional knowledge of the elements of intercepting and tracking navigational systems and DME arcs by describing

  - tuning and identification of a navigational facility.

  - setting of a selected course on the navigation selector or the correct identification of a selected bearing on the RMI.

  - method for determining aircraft position relative to a facility.

  - procedure for intercepting and maintaining a selected course.

  - procedure for intercepting and maintaining a DME arc.

  - procedure for intercepting a course or localizer from a DME arc.

  - recognition of navigation facility or waypoint passage.

  - recognition of navigation receiver or facility failure.

- Exhibits instructional knowledge of common errors related to intercepting and tracking navigational systems and DME arcs by describing

  - incorrect tuning and identification procedures.

  - failure to properly set the navigation selector on the course to be intercepted.

  - failure to use proper procedures for course or DME arc interception and tracking.

  - improper procedures for intercepting a course or localizer from a DME arc.

- Demonstrates and simultaneously explains intercepting and tracking navigational systems and DME arcs from an instructional standpoint.

- Analyzes and corrects simulated common errors related to intercepting and tracking navigational systems and DME arcs.

- Exhibits instructional knowledge on the uses of the MFD and other graphical navigational displays, if installed, to monitor position in relation to the desired flightpath during holding.

# B. TASK: HOLDING PROCEDURES

REFERENCES: 14 CFR part 91; FAA-H-8083-9, FAA-H-8083-15; FAA-S-8081-4; AIM.

Objective. To determine that the applicant:

- Exhibits instructional knowledge of holding procedures by describing

  - setting of aircraft navigation equipment.

  - requirement for establishing the appropriate holding airspeed for the aircraft and altitude.

  - recognition of arrival at the holding fix and the prompt initiation of entry into the holding pattern.

  - timing procedure.

  - correction for wind drift.

  - use of DME in a holding pattern.

  - compliance with ATC reporting requirements.

- Exhibits instructional knowledge of common errors related to holding procedures by describing

  - incorrect setting of aircraft navigation equipment.

  - inappropriate altitude, airspeed, and bank control.

  - improper timing.

  - improper wind drift correction.

  - failure to recognize holding fix passage.

  - failure to comply with ATC instructions.

- Demonstrates and simultaneously explains holding procedures from an instructional standpoint.

- Analyzes and corrects simulated common errors related to holding procedures.

- Exhibits instructional knowledge on the use of the MFD and other graphical navigational displays, if installed, to monitor position in relation to the desired flight path during holding.