IX. AREA OF OPERATION: EMERGENCY OPERATIONS

NOTE: The examiner shall select at least one TASK. The examiner shall omit TASKS C and D unless the applicant furnishes a multiengine airplane for the practical test, then TASK C or D is mandatory.

# A. TASK: LOSS OF COMMUNICATIONS

REFERENCES: 14 CFR part 91; FAA-H-8083-9, FAA-H-8083-15; FAA-S-8081-4; IAP; AIM.

Objective. To determine that the applicant exhibits instructional knowledge of the elements related to loss of communications by describing:

- Recognition of loss of communications.

- When to continue with flight plan as filed or when to deviate.

- How to determine the time to begin an approach at destination.

# B. TASK: APPROACH WITH LOSS OF PRIMARY FLIGHT INSTRUMENT INDICATORS

REFERENCES: 14 CFR part 91; FAA-H-8083-9, FAA-H-8083-15; FAA-S-8081-4; IAP; AIM.

Objective. To determine that the applicant:

- Exhibits instructional knowledge of the elements related to loss of primary flight instrument indicators by describing

  - recognition of inaccurate or inoperative primary instrument indicators and advising ATC and the examiner.

  - notification of ATC or examiner anytime that the aircraft is unable to comply with an ATC clearance or whether able to continue the flight.

  - importance of utilizing navigation equipment in an emergency situation and demonstrating a non-precision approach without the use of primary flight instruments.

- Exhibits instructional knowledge of common errors related to loss of primary flight instrument indicators by describing

  - recognition of failed system components that relate to primary flight instrument indication(s).

  - failure to notify ATC of situation.

  - failure to transition to emergency mode/standby instrumentation.

- Demonstrates and simultaneously explains loss of primary flight instrument indicators by conducting a non-precision approach without the use of these indicators.

- Analyzes and corrects common errors related to loss of primary flight instrument indicators.