V. AREA OF OPERATION: PERFORMANCE MANEUVERS

NOTE: The examiner shall at least select either TASK A or B, and either C or D.

# A. TASK: STEEP TURNS (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to steep turns.

- Establishes the manufacturer's recommended airspeed or if one is not stated, a safe airspeed not to exceed VA.

- Rolls into a coordinated 360steep turn with at least a 50° bank, followed by a 360steep turn in the opposite direction.

- Divides attention between airplane control and orientation.

- Maintains the entry altitude, ±100 feet (30 meters), airspeed, ±10 knots, bank, ±5°; and rolls out on the entry heading, ±10°.

# B. TASK: STEEP SPIRAL (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a steep spiral.

- Selects an altitude sufficient to continue through a series of at least three 360turns.

- Selects a suitable ground reference point.

- Applies wind-drift correction to track a constant radius circle around selected reference point with bank not to exceed 60at steepest point in turn.

- Divides attention between airplane control and ground track, while maintaining coordinated flight.

- Maintains the specified airspeed, ±10 knots, rolls out toward object or specified heading, 10.

# C. TASK: CHANDELLES (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to chandelles.

- Selects an altitude that will allow the maneuver to be performed no lower than 1,500 feet AGL (460 meters).

- Establishes the recommended entry configuration, power, and airspeed.

- Establishes the angle of bank at approximately 30°.

- Simultaneously applies power and pitch to maintain a smooth, coordinated climbing turn to the 90point, with a constant bank.

- Begins a coordinated constant rate rollout from the 90point to the 180point maintaining power and a constant pitch attitude.

- Completes rollout at the 180point, 10just above a stall airspeed, and maintaining that airspeed momentarily avoiding a stall.

- Resumes straight and level flight with minimum loss of altitude.

# D. TASK: LAZY EIGHTS (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to lazy eights.

- Selects an altitude that will allow the task to be performed no lower than 1,500 feet AGL (460 meters).

- Establishes the recommended entry configuration, power, and airspeed.

- Maintains coordinated flight throughout the maneuver.

- Achieves the following throughout the maneuver—

  - approximately 30bank at the steepest point.

  - constant change of pitch and roll rate.

  - altitude tolerance at 180points, 100 feet (30 meters) from entry altitude.

  - airspeed tolerance at the 180° point plus 10° knots from entry airspeed.

  - heading tolerance at the 180° point 10°.

- Continues the maneuver through the number of symmetrical loops specified and resumes straight and level flight.