VII. AREA OF OPERATION: NAVIGATION

# A. TASK: PILOTAGE AND DEAD RECKONING (ASEL and ASES)

REFERENCE: AC 61-23/FAA-H-8083-25.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to pilotage and dead reckoning.

- Follows the preplanned course by reference to landmarks.

- Identifies landmarks by relating surface features to chart symbols.

- Navigates by means of precomputed headings, groundspeed, and elapsed time.

- Corrects for and records differences between preflight groundspeed and heading calculations and those determined en route.

- Verifies the airplane's position within two (2) nautical miles of flight planned route.

- Arrives at the en route checkpoints within three (3) minutes of the initial or revised ETA and provides a destination estimate.

- Maintains appropriate altitude, 100 feet (30 meters), and headings, 10°.

# B. TASK: NAVIGATION SYSTEMS AND RADAR SERVICES (ASEL and ASES)

REFERENCES: FAA-H-8083-3, AC 61-23/FAA-H-8083-25; Navigation Equipment Operation Manuals, AIM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to navigation systems and radar services.

- Demonstrates the ability to use an airborne electronic navigation system.

- Locates the airplane's position using the navigation system.

- Intercepts and tracks a given course, radial, or bearing as appropriate.

- Recognizes and describes the indication of station passage if appropriate.

- Recognizes signal loss and takes appropriate action.

- Uses proper communication procedures when utilizing radar services.

- Maintains the appropriate altitude, 100 feet (30 meters) and heading, 10.

# C. TASK: DIVERSION (ASEL and ASES)

REFERENCES: FAA-H-8083-25; AIM

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to diversion.

- Selects an appropriate alternate airport and route.

- Makes an accurate estimate of heading, groundspeed, arrival time, and fuel consumption to the alternate airport.

- Maintains the appropriate altitude, 100 feet (30 meters), and heading, 10°.

# D. TASK: LOST PROCEDURES (ASEL and ASES)

REFERENCES: FAA-H-8083-25; AIM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to lost procedures.

- Selects an appropriate course of action.

- Maintains an appropriate heading and climbs, if necessary.

- Identifies prominent landmarks.

- Uses navigation systems/facilities and/or contacts an ATC facility for assistance as appropriate.