VIII. AREA OF OPERATION: SLOW FLIGHT AND STALLS

# A. TASK: MANEUVERING DURING SLOW FLIGHT (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to maneuvering during slow flight.

- Selects an entry altitude that will allow the task to be completed no lower than 1,500 feet (460 meters) AGL.

- Establishes and maintains an airspeed at which any further increase in angle of attack, increase in load factor, or reduction in power, would result in an immediate stall.

- Accomplishes coordinated straight-and-level flight, turns, climbs, and descents with landing gear and flap configurations specified by the examiner.

- Divides attention between airplane control and orientation.

- Maintains the specified altitude, 50 feet (15 meters); specified heading, 10°; airspeed 5/−0 knots, and specified angle of bank, 5°.

# B. TASK: POWER-OFF STALLS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, AC 61-67; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to power-off stalls.

- Selects an entry altitude that allows the task to be completed no lower than 1,500 feet (460 meters) AGL.

- Establishes a stabilized descent in the approach or landing configuration, as specified by the examiner.

- Transitions smoothly from the approach or landing attitude to a pitch attitude that will induce a stall.

- Maintains a specified heading, 10° in straight flight; maintains a specified angle of bank, not to exceed 20, 5°, in turning flight while inducing the stall.

- Recognizes and recovers promptly as the stall occurs by simultaneously reducing the angle of attack, increasing power to maximum allowable, and leveling the wings to return to a straight-and-level flight attitude with a minimum loss of altitude appropriate for the airplane.

- Retracts the flaps to the recommended setting, retracts the landing gear if retractable after a positive rate of climb is established.

- Accelerates to VX or VY speed before the final flap retraction; returns to the altitude, heading, and airspeed specified by the examiner.

# C. TASK: POWER-ON STALLS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, AC 61-67; POH/AFM.

NOTE: In some high performance airplanes, the power setting may have to be reduced below the practical test standards guideline power setting to prevent excessively high pitch attitudes (greater than 30° nose up).

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to power-on stalls.

- Selects an entry altitude that allows the task to be completed no lower than 1,500 feet (460 meters) AGL.

- Establishes the takeoff or departure configuration. Sets power to no less than 65 percent available power.

- Transitions smoothly from the takeoff or departure attitude to a pitch attitude that will induce a stall.

- Maintains a specified heading 5°, in straight flight; maintains a specified angle of bank, not to exceed a 20, 10°, in turning flight, while inducing the stall.

- Recognizes and recovers promptly as the stall occurs by simultaneously reducing the angle of attack, increasing power to maximum allowable, and leveling the wings to return to a straight-and-level flight attitude, with a minimum loss of altitude appropriate for the airplane.

- Retracts flaps to the recommended setting, retracts the landing gear if retractable, after a positive rate of climb is established.

- Accelerates to VX or VY speed before the final flap retraction; returns to the altitude, heading, and airspeed specified by the examiner.

# D. TASK: SPIN AWARENESS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, AC 61-67; POH/AFM.

Objective. To determine that the applicant exhibits knowledge of the elements related to spin awareness by explaining:

- Aerodynamic factors related to spins.

- Flight situations where unintentional spins may occur.

- Procedures for recovery from unintentional spins.