III. AREA OF OPERATION: AIR TRAFFIC CONTROL CLEARANCES AND PROCEDURES

# A. TASK: Airplane Systems Related to IFR Operations

References: 14 CFR parts 61, 91; FAA-H-8083-15; AIM

Objective: To determine the applicant exhibits satisfactory knowledge, risk management, and skills associated with ATC clearances and procedures solely by reference to instruments.

Knowledge: The applicant demonstrates understanding of:

- Elements and procedures related to ATC clearances and pilot/controller responsibilities for departure, en route, and arrival phases of flight including clearance void times.

- PIC emergency authority.

- Lost communication procedures and procedures for flights outside of radar environments.

Risk Management: The applicant demonstrates the ability to identify, assess and mitigate risks, encompassing:

- Failure to fully understand an ATC clearance.

- Inappropriate, incomplete, or incorrect ATC clearances.

- ATC clearance inconsistent with airplane performance or navigation capability.

- ATC clearance intended for other aircraft with similar call signs.

Skills: The applicant demonstrates the ability to:

- Correctly copy, read back, interpret, and comply with simulated or actual ATC clearances in a timely manner using standard phraseology as provided in the Aeronautical Information Manual.

- Correctly set communication frequencies, navigation systems (identifying when appropriate), and transponder codes in compliance with the ATC clearance. Use the current and appropriate paper or electronic navigation publications.

- Intercept all courses, radials, and bearings appropriate to the procedure, route, or clearance in a timely manner.

- Maintain the applicable airspeed ±10 knots, headings ±10°, altitude ±100 feet; and track a course, radial, or bearing within ¾-scale deflection of the CDI.

- Demonstrate SRM.

- Perform the appropriate airplane checklist items relative to the phase of flight.

# B. TASK: Airplane Flight Instruments and Navigation Equipment

References: 14 CFR parts 61, 91; FAA-H-8083-15, FAA-H-8083-16; AIM

Objective: To determine the applicant exhibits satisfactory knowledge, risk management, and skills associated with holding procedures solely by reference to instruments.

Knowledge: The applicant demonstrates understanding of:

- Elements related to holding procedures, including reporting criteria, appropriate speeds, and recommended entry procedures for standard, nonstandard, published, and nonpublished holding patterns.

Risk Management: The applicant demonstrates the ability to identify, assess and mitigate risks, encompassing:

- Recalculating fuel reserves if assigned an unanticipated EFC time.

- Scenarios and circumstances that could result in minimum fuel or the need to declare an emergency.

- Scenarios that could lead to holding, including deteriorating weather at the planned destination.

- Improper holding entry and improper wind correction while holding.

Skills: The applicant demonstrates the ability to:

- Explain and use an entry procedure that ensures the airplane remains within the holding pattern airspace for a standard, nonstandard, published, or non-published holding pattern.

- Change to the holding airspeed appropriate for the altitude or airplane when 3 minutes or less from, but prior to arriving at, the holding fix and set appropriate power as needed for fuel conservation.

- Recognize arrival at the holding fix and promptly initiate entry into the holding pattern.

- Maintain airspeed ±10 knots, altitude ±100 feet, selected headings within ±10°, and track a selected course, radial, or bearing within ¾-scale deflection of the CDI.

- Use proper wind correction procedures to maintain the desired pattern and to arrive over the fix as close as possible to a specified time and maintain pattern leg lengths when specified.

- Use an MFD and other graphical navigation displays, if installed, to monitor position in relation to the desired flightpath during holding.

- Comply with ATC reporting requirements and restrictions associated with the holding pattern.

- Demonstrate SRM.