| Private Pilot Practical Test Standards |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Maneuver | Speed | Tolerance | Alt. | Tolerance | Heading |
| Takeoff/Climb | Vy | +10/-5kts | ----- | ----- | ----- |
| Pattern Altitude | ----- | +/-10kts | Pattern Alt. | +/-100’ | ----- |
| Normal Approach | 1.3Vso | +10/-5kts | ----- | ----- | ----- |
| - Touchdown | Vso |  | TD Point | +400’ | Aligned |
| Soft-Field Takeoff | Vx or Vy | +10/-5kts | ----- | ----- | ----- |
| Soft-Field Approach | 1.3Vso | +10/-5kts | ----- | ----- | Aligned |
| - Touchdown | ----- | ----- | ----- | ----- | Aligned |
| Short-Field Takeoff | Vx -> Vy | +10/-5kts | ----- | ----- | ----- |
| Short-Field Approach | 1.3Vso | +10/-5kts | ----- | ----- | Aligned |
| - Touchdown | Vso | ----- | TD Point | +200’ | Aligned |
| Ground Reference |  |  |  |  |  |
| S-Turns | Va | +/-10kts | 600-1000 AGL | +/-100’ | ----- |
| Turns around a pt. | Va | +/-10kts | 600-1000 AGL | +/-100’ | ----- |
| Power-Off Stall |  | ----- | Selected |  | +/-10deg |
| Power-On Stall |  | ----- | Selected |  | +/-10deg |
| Slow Flight | MCA | +10/-0kts | Selected | +/-100’ | +/-10deg |
| Steep Turns | <= Va | +/-10kts | Bank +/-5deg | +/-100’ | +/-10deg |
| Instrument | Selected | +/-10kts | Selected | +/-200’ | +/-20deg |
| Turn to a Hdg | Selected | +/-10kts | Selected | +/-200’ | +/-10deg |
| Rec. Course | Va | +/-10kts | 600-1000 AGL | +/-100’ | ----- |
| Emergency | Best Glide | +/-10kts | ----- | ----- | ----- |
| Commercial Pilot Practical Test Standards |  |  |  |  |  |
| Maneuver | Speed | Tolerance | Alt. | Tolerance | Heading |
| Takeoff/Climb | Vy | +/-5kts | ----- | ----- | ----- |
| Pattern Altitude | ----- | +/-10kts | Pattern Alt. | +/-100’ | ----- |
| Normal Approach | 1.3Vso | +/-5kts | ----- | ----- | ----- |
| - Touchdown | Vso |  | TD Point | +200’ | ----- |
| Soft-Field Takeoff | Vx or Vy | +/-5kts | ----- | ----- | ----- |
| Soft-Field Approach | 1.3Vso | +/-5kts | ----- | ----- | Aligned |
| - Touchdown | ----- | ----- | ----- | ----- | Aligned |
| Short-Field Takeoff | Vx -> Vy | +5/-0kts | ----- | ----- | ----- |
| Short-Field Approach | 1.3Vso | +/-5kts | ----- | ----- | Aligned |
| - Touchdown | Vso | ----- | TD Point | +100’ | Aligned |
| Power-Off 180 | Best Glide | ----- | TD Point | +200’ | Aligned |
| S-Turns | Va | +/-10kts |  |  |  |
| Turns around a pt. | Va | +/-10kts |  |  |  |
| Power-Off Stall |  | ----- | Selected |  | +/-10deg |
| Power-On Stall |  | ----- | Selected |  | +/-5deg |
| Slow Flight | MCA | +5/-0kts | Selected | +/-50’ | +/-10deg |
| Steep Turns | <= Va | +/-10kts | Bank +/-5deg | +/-100’ | +/-10deg |
| Instrument |  |  |  |  | +/-20deg |
| Turn to a Hdg |  | +/-10kts |  |  | +/-10deg |
| Emergency | Best Glide | +/-10kts | ----- | ----- | ----- |
| Chandelles | Va | Above Stall | Selected | Max Gain | +/-10deg |
| Lazy 8’s | Va | +/-10kts | Selected | +/-100’ | +/-10deg |
| Steep Spirals | <=Va | +/-10kts | 3x 360 Turns | ----- | +/-10deg |
| 8’s on Pylons | Va | ----- | Pivotal | ----- | ----- |