II. AREA OF OPERATION: PREFLIGHT PROCEDURES

# A. TASK: PREFLIGHT INSPECTION (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to preflight inspection. This shall include which items must be inspected, the reasons for checking each item, and how to detect possible defects.

- Inspects the airplane with reference to an appropriate checklist.

- Verifies the airplane is in condition for safe flight.

# B. TASK: COCKPIT MANAGEMENT (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to cockpit management procedures.

- Ensures all loose items in the cockpit and cabin are secured.

- Organizes material and equipment in an efficient manner so they are readily available.

- Briefs occupants on the use of safety belts, shoulder harnesses, doors, and emergency procedures.

# C. TASK: ENGINE STARTING (ASEL and ASES)

REFERENCES: FAA-H-8083-3, AC 61-23/FAA-H-8083-25, AC 91-13, AC 91-55; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to recommended engine starting procedures. This shall include the use of an external power source, hand propping safety, and starting under various atmospheric conditions.

- Positions the airplane properly considering structures, surface conditions, other aircraft, and the safety of nearby persons and property.

- Utilizes the appropriate checklist for starting procedure.

# D. TASK: TAXIING (ASEL)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to safe taxi procedures.

- Performs a brake check immediately after the airplane begins moving.

- Positions the flight controls properly for the existing wind conditions.

- Controls direction and speed without excessive use of brakes.

- Complies with airport/taxiway markings, signals, ATC clearances, and instructions.

- Taxies so as to avoid other aircraft and hazards.

# E. TASK: TAXIING AND SAILING (ASES)

REFERENCES: FAA-H-8083-3; USCG NAVIGATION RULES, INTERNATIONAL−INLAND; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to water taxi and sailing procedures.

- Positions the flight controls properly for the existing wind conditions.

- Plans and follows the most favorable course while taxi or sailing considering wind, water current, water conditions and maritime regulations.

- Uses the appropriate idle, plow, or step taxi technique.

- Uses flight controls, flaps, doors, water rudder, and power correctly so as to follow the desired course while sailing.

- Prevents and corrects for porpoising and skipping.

- Avoids other aircraft, vessels, and hazards.

- Complies with seaplane base signs, signals, and clearances.

# F. TASK: BEFORE TAKEOFF CHECK (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to the before takeoff check. This shall include the reasons for checking each item and how to detect malfunctions.

- Positions the airplane properly considering other aircraft/vessels, wind and surface conditions.

- Divides attention inside and outside the cockpit.

- Ensures that engine temperature and pressure are suitable for run-up and takeoff.

- Accomplishes the before takeoff checklist and ensures the airplane is in safe operating condition.

- Reviews takeoff performance airspeeds, takeoff distances, departure, and emergency procedures.

- Avoids runway incursions and/or ensures no conflict with traffic prior to taxiing into takeoff position.