IV. AREA OF OPERATION: TAKEOFFS, LANDINGS, AND GO-AROUNDS

# A. TASK: NORMAL AND CROSSWIND TAKEOFF AND CLIMB (ASEL and ASES)

NOTE: If a crosswind condition does not exist, the applicant's knowledge of crosswind elements shall be evaluated through oral testing.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a normal and crosswind takeoff, climb operations, and rejected takeoff procedures.

- Positions the flight controls for the existing wind conditions.

- Clears the area; taxies into the takeoff position and aligns the airplane on the runway center/takeoff path.

- Retracts the water rudders, as appropriate, (ASES) and advances the throttle smoothly to takeoff power.

- Establishes and maintains the most efficient planing/lift-off attitude and corrects for porpoising and skipping (ASES).

- Lifts off at the recommended airspeed and accelerates to VY.

- Establishes a pitch attitude that will maintain VY +10/-5 knots.

- Retracts the landing gear, if appropriate, and flaps after a positive rate of climb is established.

- Maintains takeoff power and VY +10/-5 knots to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout the takeoff and climb.

- Complies with noise abatement procedures.

- Completes the appropriate checklist.

# B. TASK: NORMAL AND CROSSWIND APPROACH AND LANDING (ASEL and ASES)

NOTE: If a crosswind condition does not exist, the applicant's knowledge of crosswind elements shall be evaluated through oral testing.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a normal and crosswind approach and landing.

- Adequately surveys the intended landing area (ASES).

- Considers the wind conditions, landing surface, obstructions, and selects a suitable touchdown point.

- Establishes the recommended approach and landing configuration and airspeed, and adjusts pitch attitude and power as required.

- Maintains a stabilized approach and recommended airspeed, or in its absence, not more than 1.3 VSO, +10/-5 knots, with wind gust factor applied.

- Makes smooth, timely, and correct control application during the roundout and touchdown.

- Contacts the water at the proper pitch attitude (ASES).

- Touches down smoothly at approximate stalling speed (ASEL).

- Touches down at or within 400 feet (120 meters) beyond a specified point, with no drift, and with the airplane's longitudinal axis aligned with and over the runway center/landing path.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Completes the appropriate checklist.

# C. TASK: SOFT-FIELD TAKEOFF AND CLIMB (ASEL)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a soft-field takeoff and climb.

- Positions the flight controls for existing wind conditions and to maximize lift as quickly as possible.

- Clears the area; taxies onto the takeoff surface at a speed consistent with safety without stopping while advancing the throttle smoothly to takeoff power.

- Establishes and maintains a pitch attitude that will transfer the weight of the airplane from the wheels to the wings as rapidly as possible.

- Lifts off at the lowest possible airspeed and remains in ground effect while accelerating to VX or VY, as appropriate.

- Establishes a pitch attitude for VX or VY, as appropriate, and maintains selected airspeed +10/-5 knots, during the climb.

- Retracts the landing gear, if appropriate, and flaps after clear of any obstacles or as recommended by the manufacturer.

- Maintains takeoff power and VX or VY +10/-5 knots to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout the takeoff and climb.

- Completes the appropriate checklist.

# D. TASK: SOFT-FIELD APPROACH AND LANDING (ASEL)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a soft-field approach and landing.

- Considers the wind conditions, landing surface and obstructions, and selects the most suitable touchdown area.

- Establishes the recommended approach and landing configuration, and airspeed; adjusts pitch attitude and power as required.

- Maintains a stabilized approach and recommended airspeed, or in its absence not more than 1.3 VSO, +10/-5 knots, with wind gust factor applied.

- Makes smooth, timely, and correct control application during the roundout and touchdown.

- Touches down softly with no drift, and with the airplane's longitudinal axis aligned with the runway/landing path.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Maintains proper position of the flight controls and sufficient speed to taxi on the soft surface.

- Completes the appropriate checklist.

# E. TASK: SHORT-FIELD TAKEOFF (CONFINED AREA—ASES) AND MAXIMUM PERFORMANCE CLIMB (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a short-field (confined area ASES) takeoff and maximum performance climb.

- Positions the flight controls for the existing wind conditions; sets the flaps as recommended.

- Clears the area; taxies into takeoff position utilizing maximum available takeoff area and aligns the airplane on the runway center/takeoff path.

- Selects an appropriate take off path for the existing conditions (ASES).

- Applies brakes (if appropriate), while advancing the throttle smoothly to takeoff power.

- Establishes and maintains the most efficient planing/lift-off attitude and corrects for porpoising and skipping (ASES).

- Lifts off at the recommended airspeed, and accelerates to the recommended obstacle clearance airspeed or VX.

- Establishes a pitch attitude that will maintain the recommended obstacle clearance airspeed, or VX,+10/-5 knots, until the obstacle is cleared, or until the airplane is 50 feet (20 meters) above the surface.

- After clearing the obstacle, establishes the pitch attitude for VY, accelerates to VY, and maintains VY, +10/-5 knots, during the climb.

- Retracts the landing gear, if appropriate, and flaps after clear of any obstacles or as recommended by manufacturer.

- Maintains takeoff power and VY +10/-5 to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout the takeoff and climb.

- Completes the appropriate checklist.

# F. TASK: SHORT-FIELD APPROACH (CONFINED AREA—ASES) AND LANDING (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a short-field (confined area ASES) approach and landing.

- Adequately surveys the intended landing area (ASES).

- Considers the wind conditions, landing surface, obstructions, and selects the most suitable touchdown point.

- Establishes the recommended approach and landing configuration and airspeed; adjusts pitch attitude and power as required.

- Maintains a stabilized approach and recommended approach airspeed, or in its absence not more than 1.3 VSO, +10/-5 knots, with wind gust factor applied.

- Makes smooth, timely, and correct control application during the roundout and touchdown.

- Selects the proper landing path, contacts the water at the minimum safe airspeed with the proper pitch attitude for the surface conditions (ASES).

- Touches down smoothly at minimum control airspeed (ASEL).

- Touches down at or within 200 feet (60 meters) beyond a specified point, with no side drift, minimum float and with the airplane's longitudinal axis aligned with and over the runway center/landing path.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Applies brakes, (ASEL) or elevator control (ASEs), as necessary, to stop in the shortest distance consistent with safety.

- Completes the appropriate checklist.

# G. TASK: GLASSY WATER TAKEOFF AND CLIMB (ASES)

NOTE: If a glassy water condition does not exist, the applicant shall be evaluated by simulating the TASK.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to glassy water takeoff and climb.

- Positions the flight controls and flaps for the existing conditions.

- Clears the area; selects an appropriate takeoff path considering surface hazards and/or vessels and surface conditions.

- Retracts the water rudders as appropriate; advances the throttle smoothly to takeoff power.

- Establishes and maintains an appropriate planing attitude, directional control, and corrects for porpoising, skipping, and increases in water drag.

- Utilizes appropriate techniques to lift seaplane from the water considering surface conditions.

- Establishes proper attitude/airspeed, and accelerates to VY, +10/-5 knots during the climb.

- Retracts the landing gear, if appropriate, and flaps after a positive rate of climb is established.

- Maintains takeoff power VY +10/-5 to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout takeoff and climb.

- Completes the appropriate checklist.

# H. TASK: GLASSY WATER APPROACH AND LANDING (ASES)

NOTE: If a glassy water condition does not exist, the applicant shall be evaluated by simulating the TASK.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to glassy water approach and landing.

- Adequately surveys the intended landing area.

- Considers the wind conditions, water depth, hazards, surrounding terrain, and other watercraft.

- Selects the most suitable approach path, and touchdown area.

- Establishes the recommended approach and landing configuration and airspeed, and adjusts pitch attitude and power as required.

- Maintains a stabilized approach and the recommended approach airspeed, +10/-5 knots and maintains a touchdown pitch attitude and descent rate from the last altitude reference until touchdown.

- Makes smooth, timely, and correct power and control adjustments to maintain proper pitch attitude and rate of descent to touchdown.

- Contacts the water in the proper pitch attitude, and slows to idle taxi speed.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Completes the appropriate checklist.

# I. TASK: ROUGH WATER TAKEOFF AND CLIMB (ASES)

NOTE: If a rough water condition does not exist, the applicant shall be evaluated by simulating the TASK.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to rough water takeoff and climb.

- Positions the flight controls and flaps for the existing conditions.

- Clears the area; selects an appropriate takeoff path considering wind, swells surface hazards and/or vessels.

- Retracts the water rudders as appropriate; advances the throttle smoothly to takeoff power.

- Establishes and maintains an appropriate planing attitude, directional control, and corrects for porpoising, skipping, or excessive bouncing.

- Lifts off at minimum airspeed and accelerates to VY, +10/-5 knots before leaving ground effect.

- Retracts the landing gear, if appropriate, and flaps after a positive rate of climb is established.

- Maintains takeoff power VY +10/-5 to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout takeoff and climb.

- Completes the appropriate checklist.

# J. TASK: ROUGH WATER APPROACH AND LANDING (ASES)

NOTE: If a rough water condition does not exist, the applicant shall be evaluated by simulating the TASK.

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to rough water approach and landing.

- Adequately surveys the intended landing area.

- Considers the wind conditions, water, depth, hazards, surrounding terrain, and other watercraft.

- Selects the most suitable approach path, and touchdown area.

- Establishes the recommended approach and landing configuration and airspeed, and adjusts pitch attitude and power as required.

- Maintains a stabilized approach and the recommended approach airspeed, or in its absence not more than 1.3 VSO +10/-5 knots with wind gust factor applied.

- Makes smooth, timely, and correct power and control application during the roundout and touch down.

- Contacts the water in the proper pitch attitude, and at the proper airspeed, considering the type of rough water.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Completes the appropriate checklist.

# K. TASK: FORWARD SLIP TO A LANDING (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to forward slip to a landing.

- Considers the wind conditions, landing surface and obstructions, and selects the most suitable touchdown point.

- Establishes the slipping attitude at the point from which a landing can be made using the recommended approach and landing configuration and airspeed; adjusts pitch attitude and power as required.

- Maintains a ground track aligned with the runway center/landing path and an airspeed, which results in minimum float during the roundout.

- Makes smooth, timely, and correct control application during the recovery from the slip, the roundout, and the touchdown.

- Touches down smoothly at the approximate stalling speed, at or within 400 feet (120 meters) beyond a specified point, with no side drift, and with the airplane's longitudinal axis aligned with and over the runway center/landing path.

- Maintains crosswind correction and directional control throughout the approach and landing sequence.

- Completes the appropriate checklist.

# L. TASK: GO-AROUND/REJECTED LANDING (ASEL and ASES)

REFERENCES: FAA-H-8083-3; POH/AFM.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a go-around/rejected landing.

- Makes a timely decision to discontinue the approach to landing. 3. Applies takeoff power immediately and transitions to climb pitch attitude for VY, and maintains VY+10/-5 knots.

- Retracts the flaps as appropriate.

- Retracts the landing gear, if appropriate, after a positive rate of climb is established.

- Maneuvers to the side of the runway/landing area to clear and avoid conflicting traffic.

- Maintains takeoff power VY +10/-5 to a safe maneuvering altitude.

- Maintains directional control and proper wind-drift correction throughout the climb.

- Completes the appropriate checklist.