VI. AREA OF OPERATION: GROUND REFERENCE MANEUVERS

NOTE: The examiner shall select at least one TASK.

# A. TASK: RECTANGULAR COURSE (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to a rectangular course.

- Selects a suitable reference area.

- Plans the maneuver so as to enter a left or right pattern, 600 to 1,000 feet AGL (180 to 300 meters) at an appropriate distance from the selected reference area, 45° to the downwind leg.

- Applies adequate wind-drift correction during straight-and-turning flight to maintain a constant ground track around the rectangular reference area.

- Divides attention between airplane control and the ground track while maintaining coordinated flight.

- Maintains altitude, ±100 feet (30 meters); maintains airspeed, ±10 knots.

# B. TASK: S-TURNS (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to S-turns.

- Selects a suitable ground reference line.

- Plans the maneuver so as to enter at 600 to 1,000 feet (180 to 300 meters) AGL, perpendicular to the selected reference line.

- Applies adequate wind-drift correction to track a constant radius turn on each side of the selected reference line.

- Reverses the direction of turn directly over the selected reference line.

- Divides attention between airplane control and the ground track while maintaining coordinated flight.

- Maintains altitude, ±100 feet (30 meters); maintains airspeed, ±10 knots.

# C. TASK: TURNS AROUND A POINT (ASEL and ASES)

REFERENCE: FAA-H-8083-3.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to turns around a point.

- Selects a suitable ground reference point.

- Plans the maneuver so as to enter left or right at 600 to 1,000 feet (180 to 300 meters) AGL, at an appropriate distance from the reference point.

- Applies adequate wind-drift correction to track a constant radius turn around the selected reference point.

- Divides attention between airplane control and the ground track while maintaining coordinated flight.

- Maintains altitude, ±100 feet (30 meters); maintains airspeed, ±10 knots.