IX. AREA OF OPERATION: BASIC INSTRUMENT MANEUVERS

NOTE: The examiner shall select task E and at least two other TASKs.

# A. TASK: STRAIGHT-AND-LEVEL FLIGHT (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to attitude instrument flying during straight-and-level flight.

- Maintains straight-and-level flight solely by reference to instruments using proper instrument cross-check and interpretation, and coordinated control application.

- Maintains altitude, ±200 feet (60 meters); heading, ±20°; and airspeed, ±10 knots.

# B. TASK: CONSTANT AIRSPEED CLIMBS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to attitude instrument flying during constant airspeed climbs.

- Establishes the climb configuration specified by the examiner.

- Transitions to the climb pitch attitude and power setting on an assigned heading using proper instrument cross-check and interpretation, and coordinated control application.

- Demonstrates climbs solely by reference to instruments at a constant airspeed to specific altitudes in straight flight and turns.

- Levels off at the assigned altitude and maintains that altitude, ±200 feet (60 meters); maintains heading, ±20°; maintains airspeed, ±10 knots.

# C. TASK: CONSTANT AIRSPEED DESCENTS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to attitude instrument flying during constant airspeed descents.

- Establishes the descent configuration specified by the examiner.

- Transitions to the descent pitch attitude and power setting on an assigned heading using proper instrument cross-check and interpretation, and coordinated control application.

- Demonstrates descents solely by reference to instruments at a constant airspeed to specific altitudes in straight flight and turns.

- Levels off at the assigned altitude and maintains that altitude, ±200 feet (60 meters); maintains heading, ±20°; maintains airspeed, ±10 knots.

# D. TASK: TURNS TO HEADINGS (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to attitude instrument flying during turns to headings.

- Transitions to the level-turn attitude using proper instrument crosscheck and interpretation, and coordinated control application.

- Demonstrates turns to headings solely by reference to instruments; maintains altitude, ±200 feet (60 meters); maintains a standard rate turn and rolls out on the assigned heading, ± 10°; maintains airspeed, ±10 knots.

# E. TASK: RECOVERY FROM UNUSUAL FLIGHT ATTITUDES (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to attitude instrument flying during unusual attitudes.

- Recognizes unusual flight attitudes solely by reference to instruments; recovers promptly to a stabilized level flight attitude using proper instrument cross-check and interpretation and smooth, coordinated control application in the correct sequence.

# F. TASK: RADIO COMMUNICATIONS, NAVIGATION SYSTEMS/FACILITIES, AND RADAR SERVICES (ASEL and ASES)

REFERENCES: FAA-H-8083-3, FAA-H-8083-15, AC 61-23/FAA-H-8083-25.

Objective. To determine that the applicant:

- Exhibits knowledge of the elements related to radio communications, navigation systems/facilities, and radar services available for use during flight solely by reference to instruments.

- Selects the proper frequency and identifies the appropriate facility.

- Follows verbal instructions and/or navigation systems/facilities for guidance.

- Determines the minimum safe altitude.

- Maintains altitude, ±200 feet (60 meters); maintains heading, ±20°; maintains airspeed, ±10 knots.