# 180° POWER-OFF ACCURACY APPROACH AND LANDING (ASEL)

Objective:

To instill in the student the judgment and procedures necessary for accurately flying the airplane, without power, to a safe landing. To further develop judgment in estimating distances and glide ratios.

Before Flight:

- Review appropriate aircraft performance information

Maneuver Elements:

- Clear the area

- Choose forced landing area (runway)

- Configure aircraft: landing checklist (CCGUMPS, no flaps), insure gear extended, approx. 1000’ AGL downwind for landing

- Select outside references (e.g. aim point markers)

- Throttle to idle abeam touchdown

- Establish and maintain best glide speed or 1.4 VSO

- Select aim point ahead of touchdown point (compensate for ground effect)

- Position and adjust base leg as necessary and compensate for wind

- Flaps or forward slip as necessary for landing on touchdown point

- Land at touchdown point beyond and within 200’

- How to measure? Runway edge lights are spaced 50’

Aircraft Setup:

Flaps as required, gear down, propeller Full

Downwind: (C172RG: 18” Hg, 2700 RPM)

1.4 VSO:  (C172RG: 75 KIAS 0º Flaps)

Things to Avoid (Common Errors)

- Downwind leg too far from runway/landing area

- Overextension of downwind leg resulting from tailwind

- Inadequate compensation for wind drift on base leg

- Skidding turns in an effort to increase gliding distance

- Failure to lower landing gear in retractable gear airplanes

- Attempting to “stretch” the glide during undershoot

- Premature flap extension/landing gear extension

- Use of throttle to increase the glide instead of merely clearing the engine

- Forcing the airplane onto the runway to avoid overshooting the touchdown point

Diagrams:

References:

Airplane Flying Handbook - Chapter 8-23

Completion Standards:

The lesson is complete when the student demonstrates knowledge through a written or oral exam and is able to perform a 180° power-off approach to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.