# AIR TRAFFIC CONTROL CLEARANCES

Objective:

To familiarize the student with air traffic control clearances.

Content:

- Pilot and controller responsibilities

  - ATC clearance

  - Pilot responsibilities

  - Controller responsibilities

  - Tower en route control (TEC)

  - Clearance void time

- Copying Clearances

  - Elements of an IFR clearance

    - Aircraft ID

    - Clearance limit

    - Departure procedure

    - Route of flight

    - Altitude data

    - Holding instructions

    - Additional information

    - Departure frequency

    - Transponder code

- Read-Back

- Interpretation of an ATC clearance

- Setting of communication and navigation frequencies

References:

14 CFR Part 91, AIM

Instrument Flying Handbook, FAA-H-8083-15

Instrument Rating ACS

Completion Standards:

The lesson is complete when the instructor determines that the student has adequate knowledge of air traffic control clearances by giving an oral or written exam.

Instructors Notes

- Pilot and controller responsibilities to include tower en route control, and clearance void times

  - ATC clearance

    - Authorization by ATC, for the purpose of preventing collision between known aircraft, for the aircraft to proceed under specified conditions within controlled airspace

    - Not an authorization for a pilot to deviate from any rule, regulation or minimum altitude nor to conduct any unsafe operation of the aircraft

  - Pilot responsibilities

    - PIC is directly responsible for and the final authority as to the operation of the aircraft (14 CFR Sec. 91.3)

    - In emergencies, PIC may deviate from 14 CFR to the extent necessary to maintain safety and then, if requested, send a written report to the Administrator

    - Pilot is always responsible to see and avoid traffic when operating in VMC

    - To operate under IFR in controlled airspace (Sec. 91.173)

      - File an IFR flight plan

      - Obtain an ATC clearance

  - Controller responsibilities

    - Issuing appropriate clearances

    - In IFR clearances, assigning altitudes above the minimum IFR altitudes in controlled airspace

    - Ensuring pilot acknowledgement of clearances

    - Help ensure that read backs are correct, or make appropriate corrections as needed

  - Tower en route control (TEC)

    - ATC program to provide an overflow resource in the low altitude system for aircraft proceeding from one metropolitan area to another

    - TEC flights are entirely within the approach control airspace of multiple terminal areas

    - Some TEC routes are published in the AFD

    - Generally for non-turbojet aircraft operating below 10,000 feet on short flights with ETE < 2 hours

    - Enter "TEC" in flight plan remarks section

  - Clearance void time

    - Provision in a clearance for that clearance to be void (automatically cancelled) if aircraft not airborne by a specified time, the clearance void time

    - May be received when operating from an airport without a control tower

    - If not airborne prior to the clearance void time, advise ATC as soon as possible

      - ATC normally advises pilot of time allotted to notify ATC in this event (cannot exceed 30 minutes)

      - If ATC in not notified within 30 minutes after aircraft misses clearance void time, then aircraft is considered overdue and search and rescue procedures are begun

    - Other IFR operations are suspended until aircraft contacts ATC or until 30 minutes after clearance void time

- Correct and timely copying of an ATC clearance

  - Be ready to copy, pencil and paper in hand, when requesting clearance

  - Copy clearance as it is being read by controller

  - Use clearance shorthand

  - Elements of an IFR clearance (ATC clearance to operate under IFR) to expect to hear and copy

    - Aircraft ID

    - Clearance limit

      - Usually the destination airport

      - Occasionally ATC issues short-range clearance to fix within or just outside the terminal area and provides frequency on which the long-range clearance will be received

    - Departure procedure

    - Route of flight

      - Normally "as filed," but may be changed by ATC to comply with established flow patterns or preferred routes

      - Pilot responsibility to notify ATC if unable to comply with clearance (e.g., radio equipment unable to receive necessary signals)

    - Altitude data

      - Normally an altitude to maintain

      - Cruise instead of maintain assigns a block of airspace from minimum IFR altitude to the specified cruise altitude

        - Within this block climb, descent and level-off are at pilot's discretion

        - Once pilot begins descent and verbally reports leaving an altitude, s/he may not climb back to that altitude without further clearance

    - Holding instructions

      - ATC controller responsibility if clearance is not to destination airport and delay expected

      - If no instruction and holding pattern charted, then hold as charted

      - If unclear, ask ATC for holding instructions prior to reaching fix

      - If unable to ask ATC, then fly standard holding pattern on course on which fix approached

      - Report time/altitude reaching clearance limit and report leaving clearance limit

    - Additional information if necessary to facilitate understanding IFR clearance

    - Departure frequency

    - Transponder code

- Correct and timely read-back of an ATC clearance, using standard terminology

  - Promptly read back the clearance you just copied

  - Inform ATC of any items you missed

  - Correct any errors and read back those items again for confirmation

  - Note "read back correct" from controller confirming correctness

  - Reading back of initial clearance does not imply acceptance

  - Study pilot/controller glossary for standard terminology

- Correct interpretation of an ATC clearance and, when necessary, request for clarification, verification or change

  - Study each clearance element

    - If in doubt, request clarification

    - If unable, request amendment

  - It is the pilot's responsibility to request an amended clearance if the pilot decides that the clearance ATC issued would cause deviation from a rule or regulation or place the aircraft in jeopardy

- Setting of communication and navigation frequencies in compliance with an ATC clearance

  - Good cockpit resource management includes proper set-up of all communication and navigation equipment prior to takeoff

  - Recommend left-to-right top-to-bottom approach for complete set up

    - Set HI bug on initial assigned departure heading

    - Set top, primary, COMM radio to tower or UNICOM frequency and set departure frequency in standby position

    - Enter and check assigned route, all waypoints, including DP, in GPS and activate flight plan (e.g., GNS530)

    - Using primary navigation system (e.g., GPS): enter, check frequency and ID (if in range) of first enroute station, and set OBS to initial assigned course

    - Set secondary COMM radio to departure frequency

    - Set secondary NAV radio to the frequency of the approach facility (e.g., ILS) that would be used in case of emergent return to departure airport in IMC

    - Check transponder set to assigned code and on standby mode until ready for takeoff, then switch to ALT mode

    - Recommend: turn rotatable ADF card to set initial assigned altitude at top as reminder