# AIRPLANE FLIGHT CONTROLS

Objective:

To familiarize the student with the operation of the primary control systems on an airplane so that he may better the effects of control movements.

Content:

- Primary Flight Controls

  - Ailerons

    - Adverse Yaw

    - Types of Ailerons

  - Elevator

    - Standard Elevators

    - T-Tail Eleators

    - Stabilator

  - Rudder

- Trim Controls

  - Trim Tabs

    - Balance Tabs

    - Antiservo Tabs

    - Ground Adjustable Tabs

- Flaps

  - Plain

  - Split

  - Slotted Flap

  - Fowler Flaps

References:

Pilot’s Handbook of Aeronautical Knowledge - Chapter 4

Completion Standards:

The lesson is complete when the instructor determines the student has proper knowledge of the elevator, ailerons, and rudder by giving an oral or written exam.

Instructor Notes:

- Overview

  - Chord Line – An imaginary straight line drawn through an airfoil from the leading to the trailing edge

  - Camber – The characteristic curve of an airfoil’s upper and lower surfaces

    - The upper camber is more pronounced while the lower camber is relatively flat

      - This causes the velocity of the airflow immediately above the wing to be higher than below

    - The more curved the upper surface, the more lift is generated

- Primary Flight Controls

  - Primary flight controls are those required to safely control an airplane during flight

  - Secondary control systems improve performance characteristics or relieve excessive control forces

    - Wing Flaps, and Trim Systems

  - Ailerons

    - Control roll about the longitudinal axis

    - Operated through steel push rods

    - How they Work

      - The ailerons are interconnected, operating simultaneously in opposite directions

        - Moving the controls right causes the right aileron to deflect upward and the left downward

          - The upward deflection decreases the camber resulting in decreased lift

          - The downward deflection increases the camber resulting in increased lift

        - The increased lift on the left and decreased lift on the right cause the roll to the right

      - The airplane turns because the banking of the wings creates horizontal lift

        - With the wings banked, the lift has a horizontal component as well as a vertical component

          - The horizontal component counteracts the CF pulling the airplane straight ahead

        - The change in relative wind pushes the vertical stabilizer around turning the airplane

  - Adverse Yaw

    - Since the downward deflected aileron produces more lift, it also produces more drag (Induced)

      - The added drag attempts to yaw the nose in the direction of the raised wing (Adverse Yaw)

    - Rudder is used to counteract

      - The amount needed is greatest at low AS, high AOA, and with large aileron deflections

  - Types of Ailerons

    - Differential Ailerons

      - One aileron is raised a greater distance than the other is lowered

        - This produces an increased drag on the descending wing, reducing adverse yaw

    - Frise-Type Ailerons

      - When pressure is applied, the aileron being raised pivots on an offset hinge

        - This projects the leading edge of the aileron into the airflow and creates drag

          - This helps equalize the drag created by the lowered aileron reducing adverse yaw

      - A slot forms so air flows smoothly over the low aileron, making it more effective at high AOA

    - In both types of ailerons, adverse yaw is not eliminated and coordinated rudder is still needed

  - Elevator

    - Control pitch about the lateral axis

    - Operated through steel push rods

    - How It Works

      - Pulling the controls backward deflects the trailing edge up

        - This decreases the camber of the elevator and creates downward aerodynamic force

        - The overall effect causes the tail to move down and the nose to move up (About the CG)

          - Strength is determined by the distance between the CG and horizontal tail surface

      - Moving the controls forward deflects the trailing edge of the elevator surface down

        - This increases the camber of the elevator and creates more lift

        - This moves the tail upward and pitches the nose down (Also about the CG)

    - Types of Elevators

      - T-Tail

        - The elevator is above most effects of downwash from the propeller and airflow around the fuselage and wings in normal flight

          - Operating in this air makes for consistent control movements in most flight regimes

        - Popular on light as well as large airplanes (removes from exhaust ) and sea planes

        - At slow speeds, the elevator must be moved through a larger number of degrees to raise the nose a given amount as compared to a conventional tail aircraft

          - The conventional has the downwash from the prop assisting in raising the nose

      - Stabilator

        - A moveable horizontal surface which combines the horizontal stabilizer and the elevator

        - When the controls are pulled back, the stabiliator’s trailing edge raises, rotating the nose up

        - Pushing forward lowers the trailing edge and pitches the nose down

        - Antiservo tabs are incorporated on the trailing edge to decrease sensitivity

          - This tab moves in the same direction of the trailing edge of the stabilator

          - The movement of the anitservo tab causes it to be deflected into the slipstream

            - This provides a resistance without which the pilot would overcontrol the airplane

  - Rudder

    - Control yaw about the vertical axis

    - Operated through cables

    - How it Works

      - When the rudder is deflected, a horizontal force is exerted in the opposite direction

        - Pushing the left pedal moves the rudder left

          - This alters the airflow around the vertical stabilizer creating a sideward lift moving the tail right and yawing the nose to the left

          - Rudder effectiveness increases with speed

        - Large deflections may be necessary at low speeds and small deflections at high speeds

        - In a propeller driven airplane, any slipstream flowing over the rudder increases effectiveness

    - The primary purpose is to counteract adverse yaw and provide directional control (coordination too)

- Trim Controls

  - Trim systems are used to relieve the need to maintain constant pressure on the flight controls

  - They usually consist of cockpit controls and small hinged devices attached to the trailing edge of primary control surfaces

    - Minimize workload by aerodynamically assisting movement and position of the controls attached to

  - Trim Tabs

    - Most common installation, a single trim tab attached to the trailing edge of the elevator

    - Operation

      - Operated manually through small, vertically mounted control wheel (Trim crank can be found)

      - The trim tab moves in the opposite direction of the elevator surface

      - Placing the trim in full nose-down moves the tab to its full up position

        - With the tab up, into the airstream, the airflow over the tail forces the elevator down

          - This causes the tail to move up and results in a nose-down pitch change

          - In the full nose-up position, the tab moves to its full down position

        - Air flowing under the tail hits the tab forcing the elevator up, reducing the elevator’s AOA

          - This causes the tail to move down and results in a nose-up pitch change

    - Operating

      - Establish the desired power, pitch attitude, and configuration then trim to relieve pressures

      - Any time power, pitch attitude, or configuration is changed, retrim for the new condition

    - Balance Tabs

      - Look and function just like trim tabs, but the balance tab is coupled to the control surface rod

        - When the controls are deflected, the tab automatically moves in the opposite direction

          - Any time the control surface is deflected, the tab moves opposite and eases the load

      - If the linkage is adjustable from the cockpit, the tab acts as both a trim and balance tab

    - Antiservo Tabs

      - Serves to decrease sensitivity and also as a trim device to relieve and maintain control pressure

      - Operation

        - When the trailing edge of the stabilator moves up, the trailing edge of the tab moves up

        - Works exactly the same as the balance tab, but it moves in the same direction (not opposite)

    - Ground Adjustable Tabs

      - Metal trim tab on the rudder bent in either direction while on the ground to apply a trim force

        - Displacement is found through trial and error

- Flaps

  - The most common high lift devices used on practically all airplanes

    - Attached to the trailing edge of each wing to increase induced drag and lift for any given AOA

    - Important Functions

      - Allow a compromise between high cruise/low landing speeds since they can extend/retract

      - Permit a slower landing speed decreasing landing distance

      - Permit a steeper angle of descent without increasing speed (allowing safe obstacle clearance)

      - May be used to shorten the T/O distance and provide a steeper climb path

  - Plain

    - Simplest of the types

    - They increase the camber, resulting in a significant increase in the coefficient of lift at a given AOA

    - Drag is greatly increased the center of pressure moves aft resulting in a nose down pitching moment

  - Split

    - Deflected from the lower surface of the airfoil and produces slightly greater increase in lift that plain

      - However, more drag is produced because of the turbulent airflow behind the airfoil

  - Slotted Flap

    - Most popular on airplanes today

    - Increase the lift coefficient significantly more than plain/split flaps

    - When lowered, it forms a duct between the flap well in the wing and the flap’s leading edge

      - High energy air from the lower surface is ducted to the upper surface

        - This accelerates the upper boundary layer and delays airflow separation giving a higher CL

  - Fowler Flaps

    - A type of slotted flap which changes the camber of the wing and increases the wing area

    - It slides backward on tracks and then retracts downward

    - The first portion of its extension increases drag very little but increases lift a great deal

      - As the extension continues, the flap drops downward, and drag increases with little lift increase

    - Provide the greatest amount of lift with the least amount of drag, and creates the greatest change in pitching moment

  - Flap Control

    - Controllable by the pilot, either manually, electrically, or hydraulically

      - The flaps are controlled by a 3 position operating switch