# AIRPORT/SEAPLANE BASE, RUNWAY AND TAXIWAY SIGNS, MARKINGS, AND LIGHTING (ASEL and ASES)

Objective:

To familiarize the student with the various markings and lights on an airport.

Content:

- Runway numbering conventions

  - VFR vs. IFR runways

  - Displaced threshold

  - Blast pad/stopway

- Taxiway Markings

  - Vehicle roadway markings

  - Solid white lines are boundaries

  - Dashed white lines separate lanes within roadway

  - VOR receiver checkpoint markings

  - Painted circle w/ arrow in the middle pointing in direction of checkpoint azimuth

  - Non-movement area boundary markings

  - Two yellow lines, one solid, one dashed

- Airport Signs – 6 Types (see PHAN Figures 12-3 & 12-4)

  - Mandatory Instruction Signs

  - White letters, red background

  - Entrance to runway, critical area, or prohibited area

- Location signs

  - Yellow letters, black background, yellow border

  - Taxiway/runway location, boundary of runway, or ILS critical area

- Direction Signs

  - Black letters, yellow background

- Destination Signs

- Information Signs

- Runway Distance Remaining Signs

- Airport Lighting

  - Airport Beacon

  - Approach Light Systems

    - Visual Glideslope Indicators

    - VASI

    - PAPI

  - Runway Lighting

    - REILs

    - Runway edge lights

    - In-Runway lighting

  - Taxiway Lights

  - Obstruction Lights

  - Control of Airport Lighting

- Wind direction Indicators

Things to Avoid (Common Errors):

- Failure to comply with airport, runway, taxiway sings and markings

- Failure to comply with airport, runway and taxiway lighting

- Failure to use proper runway incursion avoidance procedures

References:

AOPA Safety Advisor No. 2 “Operations at Towered Airports” in Appendix

AIM

Completion Standards:

The lesson is complete when the instructor determines that the student has adequate knowledge of airport and runway markings and lights by giving an oral or written exam.

Instructor Notes:

- Runway Incursion Avoidance

  - Be aware of the airplane’s position and be aware of other aircraft and vehicle operations on the airport

  - Read back all runway crossing and/or hold short instructions

  - Review the Taxi Diagrams as part of preflight planning, before landing, and while taxiing as needed

  - Know airport signage

  - Review NOTAMs for information regarding runway/taxiway closures and construction areas

  - Request progressive taxi instructions from ATC when unsure of the taxi route

  - Check for traffic before crossing any runway hold line and before entering a taxiway

  - Turn on aircraft lights and the rotating beacon or strobe lights when taxiing

  - When landing, clear the runway as soon as possible, and wait for taxi instructions before moving

  - Study and use proper phraseology in order to understand and respond to ground control instructions

  - Write down complex taxi instructions at unfamiliar airports

  - Failure to use proper runway incursion avoidance procedures

    - Do not cross hold short lines until you have been cleared for T/O or to taxi onto/across a runway

    - Without a control tower, look both ways and announce your intentions before proceeding

    - Never taxi onto the runway when there is a plane on final approach

- Runway Markings

  - General

    - There are three types of markings for runways:

      - Visual; Nonprecision Instrument; Precision Instrument

  - Runway Designators

    - Purpose - To identify/differentiate runways from the approach end

        - The whole number nearest one-tenth the magnetic azimuth of the runway (To Magnetic N)

        - Letters differentiate between left (L), right (R), and center (C) runways

    - Markings - Large white numbers

  - Runway Centerline Marking

    - Purpose - Identifies the center of the runway providing alignment guidance during T/O and LDG

    - Markings - A line of uniformly spaced stripes and gaps

  - Runway Aiming Point Markings

    - Purpose - Serves as a visual aiming point for a landing aircraft

    - Markings - Broad white stripe on each side of the centerline, approx 1,000’ from the threshold

  - Runway Touchdown Zone Markers

    - Purpose - Identifies the touchdown zone for LDG ops and provide distance info in 500’ increments

    - Markings - Groups of 1, 2, and 3 rectangular bars in pairs about the runway centerline

  - Runway Side Stripe Markings

    - Purpose - Delineate the edges of the runway providing a contrast between the runway and shoulder

    - Markings - Continuous white stripes located on each side of the runway

  - Runway Shoulder Markings

    - Purpose – May be used with side stripes to identify pavement areas not intended for aircraft use

    - Markings - Yellow stripes

  - Runway Threshold Markings

    - Purpose – Identifies the beginning of the runway available for landing

    - Markings - 8 longitudinal stripes of uniform dimensions placed about the centerline (A)

      - The number of stripes is related to runway width (B):

    - Threshold can be Displaced or Relocated

      - Displaced Threshold (DT)

        - Explanation

          - A threshold at a point on the runway other than the designated beginning of the runway

          - Reduces the amount of runway available for landings

          - The portion behind it is for T/Os either direction and LDGs from the opposite direction

            - Can be used for taxiing, takeoff, and landing rollout

        - Markings

          - A 10’ wide white threshold bar is located across the runway at the displaced threshold

          - White arrow heads are located across the runway just prior to the threshold bar

          - White arrows are down the centerline between the runway and displaced threshold

          - Demarcation Line

            - Purpose - Delineates the DT from a blast pad/stopway/ taxiway prior to the runway

            - Markings - 3 feet wide and yellow

          - Chevrons

            - Purpose - Show areas aligned with the runway that are unusable for taxi, T/O, LDG

            - Markings - Yellow arrows

      - Relocated Threshold

        - Explanation

          - Sometimes construction or other activities require the threshold to be relocated

          - A NOTAM should be issued identifying the portion of the runway is closed

            - EX. 10/28 W 900 CLSD

        - Markings – Identification can vary, as the duration of the relocation varies

          - Common practice is to use a 10’ wide white threshold bar across the runway

          - Runway lights between the old threshold and new threshold will not be illuminated

          - Runway markings in this area may or may not be showing

  - Blast pad/Stopway Area

    - The blast pad area is an area where a propeller or jet blast can dissipate without creating a hazard

    - Stopway is paved in to provide space to decelerate/stop in the event of an aborted T/O (Chevrons)

- Taxiway Markings

  - General

    - Taxiways should have centerline/runway holding position markings whenever intersecting a runway

    - Edge markings separate the taxiway from areas not for aircraft use or define taxiway edges

    - May have shoulder/hold position markings for ILS critical areas and taxiway/taxiway intersections

  - Taxiway Centerline Markings

    - Normal Centerline

      - Purpose - Provides a visual cue to permit taxiing along a designated path (aircraft kept over line)

        - Markings - A single continuous yellow line that is 6” – 12” wide

    - Enhanced Centerline

      - Purpose - Same as above but at larger commercial airports to warn that a runway hold position marking is being approached and unless cleared to cross the aircraft should prepare to stop

      - Markings - A parallel line of yellow dashes on either side of the normal taxiway centerline

        - Centerlines are enhanced for a max of 150’ prior to a runway holding position marking

  - Taxiway Edge Markings

    - Purpose - Defines the edge of the taxiway (usually when edge doesn’t match up with pavement)

    - 2 types of markings, depending on whether the aircraft is supposed to cross the taxiway edge

      - Continuous Markings

        - Purpose - Define the taxiway edge from the shoulder/paved surface not for use by aircraft

        - Markings - Continuous double yellow line with each line at least 6’ wide and 6’ apart

      - Dashed Markings

        - Purpose - Define the taxiway edge when adjoining pavement is intended for aircraft (Apron)

        - Markings - Broken double yellow line ( 6” wide/spaced 6” apart, 15’ long and 25’ apart)

  - Taxi Shoulder Markings

    - Purpose - Paved shoulders prevent erosion but they may not support aircraft

    - Markings - Taxiway edge markings will usually define this area

      - If confusion exists to the side of use, yellow shoulder markings are used

  - Surface Painted Taxiway Direction Signs

    - Purpose - When it isn’t possible to offer direction signs at intersections, or to supplement such signs

    - Markings – Surface painted location signs with a yellow background and black inscription

      - Adjacent to the centerline with signs indicating left turns on the left side of the centerline and vice versa

  - Surface Painted Location Signs

    - Purpose - Supplement location signs alongside the taxiway assisting in confirming the designation of the taxiway on

    - Markings - Black background with a yellow inscription, R of center

  - Geographic Position Markings

    - Purpose - Identifies aircraft location during low visibility operations

    - Markings - Left of the taxiway centerline in the direction of taxiing

      - A circle with an outer black ring, inner white ring and a pink circle

        - When on dark pavements the white/black ring are reversed

      - Designated with either a number or a number and a letter

        - Number corresponds with consecutive position on the route

- Holding Position Markings

  - General

    - Show where an aircraft is supposed to stop when approaching a runway (Hold on the solid side)

    - 4 yellow lines (2 Solid/2 Dashed) spaced 6” or 12” apart across the width of the taxiway/runway

      - 3 locations where runway hold lines are encountered: 1Taxiways; 2Runways; 3Approach Areas

  - Runway Holding Position Markings on Taxiways

    - Purpose - Identify where to stop without a clearance onto the runway

      - Always stop so that no part extends beyond the hold markers

      - Don’t cross w/o clearance/ample separation at uncontrolled airports

  - Runway Holding Position Markings on Runways

    - Purpose - Only installed if normally used for LAHSO or taxiing operations

        - Must stop before markings/exit prior to reaching the position

    - Markings - Sign (white inscription/red background) next to hold markings

      - Markings are placed on the runway prior to the intersection

  - Taxiways Located in Runway Approach Area

    - Purpose - Hold aircraft on taxiway so it does not interfere with ops

    - Holding Position Markings for Instrument Landing System (ILS)

      - Purpose – Hold aircraft when the ILS critical area is being protected

      - Markings - 2 yellow solid lines 2’ apart joined by pairs of solid lines 10’ apart across the taxiway

    - Holding Position Markings for Taxiway/Taxiway Intersections

      - Purpose - Installed on taxiways where ATC normally holds aircraft short of a taxiway intersection

      - Markings - Single dashed line extending across the width of the taxiway

        - If requested to HS of a taxiway w/o marking, provide adequate clearance from the taxiway

    - Surface Painted Holding Position Signs

      - Purpose - Used to supplement the signs located at the holding position

        - Normally used when the width of the holding position on the taxiway is greater than 200’

      - Markings - Red background/white inscription, L of center, on the holding side, prior to hold lines

- Other Markings

  - Vehicle Roadway Markings

    - Purpose - Used to define a path for vehicle ops on or crossing areas also intended for aircraft

    - Markings - White solid line delineates each edge and a dashed line separates lanes

      - In lieu of the solid lines, zipper markings may be used to delineate edges

  - VOR Receiver Checkpoint Markings

    - Purpose - Allow the pilot to check aircraft instruments with navigational aid signals

    - Markings - A painted circle with an arrow in the middle (Arrow is aligned toward the facility)

      - Located, with a sign, on the apron/taxiway

      - Sign shows the VOR station ID letter, course for the check, and DME data (if necessary)

      - Black letters/numerals on a yellow background

  - Nonmovement Area Boundary Markings

    - Purpose - Delineates the movement area (The area under air traffic control)

    - Markings - 2 yellow lines (one solid and one dashed) 6” in width

      - Solid line is the nonmovement area side, the dashed line is the movement area side

  - Marking and Lighting of Permanently Closed Runways

    - Purpose - For runways and taxiways which are permanently closed

    - Markings - The lighting circuits will be disconnected

      - The runway threshold, designation, and touchdown markings are obliterated

      - Yellow crosses are placed at each end of the runway and at 1,000’ intervals

  - Temporarily Closed Runways and Taxiways

    - Purpose - To provide a visual indication to pilots that a runway is temporarily closed

    - Markings - Yellow crosses are placed on the runway at each end

      - A raised lighted yellow cross may be placed on each end of the runway instead

      - A visual indication may not be present depending on the reason for closure, duration of the closure, configuration and the existence and hours of operation of an airport control tower

        - Check NOTAMs and the ATIS for information

      - Closed taxiways are treated as hazardous areas and blockaded; no part of the aircraft may enter

        - As an alternative, a yellow cross may be installed at each entrance to the taxiway

- Airport Signs

  - General

    - Six types of signs installed on airfields

      - Mandatory Instruction; Location; Direction; Destination; Info; Runway Distance Remaining

  - Mandatory Instruction Signs

    - Purpose - Denote entrance to runway or critical area/area where aircraft are prohibited

    - Markings - Red background with a white inscription

    - Typical Mandatory Signs and Applications

      - Runway Holding Position Sign

        - Located at the hold position on taxiways intersecting runways/runways intersecting runways

        - The sign states the designation of the intersecting runway

      - Runway Approach Area Holding Position Sign

        - Used when necessary to hold aircraft on a taxiway in a runway approach/departure area so it doesn’t interfere with runway ops

      - ILS Critical Area Holding Position Sign

        - ILS system is being used, and it’s necessary to hold at a location other than the Hold Markers

        - The sign will have the inscription “ILS” and will be located adjacent to the holding position marking on the taxiway

      - No Entry Sign

        - Prohibits an aircraft from entering an area

        - Typically on a taxiway intended to be used only in 1 direction or vehicle intersections of that may be mistaken as a taxiway/movement surface

  - Location Signs

    - Purpose - Identify either a taxiway or runway on which the aircraft is located

      - Other location signs provide a visual cue to assist in determining when an area has been exited

    - Taxiway Location Sign

      - Purpose - Along taxiways to indicate location

      - Markings - Black background with yellow inscription and border

    - Runway Location Sign

      - Purpose - Complement compass info; typically installed where the proximity of runways to one another could cause confusion as to which runway the pilot is on

      - Markings - Black background with a yellow inscription (Runway #)/ border

    - Runway Boundary Sign

      - Purpose - Provides a visual cue to use as a guide in deciding when “clear of the runway”

        - Adjacent to the hold markings on the pavement

        - Visible when exiting the runway

      - Markings - Yellow background/black inscription depicting the hold markings

    - ILS Critical Area Boundary Sign

      - Purpose - Provides a visual cue to use as a guide in deciding when clear of the ILS critical area

        - Adjacent to ILS hold markings and can be seen leaving the critical area

      - Markings - Yellow background/black inscription depicting ILS hold markings

  - Direction Signs

    - Purpose - Identify the designations of intersecting taxiways leading out of an intersection

        - Designations and their arrows are arranged clockwise from the 1st taxiway on the pilot’s left

    - Markings - Yellow background/black inscription with an arrow indicating the turn direction

  - Destination Signs

    - Purpose - Indicates a destination on the airport

      - Destinations commonly shown are

        - Runways

        - Aprons

        - Terminals

        - Military Areas

        - Civil Aviation Areas

        - Cargo Areas

        - International Areas

        - FBOs

    - Markings - Yellow background/black inscription indicating a destination on the airport

      - Always have an arrow showing the direction of the taxiing route to that destination sign

  - Information Signs

    - Purpose - Used to provide a pilot with information on such things as:

      - Areas the tower can’t see, radio frequencies, and noise abatement procedures

    - Markings - Yellow Background with a black inscription

  - Runway Distance Remaining Signs

    - Purpose - Used to inform the pilot the amount of distance remaining on the runway

      - The number on the sign indicates the thousands of feet of landing runway remaining

    - Markings - Black background/white numeral inscription one 1/both sides of the runway

  - CE - Failure to comply with airport, runway, taxiway sings and markings

    - Make sure you know the meaning and purpose of all the signs and markings

      - If unsure, stop and ask ATC for clarification

- Airport Lighting and Other Airport Visual Aids

  - Approach Light Systems (ALS)

    - Purpose - The basic means to transition from IFR to VFR for landing

    - Explanation - A configuration of signal lights staring at the landing threshold and extending into the approach area

  - Visual Glideslope Indicators

    - Visual Approach Slope Indicator (VASI)

      - Purpose - Provide visual decent guidance information during approach

      - Explanation - The lights are visible from 3-5 miles during day and up to 20 or more at night

        - Safe obstruction clearance within +/- 10o of the centerline and 4 NM from the threshold

      - Configurations

        - 2,4,6,12, or 16 light units arranged in bars

          - Arranged as near, middle, and far bars (Mid provide another glide path for high cockpits)

          - VASIs of 2,4, or 6 light units are located on one side of the runway (usually the left)

          - VASIs consisting of 12 or 16 light units are located on both sides of the runway

        - Most installations consist of 2 bars and may consist of 2,4, or 12 light units

      - Two Bar VASIs

        - Provide one visual glide path, normally set at 3o

      - Three Bar VASIs

        - Provide two visual glide paths

          - The lower glide path is provided by the near and middle bars and is normally set to 3o

            - Some locations may have up to 4.5o glide paths for proper obstacle clearance

          - The upper glide path is provided by the middle and far bars and is normally set ¼o higher

      - How it Works

        - Each unit projects light with an upper white segment and a lower red segment

        - The light units are arranged so that the pilot will see the combinations of lights below:

        - 2-bar VASI

  - 3-bar VASI

  - For other VASI configurations

  - Precision Approach Path Indicator (PAPI)

    - General

      - Lights arranged to provide visual decent guidance information during the approach to a runway

      - Uses light units similar to the VASI but in a single row of either 2 or 4 light units

    - Configuration

      - Tri-Color Systems

        - Normally a single unit projecting a 3-color visual approach path into the final approach area

        - Glide Path Indications

          - Below - Red

          - Above- Amber

          - On - Green

        - Useful Range

          - Day – ½ to 1 mile

          - Night – Up to 5 miles (depending on the visibility)

      - Pulsating Systems

        - Normally a single unit projecting a 2-color visual approach path into the final approach area

        - Glide Path Indications

          - Slightly Below - Steady red

          - Below - Pulsating red

          - On - Steady white

          - Slightly Above - Pulsating white

          - Above - Faster pulsating white

            - Pulsating increases as the aircraft gets further above/below the glide slope

          - Useful Range

            - During the Day, 4 miles and at Night, up to 10 miles

  - Runway End Identifier Lights (REIL)

    - General - Installed to provide rapid/positive identification of the approach end of a runway

    - Configuration - A pair of synchronized flashing lights located on each side of the runway threshold

    - Effective for:

      - Identification of a runway surrounded by a preponderance of other lighting

      - Identification of a runway which lacks contrast with the surrounding terrain

      - Identification of a runway during reduced visibility

  - Runway Edge Light Systems (HIRL, MIRL, LIRL)

    - General - Outline the edges of runways during periods of darkness or restricted visibility conditions

      - Classified according to the intensity or brightness they are capable of producing

        - High Intensity (HIRL); Medium Intensity (MIRL); Low Intensity (LIRL) a HIRL and MIRL have variable intensity controls

    - Configuration

      - Runway edge lights – White

        - Instrument runways – Turn yellow the last 2,000,’ or half the runway, whichever is shorter

      - Lights marking the end of the runway – Red/Green

        - Red is emitted toward the runway to indicate the end of the runway to a departing aircraft

        - Green is emitted outward from the runway end to indicate the threshold to landing aircraft

  - In-runway Lighting

    - Runway Centerline Lighting System (RCLS)

      - General - Installed on some precision runways to facilitate landing under adverse conditions

      - Configuration

        - Located along the runway centerline and are spaced at 50’ intervals

        - From the landing threshold, the lights are white until the last 3,000’ of the runway a White lights begin to alternate with red for 2,000’, and the last 1,000’ all lights are red

    - Touchdown Zone Lights (TDZL)

      - General - On some precision runways, indicating the TD zone with adverse visibility conditions

      - Configuration

        - Two rows of transverse light bars disposed symmetrically about the runway centerline

        - Steady burning white lights starting 100’ beyond the landing threshold and extending to 3,000’ beyond the landing threshold or to the midpoint of the runway, whichever is less

    - Taxiway Centerline Lead-Off Lights

      - General

        - Provide visual guidance to persons exiting the runway

        - Color coded to warn: In runway environment/ILS critical area, whichever is more restrictive

      - Configuration

        - Alternate green/yellow lights are installed, beginning with green, from the runway centerline to 1 light position beyond the runway hold position/ILS critical area hold position

    - Taxiway Centerline Lead-on Lights

      - General

        - Provide visual guidance to persons entering the runway

        - Warn: In the runway environment/ILS critical area, whichever is more conservative

      - Configuration

        - Color coded with the same pattern as lead-off lights

        - Bidirectional (i.e., 1 side emits light for the lead-on function the other for the lead-off)

    - Land and Hold Short Lights

      - General

        - Used to indicate the hold short point on certain runways which are approved for LAHSO

          - Where installed, the lights will be on anytime LAHSO is in effect and off when not

      - Configuration - A row of pulsing white lights installed across the runway at the hold short point

  - Control of Lighting Systems

    - Operation of approach light systems and runway lighting is controlled by the control tower/FSS

    - Pilots may request the lights be turned on or off

  - Pilot Control of Airport Lighting

    - Radio control of lighting is available at some airports by keying the aircraft’s microphone

      - Often available w/o specified hours for lighting and with no tower/FSS or when they’re closed

    - All lighting systems which are radio controlled at an airport operate on the same frequency

      - The CTAF is used to activate the lights at most airports, but other frequencies may also be used

        - The frequency is in the AFD/the standard instrument approach procedures publications

        - It is not identified on the sectional charts

  - Airport Beacons

    - Vertical light distribution to make them more effective from 1o to 10o above the horizon

      - They can be seen well above and below this peak spread

    - Light Projection

      - Omnidirectional capacitor discharge device

      - Rotate at a constant speed

    - Flashes

      - 24-30 per min for airports/landmarks/points on federal airways; 30-45 per min for heliports

    - Colors and Combinations of Beacons

      - White and Green – Lighted land airport

      - *Green alone – Lighted land airport

      - White and Yellow – Lighted water airport

      - *Yellow alone – Lighted water airport

      - Green, Yellow, and White – Lighted heliport

          - Green/Yellow alone is used w/a white & green/white & yellow beacon display, respectively

    - Military Beacons

      - Two quick white followed by a green flash

    - Operation during the day

      - In Class B, C, D and E surface areas, operation of the airport beacon during the hours of daylight often indicates that the ground visibility is less than 3 miles and/or the ceiling is less than 1,000’

        - Don’t rely solely on the airport beacon to indicate if weather conditions are IFR or VFR

          - There is no regulatory requirement for daylight operation

  - Taxiway Lights

    - Taxiway Edge Lights

      - Purpose - Outline the edges of taxiways during periods of darkness or restricted visibility

      - Configuration - Emit blue light

    - Taxiway Centerline Lights

      - Purpose - Used to facilitate ground traffic under low visibility conditions

      - Configuration - Steady burning green lights along the centerline

    - Clearance Bar Lights

      - Purpose - Installed to increase the conspicuity of the holding position in low visibility conditions

        - May also be installed to indicate the location of an intersecting taxiway during darkness

      - Configuration - Three in pavement steady burning yellow lights

    - Runway Guard Lights

      - Purpose - Enhance the conspicuity of taxiway/runway intersections

      - Configuration - Either a pair of elevated flashing yellow lights on either side of the taxiway, or a row of in pavement yellow lights across the entire taxiway, at the runway hold marking

    - Stop Bar Lights

      - Purpose - Confirm ATC clearances to enter/cross the active runway in low visibility conditions

      - Configuration - Row of red, unidirectional, steady burning in pavement lights installed across the entire taxiway at the runway hold position, and elevated steady-burning red lights on each side

        - Following ATC clearance, the stop bar is turned off and the lead-on lights are turned on

      - Cautions

        - Never cross a red illuminated stop bar even if they have received ATC clearance

        - If after crossing, the lead-on lights extinguish, hold position and contact ATC for instruction

  - CE - Failure to comply with airport, runway and taxiway lighting

    - Due to reduced visibility, it can be hard to find your way on the ground at night at a new airport

      - Make sure you know the meaning of all the lights before proceeding