# STRAIGHT-AND-LEVEL FLIGHT (ASEL and ASES)

Objective:

To develop the student’s ability to fly the aircraft with reference to the instruments.

Before Flight:

- Attitude indicator as Direct Indication of pitch attitude in level flight

- Must be adjusted according to other direct indications of level flight

- Altimeter as Indirect Indication of pitch attitude in level flight

- Rate of change indicates magnitude of deviation from level attitude

Maneuver Elements:

- Instructor or student adjusts pitch based on AI to observe relationship between AI position changes and actual aircraft pitch changes

- Use half-bar-width, full-bar-width, and one-and-one-half-bar-width adjustments on AI to demonstrate this

- Instructor ensures student has relaxed grip on controls to allow for better feel of subtle control pressure changes

- Instructor ensures student is using trim properly by having student release controls occasionally

- Instructor or student adjusts pitch and observes resultant altimeter rate-of-change

Aircraft Setup:

Flaps up, gear up

Normal cruise:  (C172RG: 23” Hg, 2300 RPM)

Things to Avoid (Common Errors):

- Slow or improper cross-check during straight-and-level flight

ALT needle tip 0, AI scale pointer (bank index) 0, ASI needle tip on speed,

TC level & centered ball, HI needle tip center of bug, VSI needle tip 0

- Improper power control

Learn/use MP that you know will result in approximate desired airspeed in SLF

If airspeed changes, cross-check VSI, ALT before any power change

- Failure to make smooth, precise corrections as required

Aim for precision: heading +/- 2°, airspeed +/- 2 knots, altitude +/- 20 feet

Beware overcontrolling

Use two step corrections

Just enough control pressure to stop needle movement

A bit more to start smooth correction

- Uncoordinated use of controls

Keep the ball centered

- Improper trim control

Trim to relieve control pressures

Small, frequent adjustments

Diagrams:

References:

Instrument Flying Handbook – Chapter 5

Completion Standards:

The lesson is complete when the student demonstrates knowledge through a written or oral exam and is able to perform straight-and-level flight with reference to the instruments to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.

Instructor Notes:

Fundamentals

Attitude instrument flying means control of the aircraft's spatial position by using instrument's rather than outside visual references

Two basic methods:

- Control and performance

attitude + power = performance

- Primary and Supporting Instruments

Pitch instruments

attitude indicator (AI)

altimeter (ALT)

airspeed indicator (ASI)

vertical speed indicator (VSI)

Bank instruments

attitude indicator (AI)

heading indicator (HI)

magnetic compass (MC)

turn coordinator (TC)

Power instruments

airspeed indicator (ASI)

engine instruments

manifold pressure gauge (MP)

tachometer (RPM)

For any maneuver or flight condition

Primary instruments

One in each group (pitch, bank, power)

Provide most pertinent and essential information

Indication stable with correct flight (shows greatest change with improper aircraft control)

Secondary instruments

Back up and supplement primary instruments

Advantages

Emphasized by FAA (Instrument Flying Handbook)

Requires specific knowledge and interpretation of each individual instrument during training

Helps avoid over-dependence on attitude indicator, facilitating partial panel flight

Fundamental skills

- Cross-check

Continuous, logical observation of instruments for attitude and performance information

Proficient pilot adjusts scan rate and sequence to specific flight situation

Common errors

Fixation

Omission

Emphasis

- Interpretation

Appled knowledge of

instrument construction and operating principles

aircraft performance capabilities (POH)

Learn what performance to expect and combination of instruments to interpret for each flight condition

- Control

Four components

Pitch

Bank

Power

Trim

Common error - Tension - use relaxed touch

Straight-and-Level Flight

- The relationship of pitch, bank and power in straight-and-level flight

Pitch varies with airspeed and load

At constant airspeed, there is only one specific pitch attitude for level flight

Uncorrected bank leads to decreasing pitch with increasing airspeed

- Procedure using full panel and partial panel

Full panel

Maintain constant altitude (+/- 100 ft) and heading (+/- 10°) at a given contstant airpseed (+/- 10 kt)

Constant altitude

- Adjust pitch to maintain assigned ALT with VSI = 0

- Trim to relieve control pressures

- Correct small deviations (< 100 feet) with pitch only

  - ½ bar width on AI

  - VSI < 200 fpm

- Larger deviations may require use of pitch and power

  - Initial 1 bar width on AI

  - VSI = 2 x error in alt

- VSI is primary pitch instrument during altitude corrections

Constant heading

- Adjust aileron and rudder pressures to keep HI constant, wings level on AI and TC with ball centered

- Corrections of heading deviations

Coordinated aileron and rudder

Bank angle < number of degrees off and < standard rate (3°/sec)

Constant airspeed - adjust power (MP, RPM) to maintain airspeed

Prompt, smooth, small corrections

| Straight and Level Flight |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | HI | ASI |
| Secondary | AI, VSI | AI, TC | MP, RPM |

Partial Panel (AI and HI inoperative)

Maintain altitude with ALT, VSI, ASI

Hold heading with TC level, centered ball, and MC

| Straight and Level Flight - Partial Panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC | ASI |
| Secondary | VSI | MC | MP, RPM |

- Coordination of controls and trim

Keep ball centered

Trim to relieve control pressures