# BEFORE TAKEOFF CHECK (ASEL and ASES)

- Objective:

- To familiarize the pilot with the pre-takeoff check. To ensure the pilot understands the purpose of the pre-takeoff check and what to look for during the check.

- *Note: This lesson is a continuance of lesson 31, Taxiing Landplane, and will be accomplished on the first flight following lesson 31.

- Content:

- Pre-flight

  - Review pre-takeoff checklist and discuss desired instrument indications

  - Discuss lesson objectives

- Pre-takeoff Check

  - Instructor will walk student through the checklist and explain the desired instrument indications

- Post-flight

  - Evaluate flight and review material

Things to Avoid (Common Errors)

- Failure to properly use the appropriate checklist

- Improper positioning of the airplane

- Improper acceptance of marginal engine performance

- An improper check of flight controls

- Hazards of failure to review takeoff and emergency procedures

- Failure to avoid runway incursions and to ensure no conflict with traffic prior to taxiing into T/O position

- References:

Airplane Flying Handbook – Chapter 2-11

- Airplane Flight Manual / Pilot’s Operating Handbook

- Completion Standards:

- Student correctly follows pre-takeoff checklist. During debriefing and review of material, student explains adequately the purpose to the steps of the pre-flight check.

- Instructor Notes:

- Required Temperatures and Pressures

  - Usually performed after taxiing to a position near takeoff runway (Run-Up Area)

    - Allows time to warm up to min operating temps ensuring lubrication and internal engine clearances

      - The oil temperature must reach a minimum value (75o)

  - Scan all the engine instruments periodically to ensure they are suitable for the run-up and T/O

- Positioning the Aircraft

  - Position the aircraft on a firm surface that is free of debris (Debris can damage the prop and tail)

  - Position the aircraft somewhat diagonal so the prop will not blast anything behind the aircraft

  - Position the airplane into the wind to prevent overheating and to obtain more accurate indications

  - After positioning, allow the plane to move forward slightly to straighten the nosewheel

    - Considerable stress is placed on the nose wheel during the run-up

  - CE - Improper positioning of the airplane

    - Do not blast anything behind you, position into the wind on a firm surface free of debris

- Division of Attention

  - Attention must be divided inside and outside the airplane

    - If the brake slips, or if the toe brakes are not adequate the airplane could move forward unnoticed

- Checklist

  - The before takeoff checklist provided by the manufacturer should be used to ensure a proper check

  - Follow the checklists, item by item (Read and Do)

  - Be critical of the airplane’s performance, and determine whether it meets the guidelines in the POH

    - Do not accept any unacceptable levels of performance

  - CE - An improper check of flight controls

    - Visually check the controls for proper position/movement

    - Move the controls freely in the full range of positions (a square is a good movement pattern)

  - CE - Failure to properly use the appropriate checklist

    - A manufacturer’s checklists ensures every item is completed and checked in a logical order

- Go/No Go Decision

  - The PIC is responsible for determining whether the airplane is in a condition for safe flight

    - Remember that everything on the checklist is being checked for a reason

  - Stop at any discrepancy and note its effects – Are we still legal? (FARS)/Can the problem be recovered?

  - If there are any doubts, return to the ramp for further investigation

  - CE - Improper acceptance of marginal engine performance

    - Be safe, marginal performance may lead to a hazardous condition

    - Do not feel like you have to complete a flight at a certain time

- Departure Brief

  - Before T/O, review the performance speeds, expected T/O distance and emergency procedures

    - VR (44 knots), VX (58 knots), VY (65 knots) – Announce the speeds

    - “T/O on runway 27, we have 5,500’ of runway, T/O performance shows we will take 1,300’ today”

      - Ensure performance works with the runway and conditions have not changed to prevent that

    - Emergency –Lose engine on the roll, close the throttle and maintain control with the rudder/brakes

      - Lose engine after rotation (within about 50’ AGL), land on the remaining runway

      - Lose engine above 600’ AGL attempt to turn back to the runway otherwise land straight ahead

  - CE - Hazards of failure to review takeoff and emergency procedures

    - This helps prepare for any emergency that may occur and allows everyone to be on the same page

- Assuring the Takeoff Area is Free of Hazards

  - Visually check the area to ensure it is clear of aircraft, vehicles, persons, livestock, wildlife

  - At an uncontrolled airport, announce intentions on CTAF and make a 360o turn in the pattern direction

- Avoiding Incursions

  - Before moving, clear to the Left, Right and Center

  - Just like in taxiing, know where other aircraft are in relation to you

    - Traffic separation is a function of ATC at controlled airports but it is not their responsibility

  - Monitor the appropriate frequency (CTAF/Ground)

  - Repeat all clearances and do not cross hold short lines without a clearance to do so

  - If any doubt exists, wait for the traffic to clear

  - Clear final approach before taxiing into the T/O position

  - Be safe and don’t put yourself in a dangerous position

  - CE - Failure to avoid incursions and to ensure no conflict with traffic prior to taxiing into T/O position

    - It is the PIC’s responsibility to avoid traffic/incursions

      - Do not taxi without clearance and watch for hazards – Know what is happening around you