# CHANGE OF AIRSPEED IN STRAIGHT-AND-LEVEL AND TURNING FLIGHT

Objective:

To enhance the student skills and comfort with performing airspeed changes during straight-and-level and turning flight under instrument meteorological conditions.

Before Flight:

- Attitude indicator as Direct Indication of pitch attitude in level flight

- Must be adjusted according to other direct indications of level flight

- Altimeter as Indirect Indication of pitch attitude in level flight

- Rate of change indicates magnitude of deviation from level attitude

Maneuver Elements:

- Procedure using full panel and partial panel for maintaining altitude and changing airspeed in straight-and-level and turning flight

Full panel

Airspeed change in SLF

| Airspeed change in turn - Full panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | HI | MP(RPM) -> ASI |
| Secondary | AI, VSI | AI,TC | ASI -> MP(RPM) |

Airspeed change in turn

| Airspeed change in turn - Full panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC | MP(RPM) -> ASI |
| Secondary | AI, VSI | AI | ASI -> MP(RPM) |

Partial panel (AI and HI inoperative)

Airspeed change in SLF - partial panel

| Airspeed change in SLF - Partial panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC -> MC | MP(RPM) -> ASI |
| Secondary | VSI | TC | ASI -> MP(RPM) |

Airspeed change in turn - partial panel

| Airspeed change in turn - Partial panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC | MP(RPM) -> ASI |
| Secondary | VSI | MC | ASI -> MP(RPM) |

- Coordination of controls and trim technic

Aircraft Setup:

Flaps up, gear up

Normal climb:  (C172RG: 25” Hg, 2500 RPM)

Things to Avoid (Common Errors):

  - Slow or improper cross-check during straight-and-level flight and turns

  - Improper power control

  - Failure to make smooth, precise corrections as required

  - Uncoordinated use of controls

  - Improper trim technic

References:

Instrument Flying Handbook Chapter 5

Completion Standards:

The lesson is complete when the student demonstrates knowledge through a written or oral exam and is able to perform airspeed changes in straight-and-level and turning flight with reference to the instruments to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.

Instructor Notes:

- Change of airspeed in straight-and-level flight and turns

  - Procedure using full panel and partial panel for maintaining altitude and changing airspeed in straight-and-level and turning flight

Full panel

Airspeed change in SLF

        - Use known power settings (MP, RPM) for desired airspeeds and gear/flap configurations

        - As power is changed MP(RPM) is primary power instrument

        - Change to known power setting

        - If more rapid speed change needed, use underpower or overpower: change MP(RPM) more than required and readjust as desired airspeed is reached

        - Maintain SLF as airspeed changes

          - Airspeed decrease: pitch increase, trim

          - Airspeed increase: pitch decrease, trim

        - Maintain centered ball

          - Airspeed decrease: less right (more left) rudder, trim

          - Airspeed increase: more right (less left) rudder, trim

        - As desired airspeed reached, ASI primary for power

| Airspeed change in SLF - Full panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | HI | MP(RPM) -> ASI |
| Secondary | AI, VSI | AI, TC | ASI -> MP(RPM) |

Airspeed change in turn

        - Requires rapid cross check and interpretation plus smooth control

        - Pitch and power: same as in airspeed change in SLF

        - Bank angle must be varied in direct proportion to airspeed change to maintain constant rate of turn

          - Airspeed decrease: decrease bank, increase pitch, trim

          - Airspeed increase: increase bank, decrease pitch, trim

        - Easier to change airspeed after turn is established

| Airspeed change in turn - Full panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC | MP(RPM) -> ASI |
| Secondary | AI, VSI | AI | ASI -> MP(RPM) |

Partial panel (AI and HI inoperative)

Airspeed change in SLF - partial panel

        - TC and MC only bank instruments

        - MC acceleration errors, especially east or west headings (ANDS)

        - Rely on TC until MC stabilizes on desired heading

| Airspeed change in SLF - Partial panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC -> MC | MP(RPM) -> ASI |
| Secondary | VSI | TC | ASI -> MP(RPM) |

Airspeed change in turn - partial panel

        - TC and MC only bank instruments

        - Recommend: two steps easier:

          - Turn to desired heading

          - Change to desired airspeed

| Airspeed change in turn - Partial panel |  |  |  |
| --- | --- | --- | --- |
|  | Pitch | Bank | Power |
| Primary | ALT | TC | MP(RPM) -> ASI |
| Secondary | VSI | MC | ASI -> MP(RPM) |

  - Coordination of controls and trim technic

    - Centered ball

    - Trim to relieve control pressures

- Common Errors

  - Slow or improper cross-check during straight-and-level flight and turns

- Increase rate of cross check during speed changes

- Anticipate need to adjust pitch and bank and trim

  - Improper power control

- MP(RPM) primary for power during airspeed change until desired airspeed approached, then ASI primary for power

- Use known power settings for specific airspeeds and gear/flap configurations

  - Failure to make smooth, precise corrections as required

- Aim for precision: heading +/- 2°, airspeed +/- 2 knots, altitude +/- 20 feet

- Beware over controlling

- Use two step corrections

      - Just enough control pressure to stop needle movement

      - A bit more to start smooth correction

  - Uncoordinated use of controls

- Centered ball

  - Improper trim technic

- Trim to relieve control pressures

- Small, frequent adjustments