# COCKPIT MANAGEMENT (ASEL and ASES)

- Objectives:

To familiarize the student with cockpit management as it relates to the organization of items in the cockpit, the briefing of passengers, the use of seat harnesses to ensure safety.

- Content:

- Use of seat harnesses

- Passenger briefing

- Placement of objects in the cockpit

- Hazards of unsecured objects in turbulent conditions

- Appropriate use of written checklists

- Use of cell phones in-flight

Things to Avoid (Common Errors)

- Failure to place and secure essential materials and equipment for easy access during flight

- Failure to properly adjust cockpit items, such as safety belts, shoulder harnesses, rudder pedals, and seats

- Failure to provide proper adjustment of equipment and controls

- Failure to provide occupant briefing on emergency procedures and use of safety belts

- Failure to utilize all resources required to operate a flight safely

- References:

- Pilot’s Handbook of Aeronautical Knowledge – Chapter 17

- Use of safety belts, shoulder harnesses, and child restraint systems (14 CFR 91.107)

- Airplane Flight Manual / Pilot’s Operating Handbook

- Completion Standards:

When complete, the student exhibits knowledge of the elements related to cockpit management procedures, organizes and arranges material and equipment in a manner making items readily available, is able to brief passengers on the use of safety belts, shoulder harnesses, and emergency procedures, uses appropriate checklists

Instructor Notes:

- Arranging and Securing Materials and Equipment

  - Arranging

    - Ensure that all necessary equip, documents, checklists, and nav charts appropriate are on board

      - Materials should be neatly arranged and organized making them readily available

    - Any equipment with wires should not interfere with the motion or operation of any controls

    - A disorganized cockpit will complicate any flight, organization will contribute to safe, efficient flying

  - Securing

    - The cockpit/cabin should be checked for articles that might be tossed around in turbulence

      - Loose items should be properly secured (Baggage net in the DA20)

  - All pilots should form the habit of good housekeeping

  - CE - Failure to place and secure essential materials and equipment for easy access during flight

    - Don’t use the top of the instrument panel for storage

- Use and Adjustment of Cockpit Items

  - The pilot must be able to see inside and outside references

    - Use a cushion to provide proper seating if necessary (DA20 seats are not adjustable)

  - Seat Belt/Harnesses

    - When seated, the seat belt/harness should be adjusted to a comfortably snug fit

      - Shoulder harness must be worn at least for taxi, T/O, and LDG

      - The safety belt must be worn all times at the controls

  - Rudder Pedals

    - Adjust the rudder pedals forward or backward

      - Knees should be slightly bent

      - With heels on the floor and balls of the feet on the pedals full movement should be available

      - Using toes, the brakes should be able to be actuated

  - CE - Failure to properly adjust cockpit items, such as safety belts, harnesses, rudder pedals, and seats

    - Very important to safety – complete this on the ground as it can be hazardous in the air

  - CE - Failure to provide proper adjustment of equipment and controls

- Occupant Briefing

  - Safety Belts

    - Each person must be briefed on how to fasten and unfasten the safety belt/harness (91.107)

      - You cannot taxi, T/O, Land without notifying/ensuring each person has fastened their safety belt

  - Emergency Procedures

    - A passenger briefing on the proper use of safety equipment and exit info must also be done

      - Inform passengers what should be done before and after an off-airport landing

      - Ensure all passengers can open all exit doors and unfasten safety belts

    - Departure Plan

      - Runway available, Runway Required, Emergency procedures during T/O

  - CE - Failure to provide occupant briefing on emergency procedures and use of safety belts

    - Passengers must fully understand how to use safety belts and what to do in an emergency

- Resource Utilization

  - To make informed decisions, you must be aware of the resources found inside and outside the cockpit

  - Internal Resources

    - POH is essential for accurate flight planning and resolving equipment malfunctions

    - Checklists verify instruments and systems are checked, set, and operating properly and ensure the proper procedures are performed in the case of an emergency

    - Equipment - A thorough understanding of the equipment is necessary to fully utilize all resources

      - Program any info ahead of time (radio frequencies, fixes,

      - If you do not understand/rely on equipment so much that you are complacent it can be unsafe

    - Passengers can look for traffic, and provide helpful information (strange sound/scent, checklist help)

    - Charts, other pilots, and your own ingenuity, knowledge and skill are also excellent resources

  - External Resources

    - ATC, maintenance technicians, and flight service personnel

    - ATC/FS specialists can decrease work with traffic advisories, vectors and emergency assistance

      - May be able to access maintenance personnel, or other assistance in an emergency

    - FSS can provide weather, airport conditions

    - Other airplanes (PIREPS)

    - ASOS/AWOS can also provide weather conditions in flight

  - CE - Failure to utilize all resources required to operate a flight safely

    - Utilize all available resources during preflight planning and in the flight

    - EXS: Contact an A&P to determine the effects of an inoperative piece of equipment, etc

      - Ensure the aircraft can be flown without the equipment as well