# FEDERAL AVIATION REGULATIONS AND PUBLICATIONS

Objective:

To familiarize the student with the FAR’s and their application and applicability to the student as a pilot and to familiarize the student with the various aviation-related publications that he will use to gather information for flight.

Content:

- FARS 1, 61, 91 and NTSB Part 830

  - Part 1 – Definitions and Abbreviations

  - Part 61 – Certification: Pilots, Flight Instructors, and Ground Instructors

  - Part 91 – General Operating and Flight Rules

  - NTSB Part 830

- Aviation Related Publications

  - Airport/Facility Directory (A/FD)

  - Aeronautical Information Manual (AIM)

  - FAA Advisory Circular (AC)

  - CFR 14 (FARS)

00 – General

10 – Procedural Rules

20 – Aircraft

60 – Airmen

70 – Airspace

90 – Air Traffic and General Operating Rules

140 – Schools and Other Certificated Agencies

150 – Airport Noise Compatibility Planning

170 – Navigation Facilities

180 – Administrative Regulations

190 – Withholding Security Information

210 – Flight Info (Aeronautical charts, doesn’t relate to the FARs)

120 – Air Carriers, Air Travel Clubs, and Operators for Comp/Hire: Certification and Ops

  - Notice to Airmen (NOTAM)

    - NOTAM (D ), or Distant

    - NOTAM (L), or Local

    - Flight Data Center (FDC) NOTAM

  - Notices to Airmen Publication (NTAP)

  - Practical Test Standards

  - Pilot’s Operating Handbook (POH)

References:

Title 14 of the Code of Federal Aviation Regulations

FAR/AIM

Completion Standards:

This lesson continues throughout the student’s training and is, therefore, never complete.

The lesson is complete when the instructor determines that the student has adequate knowledge of the appropriate publications used for flight, and understands how to research the Federal Aviation Regulations pertinent to the activity being performed.

Instructor Notes:

- FARS 1, 61, 91 and NTSB Part 830

  - Part 1 – Definitions and Abbreviations

    - Provides general definitions as well as abbreviations and symbols

  - Part 61 – Certification: Pilots, Flight Instructors, and Ground Instructors

    - The requirements for issuing pilot, flight instructor, and ground instructor certificates and ratings

    - The conditions under which those certificates and ratings are necessary

    - Privileges and limitations of those certificates and ratings

    - Subparts

      - Aircraft Ratings and Pilot Authorization

      - Student Pilots

      - Recreational Pilots

      - Private Pilots

      - Commercial Pilots

      - Airline Transport Pilots

      - Flight Instructors

      - Ground Instructors

      - Sport Pilots

  - Part 91 – General Operating and Flight Rules

    - Describes rules governing the operation of aircraft

    - Subparts

      - General

      - Flight Rules (General, Visual, Instrument)

      - Equipment, Instrument, and Certificate Requirements

      - Special Flight Operations

      - Maintenance, Preventive Maintenance, and Alterations

      - Large and Turbine Powered Multiengine Airplanes and Fractional Ownership Program Aircraft

      - Additional Equipment and Operating Requirements for Large and Transport Category Aircraft

      - Foreign Aircraft Operations and operations of US Registry Civil Aircraft Outside of the US; and rules Governing Persons on Board Such Aircraft

      - Operating Noise Limits

      - Waivers

      - Fractional Ownership Operations

  - NTSB Part 830

    - NTSB is a function within the US DOT (like the FAA) that has rule-making authority similar to the FAA

    - It contains numerous forms and paragraphs, including

      - Applicability

      - Definitions

      - Immediate Notification

      - Information to be given in Notification

      - Preservation of Aircraft Wreckage, Mail, Cargo, and Records

      - Reports/Statements to be Filed

- Aviation Related Publications

  - Airport/Facility Directory (A/FD)

    - A Civil Flight Information Publication published/distributed every 8 weeks by NACO

      - NACO – National Aeronautical Charting Office, a division of the FAA

      - It is a directory of all airports, seaplane bases, and heliports open to the public; communication data; nav facilities; and certain special notices and procedures

    - A directory is published for each of seven geographical districts (NW, SW, NC, SC, EC, NE, SE)

    - The A/FD is a vital publication for cross country planning

      - All pertinent info regarding airports, FSS contact info, etc is contained in the AFD

  - Aeronautical Information Manual (AIM)

    - Provides pilots with a vast amount of basic flight info and ATC procedures in the US

    - The AIM has a comprehensive and useful index to help find topics of interest

  - FAA Advisory Circular (AC)

    - Issued as a systematic means for issuing nonregulatory material of interest to the aviation public

      - Unless incorporated into a regulation by reference, the contents of an AC are not binding

      - Provide guidance/info on a subject or show a FAA acceptable method for complying with a FAR

    - Issued in a # system of general subject matter areas corresponding with the subjects in the FARS

00 – General

10 – Procedural Rules

20 – Aircraft

60 – Airmen

70 – Airspace

90 – Air Traffic and General Operating Rules

140 – Schools and Other Certificated Agencies

150 – Airport Noise Compatibility Planning

170 – Navigation Facilities

180 – Administrative Regulations

190 – Withholding Security Information

210 – Flight Info (Aeronautical charts, doesn’t relate to the FARs)

120 – Air Carriers, Air Travel Clubs, and Operators for Comp/Hire: Certification and Ops

  - Notice to Airmen (NOTAM)

    - Disseminates time critical aeronautical info, which is either temporary in nature or is not sufficiently known in advance to permit publication on aeronautical charts or in other publications

      - Information that could affect the decision to make a flight

    - NOTAM (D ), or Distant

      - Usually regarding airport or navigation facilities

      - Includes such info as airport or primary runway closures, changes in the status of nav aids, ILSs, radar service availability, and other info essential to planned en route, terminal, or landing ops

      - Info is disseminated for all nav facilities part of the National Airspace System, public use airports, seaplane bases, and heliports listed in the A/FD

      - Attached to hourly weather reports and are available at FSS

    - NOTAM (L), or Local

      - Items of local nature

      - Includes such info as taxiway closures, personnel/equipment near /crossing runways, rotating beacon outages, airport lighting that does not affect instrument approach procedure criteria

      - Distributed only locally and is not attached to hourly weather reports

      - Maintained at the FSS nearest the affected airport

        - Information for other FSS areas must be specifically requested from the responsible FSS

      - DUATS vendors are not required to provide NOTAM (L) info

    - Flight Data Center (FDC) NOTAM

      - Regulatory in nature and include amendments to published IAPs and other aeronautical charts

        - Also advertises temporary flight restrictions caused by such things as natural disasters or large-scale public events that may generate a congestion of air traffic over a site

      - FSSs are responsible for maintaining a file of current, unpublished FDC NOTAMS concerning conditions within 400 NM of their facilities

        - FDC info concerning conditions beyond 400 NM from the FSS is given only when requested

      - DUATS vendors will provide only upon site-specific requests using a location identifier

  - Notices to Airmen Publication (NTAP)

    - Issued every 28 days and is an integral part of the NOTAM system

      - Once published here, a NOTAM isn’t provided during pilot weather briefings unless requested

    - Two Sections

      - 1st Section – NOTAMs (D) that are expected to remain in effect for an extended period and FDC NOTAMS that are current at the time of publication

        - Some NOTAMs (L) and other unique info are in this section when contributing to safety

      - 2nd Section – Special notices that are too long or concern a wide/unspecified geographic area

    - The number of the last FDC NOTAM in the NTAP is shown on the front to help update the listing with any that may have been issued between the cut-off date and the date the pub is received

      - All info is carried until it expires, is cancelled, or is published in other publications

    - All new NOTAMS entered, excluding FDC NOTAMs, will be published only if the info is expected to remain in effect for at least 7 days after the effective date of the NTAP

  - Practical Test Standards

    - FARS specify areas in which knowledge/skill must be shown before the issuance of a certificate

      - FARS provide flexibility to permit the FAA to publish PTSs with the tasks skill must be shown

    - Current PTSs

      - Sport, Private, Instrument, Commercial, Flight Instructor (Instrument), ATP and Type Rating

  - Pilot’s Operating Handbook (POH)

    - The POH describes the airplane and its operation

    - Sections include:

      - General – Description of the Airplane

      - Limitations – Description of operating limits

      - Emergency Procedures – What to do in each emergency situation

      - Normal Procedures - Checklists

      - Performance – Graphs and tables relating to airplane capabilities

      - Weight and Balance – Equipment list and airplane’s empty weight

      - Airplane and Systems Description – Description of the airplane’s systems

      - Handling, Preventative and Corrective Maintenance – Explanation of what and when and how

      - Supplements – Description of optional equipment or upgrades