# GO-AROUND/REJECTED LANDING (ASEL and ASES)

Objective:

To teach the student the proper procedures and technique for executing an alternative to a landing that cannot be accomplished for any reason.

Before Flight:

- Review relevant aircraft aerodynamics and performance characteristics

- Go-around is not an emergency-only maneuver

- Power – number one concern

- Altitude

- Configuration – flaps first, then landing gear

- Full flaps usually create more drag than landing gear

- In case aircraft accidentally touches down during go-around initiation, gear down and locked will be advantageous

- Dangers of immediate full flap retraction

- Anticipation of aerodynamic forces

Maneuver Elements:

- Clear the area

- Choose forced landing area (runway)

- Configure aircraft: landing checklist, begin the final approach to land

- Select outside references (field)

- Commit to go-around (don’t change mind!)

- Smoothly apply throttle to full while simultaneously anticipating left-turning tendencies with right rudder pressure

- Adjust pitch for VX

- Carburetor heat off (cold)

- Establish positive rate of climb

- Smoothly retract first notch of flaps; stabilize attitude

- Rough trim as needed

- Maneuver airplane to the traffic-pattern side of the runway and fly parallel to it

- Smoothly retract the remaining flaps incrementally (allow aircraft to stabilize)

- Adjust pitch for VY

- Open cowl flaps

- Retract landing gear upon passing useable runway and achieving a safe altitude and positive climb rate

- Maintain ball centered

- Look for traffic

Aircraft Setup:

Flaps as required, gear down, propeller Full

Final Approach: (C172RG: 15” Hg, 2700 RPM)

Vx: (C172RG: 55 KIAS)

Vy: (C172RG: 77 KIAS)

Things to Avoid (Common Errors)

- Failing to commit to a go-around when situation warrants

- Failing to anticipate and adjust for pitch to maintain safe airspeed

- Failing to anticipate and adjust for left turning tendencies

- Retracting landing gear before flaps, since flaps cause more drag and since an inadvertent touchdown (or second touchdown) can occur

Diagrams:

References:

Airplane Flying Handbook - Chapter 8

Aircraft Flight Manual / Pilot’s Operating Handbook

Completion Standards:

The lesson is complete when the instructor determines that the student can perform a proper go-around by making a timely decision to discontinue the landing, applying takeoff power immediately and pitching for Vy, performing other required elements of the go-around.