# HIGH ALTITUDE OPERATIONS

Objective:

To familiarize the student with the knowledge, procedures, and risks involved in high altitude operations.

Content:

- Regulatory Requirements

  - 12,500’ MSL up to/including 14,000’ – Flight crew uses oxygen for over 30 min

  - Above 14,000’ – Flight crew uses oxygen the entire time

  - Above 15,000’ – Each occupant is provided oxygen

  - Above FL 250 - 10 min supply of supplemental oxygen is available for each person

  - Above FL 350 – Quick donning oxygen masks above FL 410, mask always on when one person is at the controls

- Physiological Hazards

  - The human body functions normally from sea level to 12,000’ MSL

    - Brain oxygen saturation is at a level for normal function (Optimal functioning is 96% saturation)

      - At 12,000’, oxygen saturation is approx 87%, which gets close to a performance affecting level

  - Hypoxia (Reduced Oxygen, or not enough oxygen)

    - Hypoxic Hypoxia (Insufficient oxygen available to the lungs)

    - Hypemic Hypoxia (The blood cannot transport enough oxygen to the tissues/cells)

    - Stagnant Hypoxia (Oxygen rich blood isn’t moving to the tissues)

    - Histotoxic Hypoxia (“Histo” refers to tissues or cells, and “Toxic” means poison)

    - Symptoms:  Cyanosis; Headache; Decreased reaction time/Impaired judgment; Euphoria; Visual Impairment; Drowsiness/Lightheaded or dizzy sensation; Tingling in fingers or toes and Numbness

    - Useful Consciousness

    - Treatment

      - Flying at lower altitudes (Emergency Decent) and use supplemental oxygen

  - Prolonged O use can be harmful to health (100% aviation O can create toxic symptoms if used too long)

    - Symptoms: bronchial cough, fever, vomiting, nervousness, irregular heartbeat, lowered energy

  - Nitrogen

    - Trapped Gas

    - Evolved Gas

  - Vision tends to deteriorate with Altitude

- Pressurization in Airplanes

  - Cabin pressurization is the compression of air to maintain a cabin altitude lower than the flight altitude

  - How it Works

    - Turbine aircraft – bleed air from the engine compressor

    - Piston aircraft – turbocharger’s compressor or engine driven pneumatic pump

    - The cabin pressure control system – provides pressure regulation, pressure relief and vacuum relief and the means for selecting the desired cabin altitude

    - Instruments:  Cabin differential pressure gauge, cabin altimeter, differential pressure gauge, cabin rate of climb/descent

- Types of Oxygen Systems

  - Continuous Flow

  - Diluter Demand – Supply oxygen only when the user inhales through the mask

  - Pressure Demand – oxygen is supplied to the mask under pressure at cabin altitudes above 34,000’

- Aviator’s Breathing Oxygen

  - Aviators oxygen

  - Medical oxygen

  - Industrial oxygen

- Care and Storage of High-Pressure Oxygen Bottles

  - Storage and care

  - Dangers

  - Equipment inspections and servicing

- Rapid Decompression Problems and their Solutions

  - Explosive Decompression – (<0.5 sec)

  - Rapid Decompression

  - The primary danger of decompression is hypoxia

  - Recovery from all types of decompression involves donning oxygen masks and an emergency descent

References:

Aviation Weather (AC 00-6A) - Chapter 13

Pilot’s Handbook of Aeronautical Knowledge - Chapter 5-24 to 5-29

Completion Standards:

The lesson is complete when the instructor determines that the student understands the knowledge presented in this lesson completely.

Instructor Notes:

- Regulatory Requirements

  - No person may operate a civil aircraft of US registry at cabin pressure altitudes above:

    - 12,500’ MSL up to/including 14,000’ unless the required min flight crew is provided with & uses supplemental oxygen for the part of the flight at those alts over 30 min

    - 14,000’ unless the required min flight crew is provided with and uses supplemental oxygen during the entire flight time at those altitudes

    - 15,000’ unless each occupant of the aircraft is provided with supplemental oxygen

  - No person may operate a civil aircraft of US registry with a pressurized cabin at flight altitudes above:

    - FL 250 unless at least a 10 min supply of supplemental oxygen is available for each occupant of the aircraft for use in the event that a descent is necessitated by a loss of cabin pressure

      - This is in addition to oxygen required above

    - FL 350, unless one pilot at the controls of the airplane is wearing and using an oxygen mask that is secured and sealed

      - The mask must supply oxygen at all times or automatically supply oxygen whenever the cabin pressure altitude of the airplane exceeds 14,000’ MSL

      - Exception: One pilot need not wear and use an oxygen mask while at or below FL 410 if there are two pilots at the controls and each pilot has a quick donning type of oxygen mask that can be placed on the face with one hand from the ready position within 5 sec, supplying oxygen and properly secured and sealed

      - If one pilot leaves the controls the remaining pilot shall put on and use and oxygen mask until the other pilot has returned

- Physiological Hazards

  - The human body functions normally from sea level to 12,000’ MSL

    - Brain oxygen saturation is at a level for normal function (Optimal functioning is 96% saturation)

      - At 12,000’, oxygen saturation is approx 87%, which gets close to a performance affecting level

  - Hypoxia (Reduced Oxygen, or not enough oxygen)

    - The concern is getting enough oxygen to the brain, since it is particularly vulnerable to deprivation

    - Hypoxic Hypoxia (Insufficient oxygen available to the lungs)

    - Hypemic Hypoxia (The blood cannot transport enough oxygen to the tissues/cells)

    - Stagnant Hypoxia (Oxygen rich blood isn’t moving to the tissues)

    - Histotoxic Hypoxia (“Histo” refers to tissues or cells, and “Toxic” means poison)

    - Symptoms of Hypoxia

      - Cyanosis; Headache; Decreased reaction time/Impaired judgment; Euphoria; Visual Impairment; Drowsiness/Lightheaded or dizzy sensation; Tingling in fingers or toes and Numbness

        - Even with all of these symptoms, hypoxia can cause a pilot to have a false sense of security

    - Useful Consciousness

      - The max time to make rational, life saving decisions and carry them out at a given altitude

        - Above 10,000’ the time begins decreasing rapidly

    - Treatment

      - Flying at lower altitudes (Emergency Decent) and use supplemental oxygen

  - Prolonged O use can be harmful to health (100% aviation O can create toxic symptoms if used too long)

    - The sudden supply of pure oxygen following decompression can often aggravate hypoxia

      - Therefore, oxygen should be taken gradually to build up in small doses

    - Symptoms: bronchial cough, fever, vomiting, nervousness, irregular heartbeat, lowered energy

  - Nitrogen

    - When nitrogen is inhaled, most is exhaled with CO2, but some is absorbed into the body

      - Normally Nitrogen in the body isn’t a problem, because it’s in a liquid state

        - But, if the ambient pressure lowers drastically, it could return to a gas in the form of bubbles

    - Evolving and expanding gases in the body are known as decompression sickness

      - Trapped Gas: expanding/contracting gas in certain cavities during altitude changes can result in abdominal pain, toothache, or pain in ears and sinuses if the pressure change can’t be equalized

      - Evolved Gas: When the pressure drops sufficiently, nitrogen forms bubbles which can have adverse effects on some body tissues

        - Scuba diving compounds this problem

  - Vision tends to deteriorate with Altitude

    - The eyes require oxygen

    - Glare and deteriorated vision are enhanced at night when the body is more susceptible to hypoxia

    - Empty visual field by cloudless, blue skies during the day can cause inaccuracies when judging traffic

- Pressurization in Airplanes

  - Cabin pressurization is the compression of air to maintain a cabin altitude lower than the flight altitude

    - This removes the need for full-time use of supplemental oxygen

    - A cabin pressure altitude of approx 8,000’ is maintained and prevents rapid changes of cabin altitude that may be uncomfortable or cause injury to passengers/crew (prevents against hypoxia)

  - How it Works

    - The cabin, flight and baggage compartments are incorporated into a sealed unit capable of containing air under a higher pressure than the outside atmospheric pressure (Differential Pressure)

      - Differential Pressure - the difference between cabin pressure and atmospheric pressure normally expressed in psi (the higher the plane goes, the higher the differential)

      - Max differential pressure varies by make/model of plane – the higher it is, the higher you can go

    - Turbine powered aircraft – bleed air is from the engine compressor section is used to pressurize

    - In most light planes, the turbocharger’s compressor or engine driven pneumatic pump pressurizes

      - Compression heats the air, so it’s routed through a heat exchange unit before entering the cabin

    - The cabin pressure control system provides pressure regulation, pressure relief and vacuum relief and the means for selecting the desired cabin altitude

      - A cabin pressure regulator, an outflow valve, and a safety valve are used to accomplish this

        - CPR –controls cabin pressure – if we reach the max difference, an increase in altitude outside will result in an increase inside

        - The flow of compressed air is regulated by an outflow valve which keeps pressure constant by releasing excess pressure into the atmosphere

        - Safety Valve is a combo of a pressure relief, vacuum relief, and dump valve

          - Pressure Relief prevents the cabin pressure from exceeding a predetermined differential pressure above ambient pressure

          - Vacuum Relief prevents ambient pressure from exceeding cabin pressure by allowing external air to enter when ambient pressure exceeds cabin pressure

        - Dump Valve dumps cabin air to the atmosphere (switch in the cockpit)

    - Instruments

      - Cabin differential pressure gauge indicates the difference between inside and outside pressure

      - Cabin Altimeter shows the altitude inside the airplane

        - Differential pressure gauge and cabin altimeter can be combined into one instrument

      - Cabin Rate of Climb/Descent

- Types of Oxygen Systems

  - Continuous Flow

    - Most common in GA planes

    - Usually for passengers and has a reservoir bag which collects oxygen from the system when exhaling

    - Ambient air is added to the oxygen during inhalation after the reservoir oxygen supply is depleted

    - Exhaled air is released into the cabin

  - Diluter Demand – Supply oxygen only when the user inhales through the mask

    - Depending on the altitude, the regulator can provide 100% oxygen or mix cabin air and the oxygen

    - The mask provides a tight seal and can be used safely up to 40,000’

  - Pressure Demand – oxygen is supplied to the mask under pressure at cabin altitudes above 34,000’

    - Provide a positive pressure application of oxygen that allow the lungs to be pressurized with oxygen

    - Safe at altitudes above 40,000’

    - Some systems include the regulator on the mask to eliminate purging a long hose of air

- Aviator’s Breathing Oxygen

  - Aviators oxygen is specified at 99.5% pure oxygen and not more than .005mg of water per liter

  - Medical oxygen has too much water, which can collect in various parts of the system and freeze

    - Freezing may reduce/stop the flow of oxygen

  - Industrial oxygen is not intended for breathing and may have impurities in it (metal shavings, etc)

- Care and Storage of High-Pressure Oxygen Bottles

  - If the airplane does not have a fixed installation, portable oxygen equipment must be accessible in flight

  - Oxygen is usually stored at 1,800 – 2,200 psi

    - When the ambient temp surrounding the cylinder decreases, pressure within will decrease

      - If a drop in indicated pressure is noted, there is no reason to suspect depletion of the supply

    - High pressure containers should be marked with the psi tolerance before filling to the pressure

  - Be aware of the danger of fire when using oxygen

    - Materials that are nearly fire proof in ordinary air may be susceptible to burning in pure oxygen

      - Oils and greases may catch fire if exposed to pure oxygen and cannot be in oxygen systems

    - Smoking during any kind of oxygen equipment use is prohibited

    - Before each flight, thoroughly inspect and test all oxygen equipment

  - Examine the equipment - available supply, operational check, and assure it is readily available

  - To assure safety, periodic inspections and servicing should be done

- Rapid Decompression Problems and their Solutions

  - Decompression is the inability of the pressurization system to maintain its designed pressure differential

    - This can be caused by a malfunction in the pressurization system or structural damage to the plane

      - If the turbo charger fails, not only will the airplane descend, but pressurization will be lost

  - Explosive Decompression – A change in cabin pressure faster than the lungs can decompress (<0.5 sec)

  - Rapid Decompression – A change in cabin pressure where the lungs can decompress faster than the cabin (therefore there is no likelihood of lung damage)

    - During explosive decompression, there may be noise and one may feel dazed for a second

    - During most decompressions, the cabin will fill with fog, dust, flying debris

      - Fog is the result of the rapid change in temp and change of relative humidity

    - Air will rush from the mouth and nose due to the escape from the lungs

    - Differential air pressure on either side of the eardrum should clear automatically.

    - Exposure to wind blast and extremely cold temperatures may occur

  - The primary danger of decompression is hypoxia

    - If proper use of oxygen equipment is not accomplished quickly unconsciousness may occur quickly

      - Effective performance time is reduced by one-third to one-fourth its normal time

  - Recovery from all types of decompression involves donning oxygen masks and an emergency descent

    - Top priority is reaching a safe altitude

      - Be aware, cold shock in piston engines can result from rapid high-alt descent, cracking cylinders

      - The time to make a recovery before loss of useful consciousness is much less with explosive