# LANDING FROM A STRAIGHT-IN APPROACH

Objective:

To teach the student the elements, proper procedures and technique for landing an airplane from a straight-in approach.

Content:

- Elements related to landing from a straight-in approach

  - Effect of specific environmental, operational and meteorological factors

    - Environmental factors

    - Operational factors

    - Meteorological factors

  - Transition to, and maintenance of, a visual flight condition

  - Adherence to ATC advisories

    - NOTAMs

    - Wind shear

    - Wake turbulence

    - Braking conditions

  - Completion of appropriate checklist items

  - Maintenance of positive aircraft control

- Common Errors

  - Inappropriate division of attention during the transition from instrument to visual conditions

  - Failure to complete required checklist items

  - Failure to properly plan and complete the turn to final approach

  - Improper technic for wind shear, wake turbulence and crosswind

  - Failure to maintain positive aircraft control throughout the complete landing maneuver

References:

14 CFR Part 91, AIM, NACO TPPs

Aviation Instructor's Handbook, FAA-H-8083-9

Instrument Flying Handbook, FAA-H-8083-15, 1999

Instrument Rating ACS

Completion Standards:

The lesson is complete when the student can accurately perform the steps involved in landing an aircraft from a straight-in approach using the materials covered in this lesson to the satisfaction of the instructor.

Instructor Notes:

- Elements related to landing from a straight-in approach

  - Effect of specific environmental, operational and meteorological factors

    - Environmental factors

      - Runway dimensions

After reaching MAP determined by timing, may safely land on long runway beyond threshold, but circle or fly missed approach if shorter runway

      - Runway condition

    - Operational factors

      - Increased minimums with inoperative approach components

      - Aircraft capabilities - need distance/time to slow +/- add flaps before normal landing

    - Meteorological factors

      - Visibility

        - Only pilot can determine if flight visiblity meets the landing requirement noted on the IAP chart

        - Flight visibility below minimums => go-around, fly missed approach

      - Wind condition

Normal crosswind landing technic

  - Transition to, and maintenance of, a visual flight condition

    - Approaching DA/DH, MAP or VDP, carefully divide attention between instrument and visual references on transition to visual for landing

    - Expect need to go right back to instruments for missed approach if runway visual reference lost

    - On ILS, stay on glide slope and localizer to over runway

    - On pitch changes, flap extensions, avoid inadvertent climb back into IMC (necessitating missed approach)

  - Adherence to ATC advisories, such as NOTAMs, wind shear, wake turbulence, runway surface and braking conditions

    - NOTAMs for inoperative approach components may require higher minimums

    - Wind shear

      - Aviodance best policy

      - Avoid approach near thunderstorm

      - Possible wind shear (pirep, ATC), consider:

        - More power, faster airspeed (+½ gust)

        - Stay higher longer

        - Mentally prepare for go-around, missed approach

    - Wake turbulence

      - Stay 4 miles behind large, 5 miles behind heavy airplanes

      - May "request additional separation" from ATC

    - Braking conditions

      - May request runway braking report from ATC

      - If "nil", don't land, request proceed to alternate airport

      - May land if braking action fair or poor if

        - Relatively longer runway

        - Touchdown on runway close to threshold

        - Minimum practicable airspeed

        - Controlled landing with longitudinal axis over runway centerline

        - Aerodynamic braking

  - Completion of appropriate checklist items

    - Complete before landing checklist by FAF

    - Re-check on final approach course

    - Complete after landing checklist clear of runway

  - Maintenance of positive aircraft control

First fly your airplane

- Common Errors

  - Inappropriate division of attention during the transition from instrument to visual conditions

    - Carefully divide attention between instrument and visual references on transition to visual for landing

    - Expect need to go right back to instruments for missed approach if runway visual reference lost

    - Stay on glide slope and localizer to over runway

  - Failure to complete required checklist items

    - Complete before landing checklist at FAF

    - Re-check on final approach course

    - Complete after landing checklist clear of runway

  - Failure to properly plan and complete the turn to final approach

Especially circling approach

- Remain within protected area and

- Allow enough distance for normal descent and landing

  - Improper technic for wind shear, wake turbulence and crosswind

    - Wind shear

      - Avoid

      - More power, airspeed

    - Wake turbulence - separation

    - Normal crosswind technic

  - Failure to maintain positive aircraft control throughout the complete landing maneuver

First fly your airplane