# LOSS OF GYRO ATTITUDE AND HEADING INDICATORS

Objective:

To familiarize the student with the elements related to and common errors of loss of gyro attitude and heading indicators.

Content:

- Elements of a Loss of Gyro

  - Recognition

  - ATC Notifcation

  - Timely Transition From Full To Ppartial Panel

- Common Errors of Loss of Gyro

  - Slow to recognize inaccurate or inoperative gyro instruments

  - Failure to notify ATC of situation

  - Failure to adequately transition from full to partial panel condition

- Demonstrates and Explain Loss of Gyro

- Common Errors

References:

14 CFR Part 91, NACO TPPs, AIM

Aviation Instructor's Handbook, FAA-H-8083-9

Instrument Flying Handbook, FAA-H-8083-15, 1999

Instrument Rating ACS

Completion Standards:

The lesson is complete when the instructor determines that the student has adequate understanding of loss of gyro attitude and heading indicators by demonstration and b giving an oral or written exam.

Instructors Notes:

- Exhibits instructional knowledge of the elements related to loss of gyro attitude and heading indicators by describing

  - Recognition of inaccurate or inoperative gyro instruments

    - Monitor suction gauge (4.5-5.4 inHg)

    - Make no control inputs based on one instrument - cross check all available

    - Sudden, complete loss of vacuum gyros

      - Vacuum gauge 0

      - AI and HI fail within minutes

    - Slow deterioration in vacuum pressure

      - AI and HI slow, incorrect

      - Developing conflict between AI, HI and other instruments

    - AI, HI failure

      - AI shows turn, possibly steep, but TC and MC confirm SLF

      - AI shows SLF, but TC and MC confirm turn

      - HI shows slow (or fast) turn, but TC and MC confirm SLF

      - Difficult, confusing navigation with HI headings unsteady

    - Promptly cover failed AI and HI

  - Notifcation of ATC of gyro loss and whether able to continue with flight clearance

    - Notify ATC of loss of gyro instruments

    - Continue flight in VFR conditions (to reasonably close airport where repairs can be made)

    - If IFR, request vectors to VFR conditions

    - If unable VFR conditions, choose and execute best available approach (e.g., ILS) on partial panel with ATC vectors if available

  - Importance of timely transition from full to partial panel condition

    - Failed AI and HI can distract and trick pilot into making improper control inputs leading to disorientation in an unusal attitude

    - Ignore failed AI and HI and cover them ASAP

    - Go to partial panel scan at once

      - Wings level with TC (primary for bank), centered ball

      - Pitch level with ALT (primary for pitch), VSI, ASI

      - Navigation with MC for heading information

    - Timed turns

    - Compass turns

      - GPS "HI" mode may help navigation

- Common Errors related to loss of gyro attitude and heading indicators

  - Slow to recognize inaccurate or inoperative gyro instruments

    - Failed AI and HI can distract and trick pilot into making improper control inputs leading to disorientation in an unusal attitude

    - Cross check and interpret several to all flight instruments before any control input

  - Failure to notify ATC of situation

    - In IMC, situation is at least urgent, and may become emergent

    - ATC can help

  - Failure to adequately transition from full to partial panel condition

Cover failed instruments ASAP