# MISSED APPROACH

Objective:

To teach the student the elements, proper procedures and technique for piloting an airplane on a missed approach.

Content:

- Elements of a missed approach procedure

  - Pertinent information on the selected instrument approach chart

    - Minimums section

    - Missed approach procedure

    - Changes to charted minimums may be required

  - Conditions requiring a missed approach

  - Initiation of the missed approach

  - Required report to ATC

  - Compliance with the published or alternate missed approach procedure

  - Notification of ATC if the aircraft is unable to comply with a clearance, instruction, restriction or climb gradient

  - Performance of recommended checklist items appropriate to the go-around procedure

  - Importance of positive aircraft control

- Common Errors

  - Failure to have essential knowledge of the information on the instrument approach chart

  - Failure to recognize conditions requiring a missed approach

  - Failure to promptly initiate a missed approach

  - Failure to make the required report to ATC

  - Failure to comply with the missed approach procedure

  - Faulty basic instrument flying technic

  - Descent below the MDA prior to initiating a missed approach

References:

14 CFR Part 91, AIM, NACO TPPs

Aviation Instructor's Handbook, FAA-H-8083-9

Instrument Flying Handbook, FAA-H-8083-15, 1999

Instrument Rating ACS

Completion Standards:

The lesson is complete when the student can accurately perform the steps involved in flying a missed approach using the materials covered in this lesson to the satisfaction of the instructor.

Instructor Notes:

- Elements of a missed approach procedure

  - Pertinent information on the selected instrument approach chart

    - Minimums section

      - a) Know aircraft category based on 1.3 x VSo

        - C-TR182 - cat A but

        - Circling at 100 KIAS, use category B circling minimums

      - DA/DH, MDAs

      - Minimum visibility required for landing

      - Notes

    - Missed approach procedure

      - Profile view: text and/or icons

      - Pilot EZ-Brief section

    - Changes to charted minimums may be required

      - Check NOTAMs for IAP amendments

      - Inoperative runway visual aids increase minimum visibility required (bot not MDA)

        - Check Inop Components Table inside front cover of TPPs

        - Check minimum section of JEPP chart

      - Some IAPs require a higher MDA when alternate (not local) altimeter setting is used

      - During circling approach, apply circling minimums section using actual airspeed (Cat B for 100 KIAS)

  - Conditions requiring a missed approach

    - At DA/DH, MAP or any time below DA or MDA until touchdown if

      - Runway environment (14 CFR Sec. 91.175) not clearly seen

      - Visibility less than required minimum, or

      - Aircraft not in position for safe landing with normal maneuvering

    - Part of airport not clearly visible during circling approach above MDA

    - ATC request

    - Recommended if

      - Full scale deflection of glide slope and/or localizer needle on final approach course

      - Disorientation, loss of positional awareness, confusion

  - Initiation of the missed approach, including prompt application of power, establishment of a climb attitude and reduction of drag

    - Follow POH checklist for go-around

    - Recommend constant airspeed climb same as approach airspeed

    - C-TR182

Pitch up to climb attitude

Power up to 30-31 inHg

Flaps up to 20°

Confirm mixture rich and carb heat OFF

Confirm safe climb

Gear up

Flaps up slow at safe altitude

  - Required report to ATC

    - First fly airplane until in stable climb, then

    - Inform ATC (or CTAF and then ATC) of missed approach and intentions

  - Compliance with the published or alternate missed approach procedure

    - Fly missed approach procedure as published or

    - Alternate procedure as instructed by ATC

    - If missed approach begun before DA/DH or MAP, climb, but continue IAP route to MAP and then fly published missed approach procedure

    - Missed approach from circling to land: continue climbing circle in direction or airport until established on missed approach course

  - Notification of ATC if the aircraft is unable to comply with a clearance, instruction, restriction or climb gradient

    - Execute clearance upon request

    - If unable (or unclear) promptly request amendment (or clarification)

  - Performance of recommended checklist items appropriate to the go-around procedure

Go-around checklist

  - Importance of positive aircraft control

    - Workload increase may distract from basic attitude instrument flying tasks

    - Aircraft is close to ground

    - Prompt transition from visual to instrument references must be made

    - On missed approach, fly airplane first

    - Prompt transition from descent to climb must be made

    - Do nothing else until stable safe climb completely under control

- Common Errors

  - Failure to have essential knowledge of the information on the instrument approach chart

    - Plan ahead

    - Know approach(es) to expect (ATC, ATIS)

    - Study, record, memorize key data in advance

How low? How long? How far? Which way?

    - Use PC or simulator to practice flying approaches, including missed approach procedures, to destinations of upcoming flights

  - Failure to recognize conditions requiring a missed approach

DA/DH or MAP and runway environment not clearly in site => go missed approach now

  - Failure to promptly initiate a missed approach

    - Plan on flying the missed approach every time

    - DA/DH or MAP and runway environment not clearly in site => go missed approach now

  - Failure to make the required report to ATC

First fly, confirm controlled climb, then talk to ATC

  - Failure to comply with the missed approach procedure

Fly missed as published

  - Faulty basic instrument flying technic

May be due to high workload in disorganized cockpit

  - Descent below the MDA prior to initiating a missed approach

Maintain MDA +100 feet, -0 feet to MAP