# NAVIGATION AND FLIGHT PLANNING

Objective:

To teach the student the proper procedures and technique for piloting an airplane from one geographic position to another while monitoring his position as the flight progresses.

Content:

- Terms

  - Navigation

  - Atmospheric

  - Altitudes

  - Airspeeds

- Aeronautical Charts

  - Sectionals

  - VFR Terminal Area Charts

  - World Aeronautical Charts

  - www.naco.faa.gov for chart listings and ordering

- Pilotage and Dead Reckoning

  - Pilotage – Navigation by reference to landmarks or checkpoints

  - Dead Reckoning – Navigation solely by computations based on time, airspeed, distance, and direction

- Radio Navigation – Navigation by which a predetermined flight path is followed by using radio waves

- Weather Check

  - Obtaining a preflight weather briefing is the first step to determine if the flight can be conducted safely

  - FAR 91.103 requires familiarity with weather reports and forecasts for the flight

  - Go/No Go

- Flight Planning

  - Plotting a Course

  - Fuel Stops

  - Checkpoints

  - Unforeseen Events

- Fuel Consumption and Flight Information

- Using a Flight Log

- Filing a Flight Plan

- Diversion to an Alternate

- Lost Procedures

  - Avoiding Becoming Lost

  - Once Lost

References:

Pilot’s Handbook of Aeronautical Knowledge – Chapter 14

Completion Standards:

The lesson is complete when the student can accurately perform the steps involved in planning a flight and filing a flight plan using all the materials covered in this lesson and to the satisfaction of the instructor.

Instructor Notes:

- Terms

  - Navigation

    - True Course – The intended/desired direction of flight as measured on a chart clockwise from true N

    - True Heading – The direction the longitudinal axis of the airplane points with respect to true N

      - True heading equals true course plus or minus any wind correction angle

    - Magnetic Course – True course corrected for magnetic variation

    - Magnetic Heading – Magnetic Course corrected for wind (direction and speed)

    - Compass Heading – Aircraft heading read from the compass

      - Derived by applying correction factors for variation, deviation, and wind to true course

    - Deviation – Compass error due to magnetic disturbances from electrical/metal parts in the plane

      - The correction for this is displayed on a compass correction card near the magnetic compass

    - Variation – The angular difference between true north and magnetic north; isogonic lines on charts

  - Atmospheric

    - Standard Pressure – 29.92” Hg

    - Standard Temperature – 15o C

  - Altitudes

    - Indicated – The altitude read directly from the altimeter after it’s set to the current altimeter setting

    - Pressure – The height above the standard pressure level of 29.92 in Hg -

      - Obtained by setting 29.92 in the barometric pressure window and reading the altimeter

    - Density – Pressure altitude corrected for nonstandard temperatures -

      - Directly related to an aircraft’s takeoff, climb, and landing performance

    - True – The true vertical distance of the aircraft above sea level

  - Airspeeds

    - Indicated (IAS) – The speed of an aircraft as shown on the airspeed Indicator

    - Calibrated (CAS) – Indicated airspeed of an aircraft, corrected for installation and instrument errors

    - True (TAS)– The speed at which an aircraft is moving relative to the surrounding air

      - CAS corrected for density altitude

    - Groundspeed – The speed of the aircraft in relation to the ground

    - Equivalent (EAS) – CAS corrected for adiabatic compressible flow for the particular altitude

      - Airport, terrain, and obstacle elevations found on aeronautical charts are true altitudes

    - Absolute – The vertical distance of the aircraft above the surface of the earth, either water or terrain

- Aeronautical Charts

  - The roadmap for a pilot flying VFR

    - Provide info which allows pilots to track their position and provides available info to enhance safety

  - Types of Charts: Sectional Chart; VFR Terminal Area Chart

  - Sectional Charts (Most common used by pilots)

    - Information provided:

      - Airport data, navigational aids, airspace, and topography

    - Scale is 1:500,000 (1” = 6.86 NM)

    - Revised semiannually

  - VFR Terminal Area Charts

    - Helpful when flying in or near Class B airspace

    - They provide a more detailed display of topographical info

    - Scale is 1:250,000 (1” = 3.43 NM)

    - Revised semiannually

  - Features

    - The topographical info portrays surface elevation levels and a large number of visual checkpoints

      - Checkpoints include roads, railroads, power lines, aerial cables, and many different landmarks

    - The aeronautical information includes visual and radio aids to navigation, airports, controlled airspace, restricted areas, obstructions, and related data

  - Proper and Current Aeronautical Charts

    - It is vitally important the publication date on each aeronautical chart is checked

      - Revisions may include changes in radio frequencies, new obstructions, temporary or permanent closing of certain runways and airports, and other temporary or permanent hazards to flight

      - Obsolete charts should be discarded and replaced with new editions

- Pilotage and Dead Reckoning

  - Pilotage – Navigation by reference to landmarks or checkpoints

    - A method of navigation that can be used on any course with adequate checkpoints, but is more commonly used with dead reckoning and VFR radio navigation

      - It becomes difficult in areas lacking prominent landmarks or in low visibility

    - The checkpoints used should be prominent features common to the area of flight

      - Choose checkpoints that can be readily identified by other features such as roads, rivers, railroad tracks, lakes, and power lines

        - Roads shown are usually the most traveled/easily visible from the sky

          - New roads and structures are constantly being built and may not be on the chart

    - If possible, select features that will make useful boundaries on each side of the course

      - Keep from drifting too far off course be referring to and not crossing selected brackets

    - Never place complete reliance on any single checkpoint, choose ample checkpoints

      - If one is missed, look for the next one while maintaining the necessary heading

  - Dead Reckoning – Navigation solely by computations based on time, airspeed, distance, and direction

    - The products derived from these, when adjusted by wind speed and velocity, are heading and GS

      - The predicted heading will guide the airplane along the intended path and the GS will establish the time to arrive at each checkpoint and destination

  - Except for flights over water, dead reckoning is usually used with pilotage

    - Heading and GS is constantly monitored and corrected by pilotage as observed from checkpoints

    - Ideally, Radio navigation should be added so that a pilot uses all three forms of navigation

      - Start with dead reckoning and confirm with pilotage and radio navigation

- Radio Navigation – Navigation by which a predetermined flight path is followed by using radio waves

  - The appropriate nav facility is tuned and ground track is controlled based on the instrument’s indication

  - Ground stations or satellites in space transmitting signals to navigation radio receivers in the airplane

  - There are four radio navigation systems available: VOR, NDB, LORAN-C, GPS

- Weather Check

  - Obtaining a preflight weather briefing is the first step to determine if the flight can be conducted safely

    - It also shows where problems may occur during the flight

  - FAR 91.103 requires familiarity with weather reports and forecasts for the flight

  - Go/No Go

    - Good judgment is necessary in deciding whether or not to take the flight

      - A gutsy, dangerous condition could result very badly

    - Weather factors must be considered in relation to the equipment to be flown

      - Can the plane handle the flight?

      - The following conditions may lead to a No Go Decision

        - T-Storms of any kind, especially embedded

        - Fast-moving fronts or squall lines

        - Moderate or greater turbulence

        - Icing

        - Fog, or other visual obscurations

    - Physical/Mental condition

      - Sick, tired, upset, depressed – These factors can greatly affect the ability to handle any problem

    - Recent Flight Experience

      - Don’t go beyond your abilities or the airplane’s abilities

      - EX: Are you comfortable in MVFR if you haven’t flown in a while

- Flight Planning

  - Plotting a Course

    - First, draw a line from Point A to Point B

      - If the route is direct, the course line will consist of a single straight line

      - If it’s not, it will consist of 2 or more straight line segments

        - For example, a VOR station which is off the direct route but will make navigating easier

  - Fuel Stops

    - Based on personal comfort, and at a minimum, on regulatory requirements

      - FAR 91.151 requires that there be enough fuel onboard the airplane to fly to the point of intended landing and, at normal cruise power, to fly for at least

        - 30 min during the day, or 45 min during the night

    - Plan them accordingly as needed

  - Checkpoints

    - Appropriate checkpoints should be selected along the route and noted in some way

    - Checkpoints should be easy to locate points like large towns, lakes and rivers, or combinations of recognizable points like towns w/an airport, or a network of highways, and railroads going in/out

      - Normally choose towns indicated by splashes of yellow on the chart

      - Do not choose towns signified by a small circle - they may be only a half-dozen homes

        - In isolated areas, although, this can be a prominent checkpoint

  - Unforeseen Events

    - Once the course is drawn, survey the route of flight

      - Look for available alternate airports along the route

      - Look at the terrain (mountains, swamps, water, etc) that would have impact if an emergency landing were necessary

      - Mentally prepare for any type of emergency situation and the appropriate action to be taken

      - Also, ensure the route of flight does not penetrate any restricted (if in use) or prohibited areas

- Fuel Consumption and Flight Information

  - First, find the distance to the point by measuring the length of the course drawn on the sectional

  - Divide the distance by the calculated GS (found with the wind side of the flight computer) to get time

    - D = R x T, so T = D / R or GS

  - Multiply the projected hourly fuel burn by the number of hours to get the total fuel consumption

    - EX: Distance = 220nm, GS = 88 knots, Fuel Burn = 8 gallons/hr

      - Time = 220nm/88 knots = 2.5 hrs

      - 2.5 hrs x 8 gallons/hr = 20 gallons used

  - This must be done for every segment of the flight in which the GS changes

  - Obtaining Flight Information

    - TC – Direction of the line connecting 2 desired points, drawn on the chart and measured clockwise in degrees from True N on the mid-meridian

    - TAS – Determined with the charts in the performance section of the POH or with the flight computer

    - Wind Correction Angle – Determined on the back side of the flight computer (+W/-E)

    - TH –Direction, in degrees clockwise from true N, which the nose should point for the desired course

    - Variation – obtained from the isogonic line on the chart (+W/-E to true heading)

    - MH – An intermediate step in the conversion (Variation applied to True Heading)

    - Deviation – Obtained from the card in the plane (Added/Subtracted from MH as necessary)

    - CH –Compass reading (found by applying deviation to MH) which is to be used for the desired course

    - Total Distance – Obtained by measuring the length of the TC line on the chart

    - GS – Obtained by using the back side of the flight computer

    - Estimated Time En Route – Total distance / GS

    - Fuel Rate – Predetermined gallons per hour used during flight

    - Fuel Consumption – Estimated time en route x Gallons per hour

- Using a Flight Log

  - The navigation log will assist in planning and conducing the flight

    - It prepares the information in a logical sequence allowing the pilot to track the progress of the flight

    - Necessary frequencies, etc are in order and easy to find for the pilot’s use

- Filing a Flight Plan

  - Not required but it is a good operating practice since the info can be used for search and rescue

  - Filing can be done on the ground or in the air

    - On the ground: Call the FSS (1800- WX BRIEF) or use DUAT

      - After T/O, contact the FSS by radio and give them the T/O time to activate the flight plan

      - Once filed, the flight plan will be held for an hour after the proposed departure time

  - Don’t forget to close the flight plan

    - The FAA will institute a search 30 min after the scheduled arrival time

- Diversion to an Alternate

  - There will probably come a time when you cannot make it to the planned destination

    - This can result from weather, malfunctions, poor planning, fuel, pilot/passenger fatigue/illness, etc

  - Before flight, check the route for suitable landing areas and for nav aids that can be used in a diversion

  - Take advantage of all shortcuts/rule of thumb computations when computing course/speed/distance

    - Use your thumb to estimate distance

    - Use a compass rose, airway or any other reference to determine the approximate new heading

  - Choose an alternate shown on your sectional or use the ’Nearest’ page in the GPS

  - Procedure

    - Confirm your present position on the sectional chart

    - Divert immediately toward the alternate using shortcuts/rule of thumb calculations

      - Completing all measuring, plotting, computations first may aggravate the situation

    - Once established on course, note the time

    - Use the winds aloft nearest the diversion point to calculate a heading and GS

      - Once determined, calculate a new arrival time and fuel consumption

      - Give priority to flying while dividing attention between navigation and planning

    - When determining an altitude, consider cloud heights, winds, terrain, etc

- Lost Procedures

  - Avoiding Becoming Lost

      - Always know where you are - Plan ahead, Know the next landmark/Anticipate Nav indications

    - If the radio nav systems/visual observations do not confirm expectations, take corrective actions

  - Once Lost

    - Don’t Panic

    - The Five C’s

      - Climb – This will allow you to see more ground, increasing chances of spotting a landmark

        - Improves radio reception, extends the transmitter range, and increases radar coverage

      - Communicate – use the frequencies on the chart, including RCO frequencies at VOR stations

        - A controller can provide radar vectors

        - Use 121.5 if the situation becomes threatening and squawk 7700

      - Confess – Tell any ATC facility the situation

      - Comply – Comply with any ATC suggestions

      - Conserve – Reduce power/AS for max endurance or range (whichever is appropriate)

    - In addition,

      - Check the HI with the magnetic compass (before resetting, note the direction of error)

        - This can help determine whether you are right or left of course

      - EX: if the compass indicates 10o > than the HI, you may be to the right of course

    - Use navigational radios (VOR/ADF) to attempt to plot your position in relation to two navaids

      - GPS can also be used to determine location

    - If near a town the name of the town may be visible on a water tower