# NIGHT OPERATIONS

Objective:

To teach the pilot about the factors affecting night flying and the risks involved so that he may be able to avoid accidents due to night flying.

Content:

- How the Eyes Work

  - Rods and Cones

  - Cockpit Lighting

- Disorientation and Night Optical Illusions

  - Autokinesis

  - False Horizon

  - Featureless Terrain

  - Runway Slopes

  - Ground Lighting

  - Verify Attitude by Reference to the Flight Instruments

- Pilot Equipment

  - Flashlights

  - Aeronautical Charts

  - Cockpit organization

- Preflight Inspection (FAR 91.205)

  - Required equipment for VFR flight at night

- Engine Starting

- Taxiing, Airport Orientation, and the Runup

- Takeoff and Climb

- In-Flight Orientation

  - Checkpoints

  - Clouds/Restricted Visibility

  - Nav Lights

- Traffic Patterns

- Approach and Landing

- Go Arounds

- Night Emergencies

References:

Airplane Flying Handbook – Chapter 10

Aircraft Flight Manual/Pilots Operating Handbook

Completion Standards:

The lesson is complete when the student displays knowledge of safe night flying operations and performs such operations on night flights to the satisfaction of the instructor.

Instructor Notes:

- How the Eyes Work

  - Rods and Cones

    - Two types of light sensitive nerve endings which transmit messages to the brain via the optic nerve

      - Cones - Responsible for color, detail, and far away objects

        - The cones are located in the center of the retina

      - Rods – Function when something is seen in the peripherals and provide vision in dim light

        - The rods are located in a ring around the cones

    - Both the cones and rods are used for vision in the day

      - But, without normal light, the process of night vision is placed almost entirely on the Rods

    - Rods, Cones, and Night Vision

      - Cones are located in the center of the retina (the layer upon which all images are focused)

        - There is a small pit called the fovea where almost all the light sensing cells are cones

          - This is the area where most looking occurs

      - The Rods

        - Make night vision possible

        - During daylight, objects can be seen by looking directly at them, but at night a scanning procedure to permit off center viewing is more effective

        - Rods are located are concentrated in a ring around the cones

          - In low light, the middle field of vision isn’t very sensitive, there is a “night blind spot”

          - Farther from the fovea, the rods are more numerous and are used to see in dim light

          - Since Rods don’t lie directly behind the pupils, off-center viewing is used for night flight

        - The problem with Rods is that a large amount of light overwhelms them and they take a long time to reset and adapt to the dark again

          - The rods can take approx 30 minutes to fully adapt to the dark

            - Once fully adapted the Rods are about 100,000x more sensitive to light

            - EX: Walking into a movie theatre

          - After the rods have adapted to the dark, the process is reversed when exposed to light

            - They eyes adjust to the light in a matter of seconds to the light

            - If the dark room is reentered, the 30 min process to adapt is started again

              - Therefore, it is important to avoid bright lights before and during a flight

    - Summary

      - Night vision is based on the rods and off center viewing is necessary since they are located around the center of vision

      - It is important to avoid bright lights before and during a flight to maintain adequate night vision

  - Cockpit Lighting

    - Cockpit lighting should be at a minimum brightness that will allow reading of the instruments and switches without hindering outside vision

- Disorientation and Night Optical Illusions

  - In addition to night vision limitations, be aware that night illusions can cause confusion

  - Autokinesis

    - Caused by staring at a single point of light against a dark background for over a few seconds

    - The light appears to move on its own

    - Prevent by focusing the eyes on objects at varying distances and avoid fixating

  - False Horizon

    - Caused when the natural horizon is obscured/not readily apparent

      - Generated by confusing bright stars and city lights

  - Featureless Terrain

    - An absence of ground features can create the illusion that the aircraft is higher than it actually is

    - Resulting in a tendency to fly a lower than normal approach

  - Runway Slopes

    - An upsloping runway/terrain can create the illusion that the aircraft is higher than it actually is

      - The pilot who does not recognize this will fly a lower approach

    - Downslope – The opposite applies

  - Ground Lighting

    - Regularly spaced lights along a road/highway/etc can appear to be runway lights

    - Lights on moving trains have been mistaken for runway/approach lights

    - Bright runway or approach lights can create the illusion the airplane is closer to the runway

  - Verify Attitude by Reference to the Flight Instruments

    - Reference to the flight instruments is the best way to cope with disorientation/optical illusions

      - If making an approach and an ILS or VASI is available make use of it

    - Visual references are limited – you will need to use more instruments (don’t be dependent on them)

    - If at any time the pilot is unsure of their position, a go around should be executed

- Pilot Equipment

  - Flashlight

    - Red or white light

      - White light is used to preflight the aircraft

      - Red light is used when performing cockpit operations as it will not impair night vision

        - When using a red light on an aeronautical chart, the red colors will wash out

  - Aeronautical Charts

    - If the intended course of flight is near the edge of a chart, the adjacent chart should be available

      - City lights can be seen at far distances and confusion can result without the necessary charts

  - Regardless of equipment, organization eases the burden on the pilot

- Preflight Inspection (FAR 91.205)

  - Required equipment for VFR flight at night

    - TOMATO FFLAMES and FLAPS

  - Fuses

  - Landing Light

  - Anti Collision Lights

  - Position Lights

  - Source of Power

    - Instrument required equipment doesn’t hurt

  - Walk Around

    - Preflight inspection is still necessary

    - White light flashlight is good

    - Check all aircraft lights

    - Check the ramp for obstructions

- Engine Starting

  - Be very sure the propeller area is clear

    - Turn on position and anti-collision lights prior to start

    - Announce “Clear Prop”

  - Keep all unnecessary electrical equipment off to avoid draining the battery

- Taxiing, Airport Orientation, and the Runup

  - Taxiing

    - Due to restricted vision, taxi speeds should be reduced

      - Don’t taxi faster than a speed that will allow a stop within the distance you can clearly see

    - Use the landing/taxi lights as necessary – Caution overheating (no airflow)

    - Do not use strobes/landing lights in vicinity of other aircraft

      - Can be distracting/blinding

  - Orientation

    - Airport Diagram

    - Understanding taxiway markings, lights, and signs

  - The Runup

    - The before taxi runup should be performed with the checklist as usual

    - Forward movement of the airplane may not be easy to detect

      - Hold/lock the brakes and be alert that the airplane could creep forward without being noticed

- Takeoff and Climb

  - Clear the area for approaching traffic – Final Approach

    - Uncontrolled airports: Make a 360o turn in the direction of air traffic to clear

  - After receiving clearance, align the airplane with the centerline

    - Check to ensure the MC and HI match the runway intended

  - Perform a normal takeoff depending more on the instruments as many visual cues are not available

    - Perception of runway width, airplane speed, and flight attitude will vary at night

      - The flight instruments should be checked frequently

    - As the AS reaches VR, the pitch attitude should be adjusted to establish a normal climb

      - Refer to outside visual references (such as lights) and the flight instruments

  - Climb

    - To ensure the airplane is climbing check the AI, VSI and Altimeter; the darkness makes it hard to tell

    - Necessary adjustments should be made by referencing the attitude and heading indicators

      - Recommended that turns not be made until reaching a safe maneuvering altitude

- In-Flight Orientation

  - Checkpoints – Although there are less of them, it does not pose a problem

    - Light patterns of towns are easily identified

    - Rotating beacons are useful

    - Highways

    - Ensure you maintain orientation as it is easier to become disoriented in relation to location

      - Continuously monitor position, time estimates, and fuel consumed

    - NAVAIDS should be used whenever possible

  - Clouds/Restricted Visibility

    - It is difficult to see clouds – exercise caution to avoid flying into MVFR/IFR weather conditions

      - 1st indication - Gradual disappearance of the ground and glowing around lights

  - Nav Lights

    - Red Light on Left Wing/Green Light on Right Wing and White Light on the Tail

      - Use to orient another aircrafts direction in relation to your own

- Traffic Patterns

  - Identify runway/airport lights as soon as possible

    - May be difficult to find the airport or runways

      - Fly towards the beacon until you identify runway lights

      - Compare the runway lights with HI to ensure you are in the right place

  - Distance may be deceptive at night due to limited light conditions

    - A lack of references on the ground and the inability to compare their location and size cause this

    - More dependence must be put on the instruments (Particularly the Altimeter and ASI)

  - Landing light should be on for collision avoidance

  - Fly a normal traffic pattern

    - Know the location of the runway/approach threshold lights at all times

    - When entering, allow for plenty of time to complete the before landing checklist

      - Execute the approach in the same manner as during the day

- Approach and Landing

  - A stabilized approach should be made in the same manner as during the day

  - Use flight instruments more often (especially altimeter/ASI)

    - Distance, etc may be deceptive

    - Maintain specified airspeeds on each leg and watch VSI to keep the approach under control

  - Final Approach

    - If there are no centerline lights, align the airplane between the edge lights

      - Note and correct any wind drift

    - Power and pitch corrections to maintain a stabilized approach

      - Use approach lights (VASI, PAPI, etc) to maintain glideslope

  - Roundout/Touchdown

    - A smooth, controlled roundout and touchdown should be made in the same manner as in the day

      - Judgment of height, speed, and sink rate may be impaired

        - There may be a tendency to round out too high

    - Start the roundout when the landing lights reflects on the tire marks on the runway

      - In the case you have no landing light/can’t see tire marks start the roundout when the runway lights at the far end appear to be rising higher than the airplane

      - More of a feel for the airplane in this situation

- Go Arounds

  - Prompt decision is even more necessary at night due to the restricted visibility

    - Be prepared in case the maneuver is necessary

- Night Emergencies

    - Electrical

  - The greatest electrical load is placed on the system at night = the greatest chance of failure

  - In the case of a suspected problem

      - Reduce load as much as feasible

      - If total failure is expected, land at the nearest airport immediately

  - Engine

    - Don’t Panic - Establish a normal glide and turn toward an airport or away from congested areas

    - Check to determine the cause and correct immediately if possible (Engine restart checklist)

    - If no restart – Maintain positive control of the airplane at all times!

      - Maintain orientation with the wind – don’t land downwind

      - Check the landing lights and use them on landing if they work

    - Announce the emergency to ATC or UNICOM (If on a frequency, don’t change unless instructed to)

    - Consider an emergency landing area close to public access

      - Before landing checklist

      - Touchdown at the slowest possible AS

    - After landing, turn off all switches and evacuate as quickly as possible