# NON-PRECISION INSTRUMENT APPROACH

Objective:

To teach the student the elements, proper procedures and technique for piloting an airplane on a non-precision instrument approach.

Content:

- Elements of a Non-Precision Instrument Approach

  - Selection of the appropriate instrument approach procedure chart

  - Pertinent information on the selected instrument approach chart

    - Approach Name

    - Minimum safe/sector altitude (MSA)

    - Communications frequencies

    - Plan view

    - Profile view

    - Minimums section

    - Airport diagram

    - New NACO IAP charts

    - What to Memorize

  - Communication with ATC and compliance with ATC clearances, instructions and procedures

  - Aircraft configuration, airspeed and checklist items

  - Selection, tuning, identification of navigation equipment and stations

  - Adjustments applied to the published MDA and visibility criteria for the aircraft approach category

  - Maintenance of altitude, airspeed and track, where applicable

  - Rate of descent during the final approach segment

  - Weather Factors

- Common Errors

  - Failure to have knowledge on the instrument approach chart

  - Incorrect communications procedures or noncompliance with ATC clearances

  - Failure to accomplish checklist items

  - Faulty basic instrument flying technique

  - Descent below the MDA

References:

14 CFR Part 91, AIM, NACO TPPs

Aviation Instructor's Handbook, FAA-H-8083-9

Instrument Flying Handbook, FAA-H-8083-15, 1999

Instrument Rating ACS

Completion Standards:

The lesson is complete when the student can accurately perform the steps involved in flying a non-precision instrument approach using the materials covered in this lesson to the satisfaction of the instructor.

Instructor Notes:

- Elements of a Non-Precision Instrument Approach

  - Selection of the appropriate instrument approach procedure chart

    - 14 CFR Sec. 91.175 requires use of standard IAP prescribed for the airport by 14 CFR part 97

    - Select IAP chart from current TPP

    - GPS - use current database

  - Pertinent information on the selected instrument approach chart

    - Name contains

      - Navigation system(s) providing final approach course (FAC) guidance (VOR RWY 7)

        - "/" => both required (VOR/DME RWY 27)

        - "or" => either one sufficient (VOR or GPS RWY 23)

      - Runway number if FAC aligned within 30° of runway centerline

      - Letter from beginning of alphabet instead of number if FAC not aligned within 30° of runway (VOR-A)

    - Minimum safe/sector altitude (MSA)

      - 25(-30)NM radius

      - 1000 feet obstacle clearance

      - Maybe no navigation signal

      - For emergency use

    - Communications frequencies

    - Plan view

      - IAF(s)

      - Approach segment routes (course, distance, altitude)

      - FAC

      - NAVAIDs and frequencies

      - PT or NoPT

      - Holding pattern(s)

      - Notes

    - Profile view

      - FAF Maltese cross

      - Altitudes to fly

      - FAC length

      - Visual descent angle (VDA) from FAF to TCH

      - Visual descent point (VDP)

      - Missed approach instructions

    - Minimums section

      - Know aircraft category based on 1.3 x VSo

        - C-TR182 - cat A but

        - Circling at 100 KIAS, use category B circling minimums

      - MDAs

      - Minimum visibility required for landing

      - Notes

    - Airport diagram

      - Runway dimensions, lighting

      - FAF to MAP time table Based on ground speed

    - New NACO IAP charts

      - Pilot briefing section (EZ Brief)

      - Missed approach symbols

      - RNAV RWY # charts `

        - For GPS, WAAS, LAAS

        - Up to 4 lines of approach minimums GNS 530 use LNAV minimums

        - TAA icons

      - DA replaces DH

    - Memorize

      - MDA (How low?)

      - Time from FAF to MAP (How long?)

      - Visibility minimum required (How far?)

      - Missed approach procedure (at least first 2 steps) (Which way?)

  - Radio communication with ATC and compliance with ATC clearances, instructions and procedures

    - Center -> approach control -> tower, FSS or CTAF

    - Missed approach: first fly, get complete control of stable climb then: CTAF, tower or FSS -> approach control

    - ATC clearance, instructions

      - Execute on receipt or request amendment

      - "Cleared for approach"

        - Fly any approved IAP but

        - NOT contact or visual

      - Fly entire approach as published

  - Appropriate aircraft configuration, airspeed and checklist items

    - Use predetermined configurations

    - Slow to approach airspeed during initial approach segment

    - Before landing checklist

      - Initial approach segment: GU('wheels to go')MPS

      - FAF: GU(gear down)MPS check again

      - FAC: GUMPS (gear down, locked)

  - Selection, tuning, identification and determination of operational status of ground and aircraft navigation equipment

    - IAP chart contains frequencies and Morse code identifiers for the approach

    - Confirm correct Morse code and/or voice identification

    - Operational status check of each NAVAID to be used

      - Note VOR not flagged, CDI stable (except at station passage)

      - Monitor ADF Morse code identification, check test function

      - GPS approach

- Correct approach and IAF (or vectors) selected from current database

- Approach loaded and activated

- No RAIM failure/status annunciations

      - LOC frequency selected: note that rotating OBS has no effect on CDI

  - Adjustments applied to the published MDA and visibility criteria for the aircraft approach category

    - Check NOTAMs for IAP amendments

    - Inoperative runway visual aids increase minimum visibility required (bot not MDA)

      - Check Inop Components Table inside front cover of TPPs

      - Check minimum section of JEPP chart

    - Some IAPs require a higher MDA when alternate (not local) altimeter setting is used

    - During circling approach, apply circling minimums section using actual airspeed (Cat B for 100 KIAS)

  - Maintenance of altitude, airspeed and track, where applicable

    - Beginning prior to FAF, maintain

      - Altitude +/- 100 feet (aim for +/- 20 feet)

      - Heading +/- 10° with stable wind drift correction (aim for +/- 2°)

      - Airspeed +/- 10 knots (aim for +/- 2 knots)

      - CDI less that ¾-scale deflection (aim for < 1 dot)

    - Maintain MDA +100 feet, -0 feet to MAP

    - C-TR182: maintain approach airspeed, 100 KIAS, during missed approach climb

  - Establishment and maintenance of an appropriate rate of descent during the final approach segment

    - Establish rate of descent and track that will ensure arrival at MDA prior to reaching the MAP with airplane continuously in a position from which descent to a landing can be made at a normal rate using normal maneuvers

    - C-TR182: expeditious constant airspeed (100 knots) descent at 800-1000 fpm to allow more time at MDA to see runway environment

    - FAA recommends using visual descent angle (VDA), when available, to calculate desired constant airspeed rate of descent (e.g., VDA 3.00° and 100 KIAS => 538 fpm)

  - Factors that should be considered in determining whether

    - The approach should be continued straight-in to a landing

      - If runway environment (14 CFR Sec. 91.175) clearly seen and

      - Visibility greater than required minimum and

      - Position of aircraft such that straight-in approach and landing can be made using normal maneuvers (no more than small, shallow bank turns or small vertical speed adjustments), then

      - Transition from instrument to visual cues and descent from MDA or VDP to straight-in landing may be made

    - A circling approach to a landing should be made

      - Circling approach required if FAC deviates from runway centerline by >30° (>15° for GPS IAP)

      - Landing on runway with clearly favorable wind conditions may require a circling approach to that runway after flying an approach to another runway

    - A missed approach should be performed

      - At MAP or any time below MDA until touchdown if

        - Runway environment (14 CFR Sec. 91.175) not clearly seen

        - Visibility less than required minimum, or

        - Aircraft not in position for safe landing with normal maneuvering

      - Part of airport not clearly visible during circling approach above MDA

      - ATC request

- Common Errors

  - Failure to have essential knowledge of the information on the instrument approach chart

    - Plan ahead

    - Know approach(es) to expect (ATC, ATIS)

    - Study, record, memorize key data in advance

How low? How long? How far? Which way?

    - Use PC or simulator to practice flying approaches to destinations of upcoming flights

  - Incorrect communications procedures or noncompliance with ATC clearances or instructions

If cannot comply, request amendment

  - Failure to accomplish checklist items

    - Start before landing checklist on initial approach segment or on initial vectors for approach

    - Complete before landing checklist at FAF (gear down)

  - Faulty basic instrument flying technic

May be due to cockpit disorganization and increased workload

  - Inappropriate descent below the MDA

    - May indicate inadequate speed of cross-check and interpretation during high workload time

    - Add 50 foot (or more) buffer above MDA, especially in turbulence