# OPERATION OF SYSTEMS

- Objectives:

- To familiarize the student with the operation of various aircraft systems so that the student may have a better understanding of his aircraft.

- Content:

- Powerplant

  - Reciprocating engines

    - Radial, In-line, V-Type, Opposed

    - Liquid or air cooled

    - 4-stroke operating cycle (Intake, Compression, Power, Exhaust)

- Propeller

  - Angle of incidence – for prop, angle formed by chord line and reference plane containing prop hub

  - Fixed Pitch Propeller (2 types)

  - Climb Propeller

  - Cruise Propeller

  - Tachometer as indicator of engine power

  - Lower density, less power

  - Controllable Pitch Propeller

  - Constant Speed Propeller

    - Acts as fixed pitch prop when pitch stops contacted

    - Manifold pressure as indicator of engine power (MAP)

      - For given RPM there is Manifold pressure that shouldn’t be exceeded (accordance w/ mfcr)

- Induction Systems (2 types)

  - Carburetor system

    - Float-type and pressure-type

      - Icing most likely @ temps < 70?F (21?C) relative humidity > 80%

      - Carb Heat in float-type

    - Fuel injection system

      - Impact icing

  - Superchargers

    - Increase service ceiling

    - Sea level engine (sea level supercharger)

    - Altitude Engine

    - Turbosuperchargers

      - Critical altitude

      - Turbine and compressor

      - Waste gate

      - Intercooler

      - Overboost

  - Excessively rich mixture causes carbon buildup/spark plug fouling

  - Excessively lean mixture causes detonation

- Ignition System

  - Dual ignition systems

  - Combustion

  - Detonation

  - Preignition

- Fuel Systems (figures 5-15, 5-17 PHAN)

  - Gravity-fed

  - Fuel-pump, primer, tanks, gauges, selectors, strainers, sumps, and drains

  - Fuel Grades

  - Refueling procedures

- Starting System

- Oil Systems

  - Recip. Engines use wet sump or dry sump oil system

- Engine Cooling Systems

  - Cowl flaps

  - Cylinder Head Temperature Guage (CHT)

  - Exhaust Systems

  - Exhaust Gas Temperature (EGT) and leaning

  - Electrical Systems

  - 14- or 28-volt direct current system

  - Ground Power Unit (GPU) receptacle

  - Alternator/Generator

  - Ammeter/Loadmeter

  - Voltage regulator

- Hydraulic Systems

  - Single- or double-acting servos

- Landing Gear

  - Tricycle (Nosewheel)

    - Steerable or Castering

  - Conventional (Tailwheel)

  - Fixed and Retractable

- Brakes

- Autopilot

- References:

- Airplane Flight Manual / Pilot’s Operating Handbook

- Pilot’s Handbook of Aeronautical Knowledge – Chapter 5

- Completion Standards:

- The lesson is complete when the instructor determines that the student has adequate knowledge of the aircraft systems by giving an oral or written exam.

- Instructor Notes:

- Primary Flight Controls and Trim

  - Ailerons (CFRP)

    - Actuated via push rods

    - Attached with stainless steel and aluminum hinges

  - Elevator (CFRP)

    - Actuated via push rods

    - Semi-Monocoque sandwich construction

    - Trim

      - Controlled by a Rocker Switch

        - Switch forward = Nose Down; Switch aft = Nose Up

        - Switch controls an electrical actuator beside the vertical push rod in the vertical stabilizer

          - The actuator applies a load to compression springs on the elevator push rod

      - Trim circuit breaker can be tripped manually to disable the system

  - Rudder

    - Actuated via control cables

    - Semi-Monocoque sandwich construction

- Flaps

  - Driven by an electric motor

    - Electric flap actuator is protected by a circuit breaker (5 Amp)

      - Located on the R side of the instrument panel and can be manually tripped to disable the system

  - Controlled by 3 position flap operating switch on the instrument panel

    - Top position – Cruise – 0o (Green Light)

    - Middle Position – Approach – 15o (Yellow Light)

    - Bottom Position – Landing – 45o (Yellow Light)

    - When two lights are illuminated at the same time, the flaps are in-between positions

  - Cruise and Landing positions are equipped with position switches to prevent over-traveling

- Powerplant and Propeller

  - Powerplant

    - Continental IO-240-B Engine

      - Fuel Injected

      - 4 Cylinder

      - 4 Stroke

      - Horizontally Opposed cylinders and heads

      - Air cooled cylinders and heads

      - Propeller drive is direct from the crankshaft

      - 3.9 liters

      - 125 HP at 2800 RPM

    - Engine Controls

      - Mixture Lever

      - Throttle

      - Alternate Air

        - Full Forward = Primary Air Intake

        - Full Aft = Alternate Air Intake

  - Propeller

    - Hoffmann HO-14HM-175-157

      - Diameter - 5’ 8.9”

      - Two-bladed fixed pitch propeller

      - Wood and glass fiber

    - Sensenich W69EK7-63, 63G, or W69EK-63

      - Diameter – 5’9”

      - Two-bladed fixed pitch propeller

      - Wood prop

- Landing Gear

    - Two main landing gear wheels (mounted to aluminum spring struts)

  - Nose Wheel

    - 60o castering

    - Suspension is provided by an elastomer spring

  - Wheel Brakes

    - Hydraulically operated disc brakes

    - Operated individually using toe-brake pedals

    - Failure

      - If either the left or right wheel brake system on the pilot’s side fail, the co-pilot’s brakes fail too

      - If copilot’s master brake cylinder/input lines to the pilot’s master cylinder fail the pilot’s brakes will stop operate

  - Parking Brake

    - Repeated pushing of the toe-brake pedals will build up the required brake pressure, which will remain in effect until the brake is released

- Fuel, Oil, and Hydraulic

  - Fuel

    - Aluminum Fuel Tank

      - Located behind the seats, below the baggage compartment

      - 24.5 Gallons fuel (24 Gallons Usable)

      - Operation

        - Fuel is gravity fed to a filter bowl (gascolator) and then to the electric fuel pump

          - Filter bowl must be drained before flight (black tube)

        - Electric fuel pump primes for starting (Prime ON) and is used for low throttle operations

          - When the pump is off, fuel flows through the pump’s internal bypass

        - From electric pump, fuel is delivered to the mechanical fuel pump by the fuel supply line

        - Fuel is metered by the fuel control unit and flows via the fuel distribution manifold to the injector nozzles

        - Return line from mechanical fuel pump’s fuel vapor separator returns vapor/excess fuel to the tank

      - Electric Fuel Pump

        - DUKES constant flow, vane type, two speed, electric fuel pump

          - Fuel Prime

            - Pump’s high speed setting, used for priming the engine prior to engine start

            - Turning the prime pump on while running will enrichen the mixture considerably

              - At high throttle settings the effect is less noticeable

              - At low throttle settings may cause engine roughness or engine stoppage

            - Fuel pump

              - Required for maintaining positive fuel pressures at low throttle settings

      - Fuel Shutoff Valve

        - Closing will cause the engine to stop within a few seconds

  - Oil

    - High pressure wet sump lubrication

    - Oil is pumped by a mechanical, engine driven pump

    - Oil must be between 4 and 6 quarts

- Electrical

  - Power Generation

    - 40 Amp Generator

      - Feeds the Main Bus via the Generator Circuit Breaker (50 Amps)

    - Generator Warning Light

      - Activated by internal voltage regulator monitoring circuit - illuminates if generator fault occurs

  - Power Storage

    - 12V battery

      - Connects to the Master Bus via the Battery Circuit Breaker (50 Amps)

  - Power Distribution

    - Electrical Bus

  - Consumers

    - Individual consumers (e.g. Radio, Fuel Pump, Position Lights, etc.) are connected in series with their respective circuit breakers

  - Electrical Monitoring Instruments

    - Voltmeter

      - Indicates the status of the Electrical Bus

      - Consists of a dial marked numerically from 8 – 16 volts in divisions of 2

        - Scale

          - RED for 8.0 – 11.0 volts

          - YELLOW for 11.0 – 12.5 volts

          - Green for 12.5 – 16.0 volts

          - REDLINE at 16.1 volts

    - Ammeter

      - Indicates the charging (+) and discharging (-) of the battery

      - Consists of a dial which is marked numerically from -60 to 60 amps

    - Generator Warning Light

      - Illuminates during generator failure

        - No output from the generator

      - The only remaining power source is the battery (20 Amps for 30 min)

- Avionics

  - Center of the instrument panel contains the radio and navigation equipment

    - Operating instructions should be taken from the manuals of the respective manufacturers

  - Vertical Stabilizer contains a di-pole antenna for the VHF radio equipment

  - Horizontal Stabilizer contains an antenna for the NAV equipment (VOR)

- Pitot-static, Vacuum Pressure, and Associated Flight Instruments

  - Pitot-Static

    - Pitot pressure is measured on the leading edge of a calibrated probe below the left wing

      - Airspeed indicating error, refer to Chapter 5 of POH

    - Static pressure is measured by the same probe

      - Error of the static pressure system is negligible

    - Instruments

      - Airspeed Indicator

        - Pitot Pressure (Ram Air Pressure) and Static Pressure

      - Altimeter

        - Static Pressure

      - Rate of Climb Indicator

        - Static Pressure

  - Vacuum Pressure (Gyros)

    - Instruments

      - Heading Indicator

      - Attitude Indicator

  - Electrically Driven Instruments

    - Turn Coordinator

- Environmental

  - Cabin Heat and Defrost

    - Directs ram air through the exhaust heat shroud into the cabin heat valve

      - The air is directed to the window defrosting vents and to the cabin floor as selected by the lever

    - The cabin heat selector is used to regulate the flow of heated air