# PERFORMANCE AND LIMITATIONS

- Objectives:

- The student should develop knowledge of the elements related to airplane performance and limitations as required in the necessary PTS.

- Content:

- Structure of the atmosphere

  - Atmospheric pressure

    - Temperature/pressure lapse rates

  - Pressure altitude

    - Standard datum plane

  - Density altitude

    - Effects of pressure on density

    - Effects of temperature on density

    - Effect of humidity on density

- Performance

  - Power – work per unit time

  - Thrust – the force that imparts a change in the velocity of a mass

  - Straight-and-level

  - Climb performance

    - Reserve power

  - Range performance

    - Specific range

    - Specific endurance

    - Cruise control

    - Lift/Drag ratio

    - Brake specific fuel consumption

    - Brake horse power

  - Ground Effect

  - Region of Reversed Command

  - Runway surface and gradient

  - Water on runway and dynamic hydroplaning

- Takeoff/Landing Performance

- Performance Speeds

- Performance Charts

  - Interpolation

  - Density altitude charts

  - Takeoff charts

  - Climb and cruise charts

  - Crosswind and headwind component chart

  - Landing charts

  - Stall speed performance charts

- References:

- Airplane Flight Manual / Pilot’s Operating Handbook

Pilot’s Handbook of Aeronautical Knowledge – Chapter 9

- Completion Standards:

- The lesson is complete when the instructor determines that the student has adequate knowledge of performance and limitations of aircraft by giving an oral or written exam.

- Instructor Notes:

- Determining Weight and Balance

  - CG = Total Moment divided by Total Weight

    - Begin with the empty weight and make a list of everything that will be loaded in the airplane

      - People, items, and fuel (note the weights of everything as well)

      - Be sure the weight of what you want to load is within the max weight limits

        - If the total weight is too high, remove items/people to get in weight limits

    - Calculate the Moments of each item

      - To find the moments, use the graph or multiply the weight by the arm in the POH

        - The weight/moment of the airplane are found in its weight and balance documents

    - Then calculate the CG – (Total Moment/Total Weight)

      - For the DA20, compare the Total Weight and Total Moment on the graph in supplement 4

    - Use the chart in Supplement 4 to determine whether the airplane is within limits

- Atmospheric Conditions and Performance

  - Atmospheric Pressure

    - Though air is light, it has mass and is affected by gravity and therefore, it has a force

    - Under standard conditions at sea level, the average pressure exerted is approx 14.7 lbs/in

    - Since air is a gas, it can be compressed or expanded

    - Density of the air has significant effects on the airplane’s performance

      - As the density of the air increases (lower DA), airplane performance increases and vice versa

  - What Changes Air Density?

    - Barometric Pressure, Temperature, Altitude, and Humidity all affect air density

      - Density varies directly with pressure - As pressure increases, density increases and vice versa

      - Density varies inversely with temperature – As temp increases, density decreases and vice versa

      - Density varies inversely with altitude - As altitude increases, density decreases and vice versa

      - Density varies inversely with humidity – As humidity increases, density decreases and vice versa

  - How it affects Performance

    - As it becomes less dense, it reduces:

      - Power since the engine takes in less air

        - Power is produced in proportion to air density (As density increases, power does too)

      - Thrust since the propeller is less effective in thin air

        - Thrust is produced in proportion to the mass of air being accelerated

      - Lift because the thin air exerts less force on the airfoils

        - As air density decreases, the lift efficiency of the wing is decreased

  - Leaning the Engine

    - At power settings less than 75% or at DAs > 5,000’ the engine must be leaned for max power on T/O

      - The excessively rich mixture deters engine performance

    - At higher elevations, high temps may have such an effect on DA that safe ops may not be possible

      - Even at lower temps, with excessive temps/humidity, performance can be marginal and weight may need to be reduced

- Performance Charts

  - Airplane performance is found in Section 5 of the POH (Performance and Limitations)

    - Supplement 4, for the DA20 (any charts not shown in the supplement are found in Chapter 5)

  - Using the performance charts, and the accompanying instructions, we can calculate

    - Cruise Performance

    - Stall Speeds based on airplane configuration

    - Wind Components (Crosswind and Headwind)

    - T/O Distance and Landing Distance

    - Climb Performance (In cruise and T/O configurations as well as Balked Landing)

    - TAS

    - Maximum Flight Duration (Chart in which the PA is combined with RPM to find % bhp, KTAS, GPH)

  - In order to make use of these charts we need to know the Pressure Altitude

    - Pressure Altitude – The altitude indicated when the altimeter setting window is set to 29.92

      - PA = 1,000(29.92-Current Altimeter Setting)+Elevation

        - EX: Altimeter = 30.42 and Elevation = 808, so PA = 308’

        - EX: Altimeter = 29.84 and Elevation = 808, so PA = 888’

    - From PA we can compute DA

      - DA – PA corrected for non standard temperature (Directly related to airplane performance)

      - DA = 120(Current Temperature-15oC)+PA

        - EX: Temp = 23oC and PA = 308’, so DA = 1,268’

        - EX: Temp = 03oC and PA = 308’, so DA = -1,132

  - Once we have PA, we can start at the temp at the bottom of the chart and move upwards to the PA

    - From there, we move straight across until we reach the next stage of the chart we’re on

      - Once we reach the next step, we follow the trend line and then, again, move straight across

  - This is done until we reach the performance number

- Determining the Required Performance is Attainable

  - Obviously, the performance charts and relating them to the airport’s information (runway lengths, etc)

    - The charts will provide performance for all phases of flight

    - But, remember, the charts don’t make allowance for pilot proficiency or mechanical deterioration

      - Does the airplane have problems that may limit performance?

  - Possibility of, or the expected changing of weather resulting in useless original calculations

    - Just because the plane will perform well now doesn’t mean it will perform well later

- Exceeding Airplane Limitations

  - Operating Limitations are in Chapter 2 of the POH

    - The limits here establish the boundaries in which the airplane can be safely operated

    - Adverse Affects

      - Attempting to T/O or Land without enough runway

        - Can result in a crash into an obstacle or over-running the runway also damaging the plane

      - Attempting to clear an obstacle that the airplane performance will not allow at a certain weight

        - This can result in crashing into the obstacle

      - Not having enough fuel to reach the airport of intended landing, cruising at a high power setting

        - Can result in an emergency landing

      - Using the wrong type of fuel

        - Can result in detonation, causing large amounts of damage to the engine

      - Exceeding the structural or aerodynamic limits by being overweight or outside CG limits

        - Can result in airplane damage or structural failure

        - Airplane control may be hampered and stall speeds may be affected

      - Exceeding the maximum crosswind component (20 knots)

        - This will greatly increase the difficulty of the landing, possibly resulting in a crash

        - The airplane may not have the ability to stay aligned with the runway, resulting in a crash