# POWER-OFF STALLS (PROFICIENCY) (ASEL and ASES)

Objective:

To familiarize the student with the conditions that produce stalls. To assist in recognizing an approaching stall. To develop a habit in the student of taking prompt, preventive or corrective action. To better prepare the student to deal with stalls during approach to landing.

Before Flight:

- Review:

  - What is a stall?

  - When can a stall occur?

  - What do power-on/power-off/turning stalls simulate?

  - Wing washout

  - Warnings before a stall occurs:

  - Visual

  - Aural

  - Feel

  - Stall warning devices

- Steps to stall recovery:

  - Release back elevator pressure or apply forward pressure

  - Advance throttle

  - Regain straight and level flight using all controls

  - Aileron/rudder control should be used with care

  - Recovery should be completed no lower than 1,500 feet AGL during practice

Maneuver Elements:

- Setup

  - Clear the area

  - Choose forced landing area

  - Configure aircraft: gear down, full flaps, carburetor heat on, power set for final approach to land, altitude so recovery is ≥1500’ AGL

  - Select outside references

  - Start a descent (as if on final approach) using power and pitch

  - Power to idle

  - Smoothly raise nose until a stall is induced, maintaining constant pitch with the elevator

  - Maintain coordination (ball centered) and neutral ailerons

- Recovery

  - Reduce the angle of attack by releasing back-elevator pressure

  - Advancing the throttle to maximum power

  - Carburetor heat off, retract first notch of flaps immediately

  - Anticipate left-turning tendencies with right rudder pressure

  - Continue to lower nose to regain flying speed, slowly returning to level flight

  - Upon positive rate of climb, retract flaps and gear are as necessary

  - When in level flight, reduce power to a setting for cruise flight or climb as necessary

  - Maintain ball centered

  - Look for traffic

Aircraft Setup:

Flaps down, gear down

Approach power:  (C172RG: 18” Hg, 2700 RPM)

Approach Speed:  (C172RG: 65 KIAS)

Things to Avoid (Common Errors):

- Failure to adequately clear the area

- Inability to recognize an approaching stall condition through feel for the airplane

- Premature recovery

- Over-reliance on the airspeed indicator while excluding other cues

- Inadequate scanning resulting in an unintentional wing-low condition during entry

- Excessive back-elevator pressure resulting in an exaggerated nose-up attitude during entry

- Inadequate rudder control

- Inadvertent secondary stall during recovery

- Failure to maintain a constant bank angle during turning stalls

- Excessive forward-elevator pressure during recovery resulting in negative load on the wings

- Excessive airspeed buildup during recovery

- Failure to take timely action to prevent a full stall during the conduct of imminent stalls

Diagrams:

References:

Airplane Flying Handbook - Chapter 4

Completion Standards:

The lesson is complete when the student is able to perform power-off stalls to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.