# POWER-ON STALLS (PROFICIENCY) (ASEL and ASES)

Objective:

To familiarize the student with the conditions that produce stalls. To assist in recognizing an approaching stall. To develop a habit in the student of taking prompt, preventive or corrective action. To better prepare the student to deal with stalls during takeoff and climb out.

Before Flight:

- What is a stall?

- When can a stall occur? (High Density Altitude)

- What do power-on/power-off/turning stalls simulate?

- Wing washout

- Warnings before a stall occurs:

- Visual

- Aural

- Feel

- Stall warning devices

- Steps to stall recovery:

- Release back elevator pressure or apply forward pressure

- Advance throttle

- Regain straight and level flight using all controls

- Aileron/rudder control should be used with care

- Recovery should be completed no lower than 1,500 feet AGL during practice

Maneuver Elements:

- Setup

  - Clear the area

  - Choose forced landing area

  - Configure aircraft for just before take-off: gear down, no flaps, carburetor heat on, slow to normal take-off speed with max propeller RPM, altitude so recovery is ≥1500’ AGL

  - Select outside references

  - Throttle to full (carburetor heat off) while simultaneously applying back-elevator pressure to smoothly raise nose to a high pitch attitude

  - Maintain back-elevator pressure at its full limit until airspeed, bleeding airspeed until it falls beyond Vs1 and stalls

  - Maintain coordination (ball centered) and neutral ailerons

- Recovery

  - Reduce the angle of attack by releasing back-elevator pressure

  - Simultaneously increasing throttle to full (if not already)

  - Anticipate left-turning tendencies with right rudder pressure

  - Return nose to straight-and-level coordinated flight

  - Maintain ball centered

  - Upon positive rate of climb, retract flaps and gear as necessary

  - Look for traffic

Aircraft Setup:

Flaps up, gear down

Normal take-off speed:  (C172RG: 15” Hg, 2700 RPM, 65 KIAS)

Vs1:  (C172RG: 50 KIAS)

Things to Avoid (Common Errors):

- Failure to adequately clear the area

- Inability to recognize an approaching stall condition through feel for the airplane

- Premature recovery

- Over-reliance on the airspeed indicator while excluding other cues

- Inadequate scanning resulting in an unintentional wing-low condition during entry

- Excessive back-elevator pressure resulting in an exaggerated nose-up attitude during entry

- Inadequate rudder control

- Inadvertent secondary stall during recovery

- Failure to maintain a constant bank angle during turning stalls

- Excessive forward-elevator pressure during recovery resulting in negative load on the wings

- Excessive airspeed buildup during recovery

- Failure to take timely action to prevent a full stall during the conduct of imminent stalls

Diagrams:

References:

Airplane Flying Handbook - Chapter 4

Completion Standards:

The lesson is complete when the student is able to perform power-on stalls to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.