# PREFLIGHT INSPECTION (ASEL and ASES)

- Objectives:

- To familiarize the student with the visual checks to be performed on an aircraft before flight and to teach him what items should be checked to ensure safety.

- Content:

- Required items on board airplane (ARROW)

  - Airworthiness Certificate

  - Registration Certificate

  - FCC radio station license, if required by the type of operation

  - Airplane operating limitations (AFM/POH, placards, instrument markings, or any combination thereof)

  - Weight and Balance

- Inside the cockpit

- Outer wing surfaces and tail section

- Fuel and oil

- Landing gear, tires, and brakes

- Engine and propeller

Things to Avoid (Common Errors)

- Failure to use or the improper use of a checklist

- Hazards which may result from allowing distractions to interrupt a visual inspection

- Inability to recognize discrepancies to determine airworthiness

- Failure to ensure servicing with the proper fuel and oil

- Failure to ensure proper loading and securing of baggage, cargo, and equipment

- References:

- Role of Preflight Preperation (AC 61-84)

Airplane Flying Handbook - Chapter 2

Aircraft Flight Manual / Pilot’s Operating Handbook

- Aircraft Checklist

- Completion Standards:

- The lesson is complete when the instructor determines that the student has adequate knowledge of visual inspections of aircraft and can perform a visual inspection adequately by giving an oral or written exam and a practical application test.

- Instructor Notes:

- Reasons for the Preflight Inspection

  - To determine the airplane has no obvious problems before taking off

- Checklist

  - Each airplane has a specific preflight procedure designed by the manufacturer – Section 4 of the POH

    - Using a different checklist will result in missing equipment and confusion

    - CE - Failure to use or the improper use of a checklist

      - Checklists ensure all necessary items for a specific airplane are checked in a logical sequence

      - They are not just a crutch for poor memory

  - Always have the checklist to be used as a reference to ensure everything is checked

  - CE - Hazards which may result from allowing distractions to interrupt a visual inspection

    - Distractions may result in missing items on the checklist or not recognizing a discrepancy

    - If distracted, start at the beginning or repeat the preceding two or three items

- The Preflight Overview

  - The preflight will logically move around the airplane in order to ensure it is in a condition for safe flight

  - It should begin while approaching the airplane on the ramp

    - Make a note of the appearance, looking for obvious problems

      - Gear out of alignment, structural distortion, skin damage, dripping fuel/oil leaks

  - Upon reaching the airplane, all Tie Downs, Control Locks, and Chocks should be removed

- What to Inspect

  - Inside the Cockpit

    - Airworthiness - Required Documents (AROW)

    - Logbooks – To ensure the required tests and inspections have been completed (*Not in the cockpit)

      - Annual, 100 hr, Static/Transponder/Altimeter (24 mo), ELT (12 mo), ADs are complied with

    - Required equipment for the flight (EX: Mode C transponder in Class B/C Airspace, Instruments)

    - Inspect the items inside the airplane (Instruments, Switches, Mixture, etc, as listed on the checklist)

  - Outside the Airplane

    - Inspect the items outside the airplane (Structure, Controls, Engine, Prop, Gear, Struts, everything)

    - Defects are detected by following the checklist and looking for something wrong in each item

    - CE - Inability to recognize discrepancies to determine airworthiness

      - Understand what you are looking at or for, and what is required to be safe/normal

- Detecting Problems

  - Visible Structural Damage

    - Check for dents, cracks, tears which can affect airplane performance

      - The Diamond aircraft cannot be flown if a dent is found because of their construction

    - Check for leaks/stains as they are signs of potential problems

    - Look for missing rivets/bolts/etc.

    - Inspect the propeller for nicks/cracks

  - Flight Controls

    - Ensure the flight controls move freely/correctly and are attached securely/properly

    - Inspect the balance weights

    - Check the flap movement and connection, etc.

  - Fuel Quantity and Contamination

    - Quantity – Confirm the fuel quantity indicated on the gauge by a visual inspection (DA20: Fuel stick)

      - Airplane attitude, gauge malfunctions, etc can result in incorrect readings

    - Contamination

      - Type, Grade of Fuel – Critical to safe flight

        - Looking for 100LL (AVGAS) – Blue with a familiar gasoline scent

        - Jet-A is clear, has a kerosene scent, and disastrous effects when in reciprocating engines

          - The engine will be destroyed from detonation

            - Detonation – The uncontrolled explosive combustion of the fuel/air mixture in the cylinder’s combustion chamber

          - Refueling trucks are marked with JET-A placards

        - Supervise fueling to ensure the right type, and grade of fuel and the fuel caps are in place

        - Never substitute a lower grade of fuel for a required higher grade (Detonation will result)

          - 80 is Red; 100LL is Blue; 100 is Green; Jet Fuel is Clear

      - Water and Other Sediment – Usually from condensation in partially filled tanks or bad seals

          - Water is heavier than fuel and therefore accumulates in the low points

          - Prevented by minimizing the opportunity for condensation – keep the tanks full

        - Sediment can arise from dust/dirt entering the tanks

      - Checking the Grade and Removing the Water and Other Contamination

        - Drain the fuel from the gascolator/tanks checking for color/smell/water/contamination

          - If water/contaminants are found continue draining until they have been removed

  - Oil Quantity and Contamination

    - Check the oil level on the oil dip stick to ensure it is at an acceptable amount (4-6 quarts)

      - The plane will use a small amount each flight, if a large amount is used there may be a problem

    - Contamination can be detected by any discoloration

  - CE - Failure to ensure servicing with the proper fuel and oil

    - Always drain a sample of fuel and ensure the correct fuel is being put into the airplane

    - When adding oil, ensure it is the type called for in the POH

  - Leaks (Fuel, Oil, Hydraulic)

    - Check to see there are no leaks under the airplane, inside the cowling, or on the wheel struts

      - The fuel vent may appear to be a leak, but allows air into the tank/vents excess fuel

- Ice and Frost

  - Small amounts of ice/frost can disrupt the airflow over the wing, increasing stall speed and reducing lift

  - Do not fly unless the ice/frost is removed from the airfoils

    - Heated hangar, spraying deicing compounds, scraping it off

- Loading and Securing (Baggage, Equipment, Cargo)

  - Ensure everything is in its place and secure, you do not want items flying around in flight (turbulence)

    - CG could be affected if items are moving during flight

    - Movement of heavy items could damage the aircraft

  - CE - Failure to ensure proper loading and securing of baggage, cargo, and equipment

- Determining the Airplane is Safe

  - Note any discrepancies during the preflight and make sound judgments

    - PIC is responsible for determining the airplane is airworthy and safe

      - If you have doubt, ask someone with more experience

    - Don’t attempt a flight if you are uncomfortable or not completely satisfied the airplane is safe