# PRECISION INSTRUMENT APPROACH

Objective:

To teach the student the elements, proper procedures and technique for piloting an airplane on a precision instrument approach.

Content:

- Elements of a precision instrument approach

  - Selection of the appropriate instrument approach chart

  - Pertinent information on the selected instrument approach chart

    - Name contains letter from end of alphabet (ILS Y RWY 28)

    - Minimum safe/sector altitude (MSA)

    - Communications frequencies

    - Plan view

    - Profile view

    - Minimums section

    - Airport diagram

    - New NACO IAP charts

    - Memorize

  - Selection, tuning, identification and determination navigation equipment

  - Radio communication with ATC and compliance with ATC clearances, instructions and procedures

  - Aircraft configuration, airspeed and checklist items

  - Adjustments applied to the DH/DA and visibility criteria for the aircraft approach category

  - Maintenance of altitude, airspeed and track

  - Rate of descent during the final approach segment

  - Weather Factors

- Common Errors

  - Failure to have essential knowledge of the information on the instrument approach chart

  - Incorrect communications procedures or noncompliance with ATC clearances or instructions

  - Failure to accomplish checklist items

  - Faulty basic instrument flying technic

  - Inappropriate application of DA/DH

References:

14 CFR Part 91, AIM, NACO TPPs

Aviation Instructor's Handbook, FAA-H-8083-9

Instrument Flying Handbook, FAA-H-8083-15, 1999

Instrument Rating ACS

Completion Standards:

The lesson is complete when the student can accurately perform the steps involved in flying a precision instrument approach using the materials covered in this lesson to the satisfaction of the instructor.

Instructor Notes:

- Elements of a precision instrument approach

  - Selection of the appropriate instrument approach chart

    - 14 CFR Sec. 91.175 requires use of standard IAP prescribed for the airport by 14 CFR part 97

    - Select IAP chart from current TPP

    - GPS - use current database

  - Pertinent information on the selected instrument approach chart

    - Name contains letter from end of alphabet (ILS Y RWY 28)

      - Means there is more than one ILS RWY 28

      - Make sure you have the right IAP chart for the right IAP

    - Minimum safe/sector altitude (MSA)

      - 25(-30)NM radius

      - 1000 feet obstacle clearance

      - Maybe no navigation signal

      - For emergency use

    - Communications frequencies

    - Plan view

      - IAF(s)

      - Approach segment routes (course, distance, altitude)

      - FAC

      - NAVAIDs and frequencies

      - PT or NoPT

      - Holding pattern(s)

      - Notes

    - Profile view

      - Lightning bolt symbol

        - Position of glide slope intercept at minimum altitude for intermediate approach segment

        - Start of final approach segment

        - Begin descent on glide path to DA

      - Maltese cross = FAF for LOC approach

Begin timing

      - Altitudes to fly

      - Distances along final approach segment

      - Glide path descent angle

      - Altitude of glide path at LOC FAF

      - Threshold crossing height (TCH)

      - Missed approach instructions

    - Minimums section

      - Know aircraft category based on 1.3 x VSo

        - C-TR182 - cat A but

        - Circling at 100 KIAS, use category B circling minimums

      - DAs (ILS) and MDAs (LOC)

      - Minimum visibility required for landing

      - Notes

    - Airport diagram

      - Runway dimensions, lighting, TDZE

      - LOC FAF to MAP time table

Based on ground speed

    - New NACO IAP charts

      - Pilot briefing section (EZ Brief)

      - Missed approach symbols

      - RNAV RWY # charts `

        - For GPS, WAAS, LAAS

        - Up to 4 lines of approach minimums

GNS 530 use LNAV minimums

        - TAA icons

      - DA replaces DH

    - Memorize

      - DA (How low?)

      - Time from LOC FAF to MAP (How long?)

      - Final approach course (Which way in?)

      - Missed approach procedure start (Which way out?)

  - Selection, tuning, identification and determination of operational status of ground and aircraft navigation equipment

    - IAP chart contains frequencies and Morse code identifiers for the approach

    - Confirm correct Morse codes

    - Operational status check of each NAVAID to be used

      - Note VOR and glide slope alarm flags not on, CDI stable

      - Monitor ADF Morse code identification, check test function

      - GPS approach

- Correct approach and IAF (or vectors) selected from current database

- Approach loaded and activated

- No RAIM failure/status annunciations

      - ILS frequency selected: note that rotating OBS has no effect on CDI

  - Radio communication with ATC and compliance with ATC clearances, instructions and procedures

    - Center -> approach control -> tower, FSS or CTAF

    - Missed approach: first fly, get complete control of stable climb then: CTAF, tower or FSS -> approach control

    - ATC clearance, instructions

      - Execute on receipt or request amendment

      - "Cleared for approach"

        - Fly any approved IAP but

        - NOT contact or visual

      - Fly entire approach as published

  - Appropriate aircraft configuration, airspeed and checklist items

    - Use predetermined configurations

    - Slow to approach airspeed during initial approach segment (or vectors to final)

    - Before landing checklist

      - Initial approach segment: GU('wheels to go')MPS

      - Glide slope intercept: GU(gear down)MPS check again

      - LOC FAF: altimeter check

      - final approach segment: GUMPS (gear down, locked)

  - Adjustments applied to the published DH/DA and visibility criteria for the aircraft approach category

    - Check NOTAMs for IAP amendments

    - Inoperative runway visual aids increase minimum visibility required (bot not MDA)

      - Check Inop Components Table inside front cover of TPPs

      - Check minimum section of JEPP chart

    - Approved substitutes for (outer and) middle marker

      - Compass locator (LOM, LMM)

      - Precision radar (PAR)

    - Approved substitutes for outer marker

      - LOM

      - PAR

      - DME

      - VOR

      - NDB

      - ASR

    - See note below minimums section for changes required by weather/altimeter variables

    - During circling approach, apply circling minimums section using actual airspeed (Cat B for 100 KIAS)

  - Maintenance of altitude, airspeed and track, where applicable

    - Beginning prior to glide slope intercept, maintain

      - Altitude +/- 100 feet (aim for +/- 20 feet)

      - Heading +/- 10° with stable wind drift correction (aim for +/- 2°)

      - Airspeed +/- 10 knots (aim for +/- 2 knots)

      - CDI less that ¾-scale deflection (aim for < 1 dot)

    - On descent on glide path, allow no more than ¾-scale deflection of either glide slope or localizer indicator (aim for < 1 dot) until over the runway

    - C-TR182: maintain approach airspeed, 100 KIAS, during missed approach climb

  - Establishment and maintenance of an appropriate rate of descent during the final approach segment

    - Maintain a stabilized final approach from the final approach fix to the DA/DH, allow no more that ¾-scale deflection of either glide slope or localizer indications (aim for < 1 dot), and maintain the desired airspeed within +/-10 knots (aim for +/-2 knots)

    - Use small heading changes (<2°) to track LOC

    - If airplane is properly trimmed for approach speed, both glide slope and airspeed can be maintained with small power adjustments

    - If small power adjustments not enough then

      - Adjust pitch to maintain desired rate of descent, and

      - Adjust power to maintain airspeed

  - Factors that should be considered in determining whether

    - The approach should be continued straight-in to a landing

      - If runway environment (14 CFR Sec. 91.175) clearly seen and

      - Visibility greater than required minimum and

      - Position of aircraft such that straight-in approach and landing can be made using normal maneuvers (no more than small, shallow bank turns or small vertical speed adjustments), then

      - Transition from instrument to visual cues and descent from DA (or LOC MAP) to straight-in landing may be made

    - A circling approach to a landing should be made

      - Landing on runway with clearly favorable wind conditions may require a circling approach to that runway after flying an ILS approach to another runway

      - ATC request

    - A missed approach should be performed

      - At DA (or LOC MAP) or any time below DA or MDA until touchdown if

        - Runway environment (14 CFR Sec. 91.175) not clearly seen

        - Visibility less than required minimum, or

        - Aircraft not in position for safe landing with normal maneuvering

      - Part of airport not clearly visible during circling approach above MDA

      - ATC request

- Common Errors

  - Failure to have essential knowledge of the information on the instrument approach chart

    - Plan ahead

    - Know approach(es) to expect (ATC, ATIS)

    - Study, record, memorize key data in advance

How low? How long? How far? Which way?

    - Use PC or simulator to practice flying approaches to destinations of upcoming flights

  - Incorrect communications procedures or noncompliance with ATC clearances or instructions

If cannot comply, request amendment

  - Failure to accomplish checklist items

    - Start before landing checklist on initial approach segment or on initial vectors for approach

    - Complete before landing checklist at glide slope intercept (gear down)

  - Faulty basic instrument flying technic

May be due to cockpit disorganization and increased workload

  - Inappropriate application of DA/DH

    - May indicate inadequate speed of cross-check and interpretation during high workload time

    - Stay on localizer and glide slope until over the runway