# SECONDARY STALLS (DEMONSTRATION) (ASEL and ASES)

Objective:

To familiarize the student with the dangers of secondary stalls, when they may occur, and how to avoid them.

Before Flight:

- Review stalls

- Review recovery procedures

- Secondary stalls occur as a result of attempt to hasten stall recovery

Maneuver Elements:

- Setup

  - Clear the area

  - Choose forced landing area

  - Configure aircraft for a normal power-on or power-off stall, with altitude as necessary to recover ≥1500’ AGL

  - Select outside references

  - During recovery from initial stall, quickly increase the pitch attitude again while maintaining directional control with aileron and rudder pressure, allowing the airplane to enter a secondary stall

- Recovery

  - Reduce the angle of attack by releasing back-elevator pressure, simultaneously increasing throttle to full (if not already)

  - Anticipate left-turning tendencies with right rudder pressure

  - Return nose to straight-and-level, coordinated flight

  - Maintain ball centered

  - Upon positive rate of climb, retract flaps and gear as necessary

  - Look for traffic

Aircraft Setup:

Flaps as required, gear as required

Approach power:  (C172RG: 15” Hg, 2700 RPM)

Things to Avoid (Common Errors):

- Failure to adequately clear the area

- Inability to recognize an approaching stall condition through feel for the airplane

- Premature recovery

- Over-reliance on the airspeed indicator while excluding other cues

- Inadequate scanning resulting in an unintentional wing-low condition during entry

- Excessive back-elevator pressure resulting in an exaggerated nose-up attitude during entry

- Inadequate rudder control

- Inadvertent secondary stall during recovery

- Failure to maintain a constant bank angle during turning stalls

- Excessive forward-elevator pressure during recovery resulting in negative load on the wings

- Excessive airspeed buildup during recovery

- Failure to take timely action to prevent a full stall during the conduct of imminent stalls

References:

Airplane Flying Handbook - Chapter 4

Completion Standards:

The lesson is complete when the student demonstrates knowledge through a written or oral exam and is able to perform secondary stalls to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.