# SHORT-FIELD (CONFINED AREA—ASES) TAKEOFF AND MAXIMUM PERFORMANCE CLIMB (ASEL and ASES)

Objective:

To familiarize the student with the proper procedures and techniques for short-field takeoff and climb.

Before Flight:

- Review aircraft V-speeds

- Terms:

  - Takeoff Roll (Ground Roll)

  - Lift-off (Rotation)

  - Ground Effect

  - Initial Climb

  - Torque/P-Factor

  - Propeller slipstream

- Pre-takeoff considerations

  - Preflight check

  - CTAF or ATC

  - Wake turbulence

  - Maintaining runway centerline during takeoff roll and climbout

  - Why takeoff into the wind?

- Noise abatement procedures

- Obstacles or short runway

Maneuver Elements:

- Clear the area

- Choose forced landing area

- Configure aircraft: flaps as specified, cowl flaps open, propeller to full

- Select outside references: vanishing point on runway

- Taxi onto runway centerline

- Apply full brakes

- Smoothly apply full power, upon engine peak, release brakes

- Anticipate need for right rudder pressure

- Check engine instruments (in green)

- Attitude slightly tail-low

- Do NOT rotate at VR for short field take-off

- At VX (gear out), apply back pressure to lift nose wheel

- Adjust pitch to maintain steep climb out at VX

- Clear the real or simulated obstacle (if applicable)

- Adjust to normal climb VY (gear out)

- Gear up upon positive rate of climb, safe airspeed, no useable runway and clear of obstacle

- After 500’ AGL, flaps up

- Maintain ball centered

- Look for traffic

Aircraft Setup:

Flaps (C172RG: 10º), gear down, propeller Full

Vx (C172RG: 63 KIAS)

Vy (C172RG: 84 KIAS)

Things to Avoid (Common Errors)

- Failure to adequately clear the area

- Failure to utilize all available runway/takeoff area

- Failure to have the airplane properly trimmed prior to takeoff

- Premature lift-off resulting in high drag

- Holding the airplane on the ground unnecessarily with excessive forward-elevator pressure

- Inadequate rotation resulting in excessive speed after lift-off

- Inability to attain/maintain best angle-of-climb airspeed

- Fixation on the airspeed indicator during initial climb

- Premature retraction of landing gear and/or wing flaps

Diagrams:

References:

Airplane Flying Handbook - Chapter 5

Aircraft Flight Manual / Pilot’s Operating Handbook

Completion Standards:

The lesson is complete when the student is able to perform short-field takeoffs and climbs to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.