# SOFT-FIELD APPROACH AND LANDING (ASEL)

Objective:

To teach the pilot proper procedures and technique for making landings on soft surfaces.

Before Flight:

- When is this used?

- Lower than normal airspeed requires vigilance

- Used to minimize nose-over force from touchdown on soft fields

- Wider than normal pattern should be flown to allow time for proper setup

- Touchdown should be made at lowest possible speed

- See aircraft AFM/POH for specific procedures

- Region of reversed command and its applicability to soft-field approaches

- Considerations after touchdown

Maneuver Elements:

- Clear the area

- Choose forced landing area (should be runway)

- Configure aircraft for normal approach and begin descent as normal

- Select outside references (e.g. runway numbers)

- Clear area, then turn to final (≤30° bank)

- On final: remaining flaps when runway is assured, CCGUMPS check

- Select aim point (e.g. before runway numbers)

- Adjust pitch and power to maintain normal approach speed and descent angle

- Trim to relieve control pressures

- Make sure feet are not on brakes

- 10-20’ off ground: power to maintain minimum descent rate (e.g. throttle to idle then re-add some power to slow descent rate)

- Gradually apply back pressure to pitch for straight-and-level attitude, attempting to fly just above runway (fly in ground effect) straight-and-level until passing aim point, then continue adjusting pitch for climb attitude just above horizon

- Hold aircraft in ground effect, 1-2’ above surface, as long as possible

- Upon touchdown on main gear, smoothly apply elevator pressure to hold nose wheel off surface as long as possible

- Gently lower the nose wheel to the surface

- Slight addition of throttle upon touchdown can assist softly lowering the nose wheel

- Do not use brakes and maintain elevator back pressure for taxiing

Aircraft Setup:

Flaps as required, gear down, propeller Full

Final approach speed:  (C172RG: 65 KIAS)

Things to Avoid (Common Errors)

- Excessive descent rate on final approach

- Excessive airspeed on final approach

- Unstabilized approach

- Roundout too high above the runway surface

- Poor power management during roundout and touchdown

- Hard touchdown

- Inadequate control of the airplane weight transfer from wings to wheels after touchdown

- Allowing the nose wheel to “fall” to the runway after touchdown rather than controlling its descent

Diagrams:

References:

Airplane Flying Handbook - Chapter 8-17

Aircraft Flight Manual / Pilot’s Operating Handbook

Completion Standards:

The lesson is complete when the student can fly a stabilized soft-field approach and landing at 1.3VSO +- 5 knots and can touch down as required by a soft-field landing. The instructor must also determine that the student has the appropriate knowledge required for soft-field landings by giving an oral or written exam.