# SOFT-FIELD TAKEOFF AND CLIMB (ASEL)

Objective:

To familiarize the student with the proper procedures and techniques for soft-field takeoff and climb.

Content:

- Before Flight:

  - Review aircraft V-speeds

  - Terms:

    - Takeoff Roll (Ground Roll)

    - Lift-off (Rotation)

    - Ground Effect

    - Initial Climb

    - Torque/P-Factor

    - Propeller slipstream

  - Pre-takeoff considerations

    - Preflight check

    - CTAF or ATC

    - Wake turbulence

    - Maintaining runway centerline during takeoff roll and climbout

    - Why takeoff into the wind?

  - Noise abatement procedures

  - Runway condition

Maneuver Elements:

- Clear the area

- Choose forced landing area

- Configure aircraft: flaps as specified, cowl flaps open, propeller to full

- Select outside references: vanishing point on runway

- Taxi onto runway centerline with full elevator back pressure: do not stop once taxiing

- Smoothly apply full power

- Anticipate need for right rudder pressure

- Check engine instruments (in green)

- As aircraft accelerates, apply enough back pressure to establish positive angle of attack

- After lift-off, lower the nose gently with the wheels clear of the runway, attempting to fly in straight-and-level flight within a half-wingspan above the ground

- Accelerate in ground effect to VY

- Gear up upon positive rate of climb, safe airspeed, no useable runway

- After 500’ AGL, flaps up

- Maintain ball centered

- Look for traffic

Aircraft Setup:

Flaps (C172RG: 10º), gear down, propeller Full

Vy (C172RG: 84 KIAS)

Things to Avoid (Common Errors)

- Failure to adequately clear the area

- Insufficient back-elevator pressure during initial takeoff roll resulting in inadequate angle of attack

- Failure to cross-check engine instruments for indications of proper operation after applying power

- Poor directional control

- Climbing too steeply after lift-off

- Abrupt and/or excessive elevator control while attempting to level off and accelerate after lift-off

- Allowing the airplane to “mush” or settle resulting in an inadvertent touchdown

- Attempting to climb out of ground effect area before attaining sufficient climb speed

- Failure to anticipate an increase in pitch attitude as the airplane climbs out of ground effect

Diagrams:

References:

Airplane Flying Handbook - Chapter 5

Aircraft Flight Manual / Pilot’s Operating Handbook

Completion Standards:

The lesson is complete when the student is able to perform soft-field takeoffs and climbs to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.