# STRAIGHT-AND-LEVEL FLIGHT (ASEL and ASES)

Objective:

To develop the fundamental techniques required for basic flight.

Before Flight:

- Review general aircraft operation and speeds

- Review aerodynamics for straight and level flight

- Straight and level flight is a matter of consciously correcting the relationship between the position of some portion of the airplane, used as a reference point, with the horizon (AFH 3-4)

- Level flight accomplished by checking a reference point (usually point on nose) with natural horizon

- Straight flight accomplished by checking airplanes wingtips with the horizon

Maneuver Elements:

- Clear the area

- Choose forced landing area (always be aware of options)

- Configure aircraft for cruise

- Select outside references (point on the horizon corresponding to heading)

- Periodically insure the nose is fixed below the horizon

- Periodically insure the wingtips are equally above or below the horizon (level)

- Compensate for cross wind to maintain heading

- Trim as necessary to maintain altitude

- Maintain ball centered

- Look for traffic

Aircraft Setup:

Flaps up, gear up

Normal cruise:  (C172RG: 23” Hg, 2300 RPM)

Things to Avoid (Common Errors)

- Attempting to use improper reference points on the airplane to establish attitude

- Forgetting the location of preselected reference points on subsequent flights

- Attempting to establish or correct airplane attitude using flight instruments rather than outside visual reference

- Attempting to maintain direction using only rudder control

- Habitually flying with one wing low.

- “Chasing” the flight instruments rather than adhering to the principles of attitude flying

- Too tight a grip on the flight controls resulting in over-control and lack of feel

- Pushing or pulling on the flight controls rather than exerting pressure against the airstream

- Improper scanning and/or devoting insufficient time to outside visual reference (head in the cockpit)

- Fixation on the nose (pitch attitude) reference point

- Unnecessary or inappropriate control inputs

- Failure to make timely and measured control inputs when deviations from straight-and-level flight are detected

- Inadequate attention to sensory inputs in developing feel for the airplane

References:

Airplane Flying Handbook - Chapter 3

Completion Standards:

The lesson is complete when the student can detect and correct for deviations in straight-and-level flight with small corrections.