TURNS AROUND A POINT (ASEL and ASES)

Objective:

To teach turning technique, subconscious control of the airplane, division of attention, effects of bank angle to radius of a turn, perception of altitude and correction of wind drift.

Before Flight:

- Review S-Turn principles

- Review applicable aircraft performance information

- Maximum bank during the maneuver should not exceed 45°

Maneuver Elements:

- Clear the area

- Choose forced landing area

- Configure aircraft: flaps and gear up, speed at or below Va, straight-and-level at altitude between 600-1000’ AGL, trim to maintain altitude

- Select outside reference (a distinguished point to encircle)

- Enter the circle, ideally on downwind and close enough to allow for a 30º to 45º bank at steepest point (this lesson assumes downwind entry)

- Turn to the steepest bank at the start of the circle (on downwind)

- Upon rounding the turn, the bank will become progressively shallower until on upwind, at which point it will be shallowest, then start to become steeper again until again on downwind

- Maintain radius of circle by adjusting bank as necessary to compensate for wind

- Maintain ball centered

- Look for traffic throughout

Aircraft Setup:

Flaps and gear up

Power and speed at or below Va

Traffic pattern altitude of 600 – 1000’ AGL

C172RG: 	MP: 	18” Hg

RPM: 	2300 RPM

Speed: 	100 KIAS

Things to Avoid (Common Errors)

- Failure to adequately clear the area

- Failure to establish appropriate bank on entry

- Failure to recognize wind drift

- Excessive bank and/or inadequate wind correction angle on the downwind side of the circle resulting in drift towards the reference point

- Inadequate bank angle and/or excessive wind correction angle on the upwind side of the circle resulting in drift away from the reference point

- Skidding turns when turning from downwind to crosswind

- Slipping turns when turning from upwind to crosswind

- Gaining or losing altitude

- Inadequate visual lookout for other aircraft

- Inability to direct attention outside the airplane while maintaining precise airplane control

Diagram:

References:

Airplane Flying Handbook – Chapter 6

Completion Standards:

The lesson is complete when the student demonstrates knowledge through a written or oral exam and is able to perform turns around a point to the satisfaction of the instructor and in accordance with the current Practical Test Standards for the student’s stage of training.